3GPP TSG-RAN WG3 Meeting # 129bis	R3-256764

# Prague, Czech, 13 – 17, October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1535 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on OD-SIB1 Configuration Provision |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - According to present spec, if the OD-SIB1 config is transmitted in the Cell A indicated by the request message, the response message is sent including the Cell A ID. Otherwise, the response message without Cell A ID should be sent in case the Cell A ID IE is not included in the request message and the OD-SIB1 config is transmitted in the pre-configured cell(s). However, the later case is not captured in spec. Relatively, the failure message should be sent without the Cell A ID IE in case the Cell A ID IE is not included in the request message and the OD-SIB1 config cannot be transmitted in any cell pre-configured, which is not captured in spec. - If the transmission of OD-SIB1 config is stopped (as requested by NG-RAN node1 or by NG-RAN node2 proactively), it’s better to make clear that all cells of NG-RAN node2 stop the transmission. - Criticality of NES Cell ID IE should be ‘reject’, and the criticality of Cell A ID IE should be ‘ignore’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the OD-SIB1 Configuration Provision procedure to capture the case where the response message and the failure message are sent without the Cell A ID. - Clarify in the OD-SIB1 Configuration Provision Status Update procedure that all cells has stopped OD-SIB1 broadcasting. - Change the criticality of Cell A ID IE in tabular of OD-SIB1 CONFIGURATION PROVISION FAILURE to ‘ignore’. - Change asn.1 for OD-SIB1 CONFIGURATION PROVISION RESPONSE/OD-SIB1 CONFIGURATION PROVISION FAILURE/OD-SIB1 CONFIGURATION PROVISION STATUS on criticality of multiple IEs. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact. Because the it affects the OD-SIB1 Configuration Provision and the tabular of OD-SIB1 CONFIGURATION PROVISION FAILURE message. The changes to asn.1 only impacts R19. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The condition for sending the messages without Cell A ID is not clear. Mistakes on IEs criticality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.17.2, 8.4.17.3, 8.4.18.2, 9.1.3.32, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR0490 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.17	OD-SIB1 Configuration Provision

#### 8.4.17.1	General

The purpose of the OD-SIB1 Configuration Provision procedure is to enable an NG-RAN node1 to provide OD-SIB1 configuration information to an NG-RAN node2 and to request the NG-RAN node2 to transmit the provided OD-SIB1 configuration information.

The procedure is also used to enable the NG-RAN node1 to request the NG-RAN node2 to stop transmission of the OD-SIB1 configuration.

The procedure uses non-UE -associated signaling.

#### 8.4.17.2	Successful Operation

Figure 8.4.17.2-1: OD-SIB1 Configuration Provision procedure, successful operation

The NG-RAN node1 initiates the procedure by sending the OD-SIB1 CONFIGURATION PROVISION REQUEST message to the NG-RAN node2.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is set to "start", and Cell A ID IE is not included in the message, the NG-RAN node2 shall, if supported, transmit the provided OD-SIB1 Configuration in SIBx of the pre-configured cell(s) and reply with the OD-SIB1 CONFIGURATION PROVISION RESPONSE message. The NG-RAN node2 stores the OD-SIB1 Configuration information.

If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, transmit the OD-SIB1 Configuration in the cell indicated by the Cell A ID IE and includes the Cell A ID IE indicating the same cell in the OD-SIB1 CONFIGURATION PROVISION RESPONSE message.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is set to "start" and the NES Cell ID IE indicates a cell for which the NG-RAN node2 is already transmitting the OD-SIB1 Configuration:

-	If the request is for a cell already transmitting the OD-SIB1 configuration for the cell identified by the NES Cell ID IE, the NG-RAN node2 shall, if supported, stop the ongoing transmission and start transmission of the newly provided OD-SIB1 Configuration in SIBx. The NG-RAN node2 replaces the stored OD-SIB1 Configuration information with the newly received OD-SIB1 Configuration information.

-	If the request is for a new cell indicated by the Cell A ID IE, the NG-RAN node2 shall, if supported, start transmitting the provided OD-SIB1 Configuration in SIBx in the cell indicated by Cell A ID IE.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, is set to "stop" the NG-RAN node2 shall stop broadcasting OD-SIB1 Configuration for the cell indicated by NES Cell ID IE in SIBx of any cell. The NG-RAN node2 removes the stored OD-SIB1 Configuration.

#### 8.4.17.3	Unsuccessful Operation

Figure 8.4.17.3-1: OD-SIB1 Configuration Provision procedure, unsuccessful operation

If the Cell A ID IE is not included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message and the NG-RAN node2 cannot transmit the provided OD-SIB1 Configuration in SIBx of any pre-configured cell, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message.

If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message and the NG-RAN node2 cannot transmit the provided OD-SIB1 Configuration in SIBx of the cell indicated by the Cell A ID IE, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message. The NG-RAN node2 includes the Cell A ID IE in the OD-SIB1 CONFIGURATION PROVISION FAILURE message.

The message shall contain the Cause IE with an appropriate value.

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

### 8.4.18	OD-SIB1 Configuration Provision Status Update

#### 8.4.18.1	General

This procedure is initiated by an NG-RAN node2 to inform a neighbouring NG-RAN node1 about a previously admitted UL WUS configuration provision being stopped.

The procedure uses non-UE associated signalling.

#### 8.4.18.2	Successful Operation

Figure 8.4.18.2-1: OD-SIB1 Configuration Provision Status Update procedure, successful operation

The NG-RAN node2 initiates the procedure by sending the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message to the NG-RAN node1.

Upon reception of the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message with the Provision Status IE set to "stopped", the NG-RAN node1 shall consider that a previously admitted OD-SIB1 configuration for the cell identified by the NES Cell ID IE is no longer transmitted in the cell identified by the Cell A ID, if it is present. Else the NG-RAN node1 shall consider the NG-RAN node2 has stopped the OD-SIB1 broadcasting in all cell(s). The NG-RAN node2 removes the stored OD-SIB1 Configuration that is no longer transmitted.

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.32	OD-SIB1 CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration cannot be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unchanged parts omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION RESPONSE

--

-- **************************************************************

ODSIB1ConfigurationProvisionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ODSIB1ConfigurationProvisionResponse-IEs}},

...

}

ODSIB1ConfigurationProvisionResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NES-Cell-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY ignore	TYPE NR-CGI										PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION FAILURE

--

-- **************************************************************

ODSIB1ConfigurationProvisionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ ODSIB1ConfigurationProvisionFailure-IEs}},

...

}

ODSIB1ConfigurationProvisionFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause									CRITICALITY ignore	TYPE Cause										PRESENCE mandatory}|

{ ID id-NES-Cell-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY ignore	TYPE NR-CGI										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION STATUS

--

-- **************************************************************

ODSIB1ConfigurationProvisionStatus ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ODSIB1ConfigurationProvisionStatus-IEs}},

...

}

ODSIB1ConfigurationProvisionStatus-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NES-Cell-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY ignore	TYPE NR-CGI										PRESENCE optional }|

{ ID id-ProvisionStatus							CRITICALITY reject	TYPE ProvisionStatus							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

ProvisionStatus ::= ENUMERATED {stopped, ...}

<<<<<<<<<<<<<<<<<<<< Unchanged parts omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< End of changes >>>>>>>>>>>>>>>>>>>>