3GPP TSG-RAN WG3 Meeting #129bis                                                               R3-256771

Prague, Czech Republic, 13 - 17 October, 2025

Source:	CATT

Title:	Discussion on network architecture for ISAC

Agenda Item:	13.2

Document for:	Discussion

Introduction

In RAN#108 meeting held in June 2025, the new study item on Integrated Sensing and Communication (ISAC) for 5G NR was approved. This topic only supports the gNB-based mono-static sensing for UAV target use cases. The objectives of this topic are described in the SID [1] as follows:

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. o For other waveform and reference signals, companies are to share relevant information o No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. o FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

It is noted that SA has approved the study on functionality enhancement, end to end service and procedures for 5G to support ISAC in SA#107 (March 2025), with the scope aligned with RAN SI (focusing on gNB-based sensing for UAV detection and tracking). The following aspects captured in SID [2] were discussed during the last two SA2 meetings:

- WT-1: Architecture and function enhancements to support sensing. - WT-2: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer. - WT-3: Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities. - WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation. - WT-5: Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting. - WT-6: Configuration parameters/policy provisioning for the support of ISAC services.

Per RAN SID [1], RAN3 will study the procedure, signalling between CN and RAN as well as the network architecture for sensing. In this paper, we discuss the network architecture for gNB-based mono-static sensing for UAV target use cases. Section 2 gives the details discussion and followed by Section 3 that summarizes the conclusion of this paper.

Discussion

RAN and SF functionality split

Before discussing the network architecture, it should be make clear the function of RAN and the core network and the necessary interaction between them.

According to SA2’s TR 23.700-14, sensing entity (SE) performs the sensing operation, and sensing function (SF) supports the sensing service and locates in core network.

TR 23.700-14 ## 3.1 Terms For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1]. Sensing Service Consumer: The entity that may consume the Sensing Result. The Sensing Service Consumer may also request the Sensing Result. Sensing Entity: The Sensing Entity referring to a Sensing Transmitter and/or to a Sensing Receiver. NOTE 1: In this release, the Sensing Entity is only referring to gNB. Sensing Function: Indicating the logical function which is involved to support Sensing Service. NOTE 2: The Sensing Function cannot be a Sensing Entity. Sensing Service: Capability to collect and provide information about object and/or characteristics of the environment using radio signals. gNB-based Sensing: it indicates gNB acting as Sensing Entity. This doesn’t imply any assumptions for the location of the sensing result generation.

The SF can perform the sensing service authorization and support the following functions based on the requirement of sensing service:

- SEs discovery and selection

- Configure the parameters for sensing operations in SEs

- Sensing results exposure.

The gNB should support functionalities of SE. In wireless sensing, the reflections of the sensing signal sent from the sensing transmitter are received by the sensing receiver, and the signal is measured and processed to obtain characteristics of the sensing target. For gNB-based mono-static sensing, the gNB should receive the sensing signal which is transmitted by itself.

Sensing result related to target detection and trajectory (e.g. target location) can be obtained after a series of processing on the raw data of the received sensing signal. However, the functionality split on sensing results generation between RAN and SF is not clear. Two different assumptions are observed.

Assumption 1: RAN produces the intermediate data based on the sensing measurements and reports the data to the SF, in which case the SF generates the sensing results. For example, the sensing report can be the intermediate sensing results per detect point or the spectrum of delay, doppler and angle.

Assumption 2: RAN generates the final results of the target trajectory and reports them to the SF.

The processing on the raw data to derive sensing result is based on the specific sensing method and algorithm.

In our view, the optimized performance can be achieved if all the processing is performed on the RAN. There are multiple disadvantages if the series of processing is split between RAN and SF:

- The exact handling carried out by gNB needs to be specified, as SF can receive sensing report from different gNBs and various algorithms can be used for generating intermediate data. Otherwise, the inter-operation between SF and gNBs of different vendors cannot be achieved.

- SF may hardly derive the finally detected targets just based on intermediate results, such as the detected point (position, delay, velocity, angle and power), without other information.

- Performance metrics discussed in RAN1 are based on the final sensing results. There is no evaluation for the performance based on intermediate results at present, thus the performance cannot be guaranteed.

- Requires enormous data to be uploaded from RAN to SF, which imposes to network heavily

On the contrast, all processing at RAN can avoid above issues. RAN has the full background information and thus can utilize the raw data to derive necessary information to figure out which target the detected points belong to. And this approach requires much smaller data to be uploaded to 5GC.

Therefore, sensing result generating at RAN side is preferred and we think it should be supported in Rel-20.

- Proposal 1: RAN3 adopt that RAN can generate the final results of target trajectory and report to SF.

Network architecture for C-plane

Based on the RAN-CN function split, the RAN and SF should support the interactions with each other on the sensing configuration and the sensing results reporting and collection. The interactions between RAN and SF include:

- SF sending sensing configuration to gNBs,

- gNBs reporting the sensing results to SF.

The interactions between SF and RAN should be at least carried out through C-plane. For C-plane architecture, RAN and SF can communicate with each other via AMF or RAN connects to SF directly, i.e., two architectures can be considered, indirectly and directly connections between RAN and SF. The two architectures are discussed as below.

Figure1. Indirect connecting through AMF

The indirect connection architecture has following characteristic:

- Reusing NG-C interface

  - SF signaling is carried via non-UE Associated NGAP messages, potentially impacting NGAP.

  - May require the SF application protocol layer (as NRPPa). If no SF protocol layer is introduced, the sensing related information is defined within NGAP protocol (AmIoT approach).

- New 5GC Interface: A new 5GC internal interface is required between AMF and SF.

- SF selects the RAN nodes which perform sensing and decide the corresponding AMFs, and trigger the AMF to initiate sensing configuration for RAN nodes. Whether AMF is transparent to the sensing signaling should be discussed.

Option A: Direct connection over new interface                Option B: Direct connection over NG interface

Figure2. Direct connection architecture

The characteristics of the direct connection architecture:

- SF connects to RAN directly

  - No impact to AMF

  - SF can send the sensing configurations directly to RAN.

- Isolating sensing from communication. RAN nodes only supporting sensing functionality do not need to connect with AMF.

- RAN nodes supporting both sensing and communication need to establish the new C-plane connection with core network (besides the connection to AMF), which introduces more effort on the connection management.

It can be observed that indirect and direct architecture would be applicable to different scenarios. Indirect architecture is appropriate to be used for the RAN devices which supports both sensing and communication, while direct architecture is suitable in case sensing is isolated from communication to the RAN devices which only support sensing.

Observation 1：Indirect C-plane architecture would be appropriate for the RAN devices which supporting both sensing and communication, while direct C-plane architecture can be applied to the RAN devices which only support sensing.

For the direct connection architecture, two options can be considered:

- Option A: Direct connection over new interface

- Option B: Direct connection over NG interface

For Option A, a new C-plane connection is to be specified and the SF application protocol should be introduced. For Option B, RAN connects to SF reusing the NG-C interface (may introduce a SF application protocol or not), and the impact to NGAP is assumed similar as the indirect architecture. Since Option A requires more specification effort for specifying the new interface, it’s not proper to introduce it in the last 5G release. So, Option A for direct architecture is not preferred from our view.

As 5G-A intends to realize sensing for UAV targets, which is suitable to be carried out by the outdoor macro gNBs, it’s important if the macro gNBs used for communication at present can be upgraded to support sensing due to the low expenses, for example, the operators do not need to allocate new sites for the sensing devices.  The indirect architecture is beneficial, considering that the RAN devices of communication can be reused efficiently.

Observation 2：For the C-plane, the indirect connection architecture and direct connection via NG can be considered.

Proposal 2: RAN3 discuss following architectures for the C-plane connection between RAN and SF:

- Indirect connecting through AMF,

- Direct connection over NG interface.

Whether to support User/Data plane connection

The sensing reports of target trajectory may lead to continuous data stream from RAN to SF over time, because of the real-time feature of the sensing service. If the data stream of sensing report is transmitted via C-plane, the transmission would not be efficient due to the integrity protection applied by the C-plane protocols, but in the meantime, the continuous data will lead to particularly high processing load to C-plane. So, it’s not appropriate for RAN to report the sensing results to SF via C-plane connection in some cases. A user or data plane connection between RAN and SF would be needed.

Proposal 3: RAN3 discuss to introduce a U/D plane connection between RAN and SF for sensing results transmission.

Please note that the pure user/data plane between SF and RAN is not suitable, because the signaling procedures for sensing configuration need to be performed between SF and RAN. These signaling procedures should be carried out through C-plane, considering that the performance (e.g., security and reliability) of the signaling delivery needs to be guaranteed. Therefore, if RAN3 decide to introduce the user/data plane connection between RAN and SF, it should be applied in conjunction with the C-plane connection, i.e., to support the separation of C-plane and U/D-plane.

Proposal 4: If a U/D plane connection is introduced, it should be used in conjunction with the C-plane connection.

  - Network architecture for User/Data plane

For the User/Data plane architecture, there are also two categories, i.e., the direct connection and the indirect connection architecture, where the sensing reports which are transmitted from RAN to SF are transferred by some core network function.

The direct connection for U/D-plane is shown in the following figure.

Figure 3 Direct connection for U/D-plane

Both the direct and indirect User/Data plane connections are managed by the C-plane connection, for example, for the configuration, update or release of the U/D plane connection.

The direct architecture provides a direct U/D-plane connection between RAN and SF. And the direct U/D-plane connection between RAN and SF can be managed by the direct or the indirect C-plane connection between RAN and SF, for example, for the configuration, update or release of the U/D-plane connection. Namely, it can be used in conjunction with the direct C-plane connection or the indirect C-plane connection. A new U/D-plane interface between RAN and core network is introduced, and it is assumed with no impact to legacy CN functions.

- Direct C-plane connection is used with the direct U/D plane connection:

Upon receiving sensing service request message, the SF selects sensing-capable RAN nodes and initiates configuration of the direct U/D-plane connection between RAN and SF.

- Indirect C-plane connection is used with the direct U/D plane connection:

The SF selects the sensing-capable RAN nodes and determines the corresponding AMFs. For the selected RAN node, SF can exchange the signal for U/D plane management to the RAN node with the AMF being transparent to the signal, as the U/D plane connection is terminated at SF and RAN.

Observation 3: The direct U/D-plane connection between RAN and SF can be managed by either the direct or indirect C-plane connections, namely, the direct U/D-plane connection can be used in conjunction with the direct C-plane connection or the indirect C-plane connection.

The indirect connection architecture deploys a CN entity (like the UPF) to allow the concentration of the U/D-plane connection between RAN and the SF. As a result, the sensing reports transmitted from RAN to SF can be concentrated and transferred by the CN function entity. In our view, the indirect U/D-plane connection can also be managed by the C-plane connection, i.e., used in conjunction with the C-plane connection. But the necessity should be discussed in SA2.

Observation 5: It’s up to SA2 whether to introduce an indirect U/D-plane connection between RAN and SF.

Conclusion

According to the analysis in section 2, we propose:

RAN and SF functionality split

Proposal 1: RAN3 adopt that RAN can generate the final results of target trajectory and report to SF.

Network architecture for C-plane

Observation 1：Indirect C-plan architecture would be appropriate for the RAN devices which supporting both sensing and communication, while direct C-plane architecture can be applied to the RAN devices which only support sensing.

Observation 2：For the C-plane, the indirect connection architecture and direct connection via NG can be considered.

Proposal 2: RAN3 discuss following architectures for the C-plane connection between RAN and SF:

- Indirect connecting through AMF,

- Direct connection over NG interface.

Whether to support User/Data plane connection

Proposal 3: RAN3 discuss to introduce a U/D plane connection between RAN and SF for sensing results transmission.

Proposal 4: If a U/D plane connection is introduced, it should be used in conjunction with the C-plane connection.

Network architecture for User/Data plane

Observation 3: The direct U/D-plane connection between RAN and SF can be managed by either the direct or indirect C-plane connections, namely, the direct U/D-plane connection can be used in conjunction with the direct C-plane connection or the indirect C-plane connection.

Observation 5: It’s up to SA2 whether to introduce an indirect U/D-plane connection between RAN and SF.

Reference

- RP-251861 New SID: Study on Integrated Sensing And Communication (ISAC) for NR, RAN#108, AT&T

- SP-250401 New SID on Study on Stage 2 for Integrated Sensing and Communication

- 3GPP TR 23.700-14 V1.0.0 (2025-09)