3GPP TSG-RAN WG3 Meeting #129-bis                                                             R3-2506773

Prague, Czech Republic, Oct 13th – 17th, 2025

Source:	CATT

Title:	Discussion on Topology-2 for Ambient IoT

Agenda Item:	14.2

Document for:	Discussion

1.	Introduction

This contribution gives an initial discussion on the D2T2-B for Device 1, with the objective of the WID [1].

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility.

Section 2 gives our detailed analysis, including

- Architecture and Protocol Stacks for supporting Topology 2

- UE reader authorization

- UE reader selection

- Inventory procedures in Topology 2

- Command after Inventory procedures in Topology 2

- Inter-gNB RRC-connected UE Reader Mobility

Section 3 summarizes the contribution of this contribution.

2.	Discussion

## 2.1 Architecture and Protocol Stacks for supporting Topology 2

Architecture for supporting Topology 2

In up-to-date SA2 TR 23.700-30 [2], SA2 approved the architecture assumptions and requirements for supporting Topology 2.

SA2 TR 23.700-30 4 Architectural Assumptions and Requirements 4.1 Architectural Assumptions The following traffic types for AIoT Device are to be studied: - DT: Device-terminated; - DO-DTT: Device-originated - device-terminated triggered; and - DO-A: Device-originated - autonomous. Common architectural assumptions: - Architecture reference models defined in TS 23.369 [3] are used as the baseline architecture for this study. - The AIoT Device is agnostic to topologies, i.e. Topology 1 and Topology 2. Architectural assumptions for support of Topology 2 for Key Issue#1: - No Rel-19 AIoT Device impact is expected. - The following topology as defined in TR 38.848 [10] is assumed: - BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control - Only Indirect Connectivity (i.e. NG-RAN <-> AMF <-> AIOTF) is applied. - The support of Topology 2 in this study is based on the RRC-based option and the interim conclusions of Rel-19 in TR 23.700-13 [7] is the baseline. … 4.2 Architectural Requirements The following architectural requirements are applicable to this study: - Solutions shall build on the 5G System architectural principles as in TS 23.501 [4], including flexibility and modularity for newly introduced functionalities. - The enhancements to support topology 2 shall not impact Rel-19 AIoT devices. - Solutions shall build on the AIoT architectural principles as in TS 23.369 [3]. - Support for AIoT Services needs to adhere to the nature of the AIoT Devices (e.g. ultra-low complexity, cost and resource-constrained). NOTE: Privacy protection and other security aspects will be coordinated with SA WG3.

In SA2 SID SP-250834 [3], there are also the following descriptions about architecture option for Topology 2.

SA2 SID SP-250834 WT#1: Study the support of AIoT services in Topology 2, including: - Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity.

According to SA2 progress and the description in RAN WID [1], only RRC-based architecture for D2T2-B is considered in R20, and only Indirect Connectivity (i.e. NG-RAN - AMF - AIOTF) is applied for Topology 2.

Observation 1-1: 	Only RRC based architecture is support for Topology 2, and only indirect connectivity (i.e. gNB communicates with AIOTF via AMF) is applied for Topology 2.

Proposal 1-1: 	RAN3 discuss indirect connectivity for Topology 2 with RRC-based architecture.

RAN architecture and Protocol Stacks of Topology 2 with RRC-based option

In R19 SI discussion phase, there are the following descriptions about RAN architecture aspects for supporting Topology 2 are achieved, which were captured in RAN3 part of RAN TR 38.769 [4]:

RAN TR 38.769 6.4.2 Support of Topology 2 Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB. The following definitions apply: A-IoT-enabled gNB: A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT-enabled UE via NR Uu interface. A-IoT-enabled UE: A UE supporting Common reader function, which is able to communicate with the A-IoTdevice via the A-IoT radio interface. Figure 6.4.2-1 Logical system architecture for topology 2 NOTE 1: Figure 6.4.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed. NOTE 2: The A-IoT CN could include AMF, UPF and A-IoTF, which is up to SA2 decision. NOTE 3: Details of the radio resource management performed by the A-IoT enabled gNB are subject to RAN1 and RAN2 mechanisms. For Topology 2, architecture and protocol aspects of split RAN architecture are not studied. In Topology 2, XX interface is NG interface. In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers. NOTE 4: How to perform coordination of the usage of the A-IoT radio resources among readers needs further discussion. An A-IoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter.

RAN TR 38.769 also captured the following descriptions regarding Protocol Stacks for Topology 2 with RRC-based solution:

RAN TR 38.769 6.4.2.1.1 Solution 1: RRC based solution Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa. Figure 6.4.2.1.1-1: RRC based solution of Topology 2 In this solution, XX interface is NG-C interface. NOTE: Whether "XXAP" is realised by including AIoTF information in NGAP or by carrying a new protocol layer over NGAP, needs further discussion and decision.

For Topology 1 with indirect connectivity case, RAN3 achieved the agreement that the interface between gNB and AMF is NG-C interface. For Topology 2 with indirect connectivity, since gNB also needs to communicate with AIOTF via AMF, the interface between gNB and AMF is also NG-C interface, in which NGAP is used between gNB and AMF to carry A-IoT specific information.

RAN3#126 agreements NGAP is always be used between RAN and AIoT CN.

CR R3-255964 agreed by RAN3 ## 16.xx Support of Ambient IoT ### 16.xx.x1 Introduction ### 16.xx.x2 Architecture The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions: … - As specified in TS 23.369 [xx], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF). A gNB is aware of direct connectivity with the AIOTF via OAM. NOTE 1: A deployment with both direct connectivity and indirect connectivity is not expected. - In this version of the specification, A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

Proposal 1-2:	For Topology2 with RRC-based architecture, RAN3 confirms that A-IoT enabled gNB connects to AMF via NG-C interface and A-IoT specific information is included within NGAP.

Regarding protocol stacks of Topology 2 with RRC-based option, the design on the A-IoT radio interface between reader and A-IoT device is common for Topology 1 and Topology 2, namely, for Topology 2 only A-IoT PHY and A-IoT MAC are included in A-IoT AS layer between A-IoT device and UE reader, and the difference of topologies is transparent to the A-IoT devices. Therefore, the Protocol Stacks for Topology 2 with RRC-based architecture are as follows:

Figure 1: Protocol Stacks for Topology 2 with RRC-based architecture

Proposal 1-3:	The Protocol Stacks in Figure 1 is used for Topology 2 with RRC-based architecture.

## 2.2	UE reader authorization

NGAP impact

Regarding to UE reader authorization, there are the following interim conclusions captured in R19 SA2 TR 23.700-13:

SA2 TR 23.700-13 8.1.3.4 Interim Conclusion for authorization The following interim conclusion are agreed for the UE Reader authorization in Topology 2: - The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader. - The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader. Editor's note: How to authorize the UE Reader will be determined later.

RAN TR 38.769 also captured the following descriptions about UE reader authorization:

RAN TR 38.769 6.3.6 Topology 2 aspects on the interface between UE reader and RAN … It is assumed that the intermediate UE authorization is performed by upper layers, according to [151] and [152]. It is not studied, from RAN2 signalling perspective, the scenario to support bistatic mode of operation (e.g., D2T2-A1). 6.4.2.1 Solutions for Topology 2 6.4.2.1.0 General … Only if the UE is authorized to perform the A-IoT service, the UE can communicate with the A-IoT device and be configured with AIoT radio resources controlled by the A-IoT enabled gNB.

From SA2 interim conclusions, we can see that the AMF determines whether the UE is authorized to work as reader. And according to RAN TR 38.769, UE reader authorization is performed by upper layers. Based on the RRC based architecture, in our opinion, the gNB serving the UE reader should be able to decide allocation of the AIoT radio resources based on the authorization status of a UE reader.

To achieve that, the UE reader firstly should include the reader indicator in the RRC Connection Setup Complete  message sent to its serving gNB, so that the gNB can include the UE reader indicator to AMF via the INITIAL UE MESSAGE message over NG interface. Next, the AMF carries out the authorization to the UE reader, i.e., determines whether the UE is authorized to work as reader based on UE reader indicator received and subscription data of the UE Reader, after which the AMF can send the UE reader authorization status indication (e.g., authorized or not authorized) in INITIAL CONTEXT SETUP REQUEST message sent to the gNB serving the UE reader. Then the gNB can determine whether to allocate the AIoT radio resources to the UE.

Proposal 2-1:	The UE sends the reader indicator to gNB via RRC message.

Proposal 2-2:	Upon the receiving the UE reader indicator, the gNB includes the UE reader indicator in NGAP message sent to AMF.

Proposal 2-3:	AMF indicates the UE reader authorization status to the gNB via NGAP message after authorization to the UE reader.

F1AP impact

When considering the CU-DU split architecture in A-IoT enabled gNB in Topology 2, if the gNB-DU is responsible for allocating and coordinating usage of A-IoT radio resources, upon the gNB-CU receives the UE reader authorization status indication from AMF, the gNB-CU can send A-IoT radio resource allocation indication or UE reader authorization status indication in F1AP message to the gNB-DU, such that the gNB-DU determines whether to allocate A-IoT radio resources for the UE reader.

Proposal 2-4:	Upon the gNB-CU receives the UE reader authorization status from AMF, the gNB-CU can indicate to the gNB-DU whether to allocate A-IoT radio resource via F1AP message.

## 2.3	UE reader selection

Regarding to UE reader selection, RAN TR 38.769 captured the following descriptions about UE reader selection:

RAN TR 38.769 6.3.6 Topology 2 aspects on the interface between UE reader and RAN For the UE reader selection, it is up to RAN3 and SA2 on whether BS or CN selects the UE reader(s) and what information is shared between BS and CN. From RAN2 perspective, it is assumed, at least for RRC based solution, there may be radio related reasons for the BS to be involved in the UE reader selection. 6.4.2.1 Solutions for Topology 2 6.4.2.1.0 General … In RRC based solution, Reader selection may need coordination between A-IoT-enabled gNB and A-IoT CN. NOTE 3: How to perform reader selection needs further discussions.

In our view, in Topology 2 RRC based solution, since the gNB is aware of the ongoing A-IoT services, we believe that the reader selection is similar as that in Topology 1, the difference is that readers are mobile in Topology 2. Thus, we understand that the UE reader selection can be done by either the gNB or the CN or both. In Topology 1, if the UE reader selection is performed by the A-IoT enabled gNB, AIOTF can provides the A-IoT Area ID(s) via Requested Service Area Information IE to the gNB to perform UE reader selection. If the UE reader selection is performed by the AIOTF, the AIOTF can provide the selected UE reader list to the gNB also via the Requested Service Area Information IE.

CR R3-255966 agreed by RAN3 #### 9.3.3.xx3 Requested Service Area Information This IE includes the Requested Service Area Information for the A-IoT inventory. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Requested Reader List; 0..1. >Requested Reader Item; 1..<maxnoofReaders>. >>Global gNB ID; M; 9.3.1.6. >>Reader Index; M; 9.3.1.aa1. Requested A-IoT Area List; 0..1. >Requested A-IoT Area Item; 1..<maxnoofAIoTAreas>. >>A-IoT Area ID; M; 9.3.1.aa2. Range bound; Explanation. maxnoofReaders; Maximum no. of Readers. Value is 65535.. maxnoofAIoTAreas; Maximum no. of A-IoT Areas. Value is 256..

Topology 2 can reuse the mechanism of reader selection in Topology 1. To enable Topology 2 reusing this Requested Service Area Information IE, the AIOTF first needs to obtain the UE reader index which is unique within the gNB. The UE reader index should be allocated by the gNB serving the UE. Regarding how to obtain the UE reader index by AIOTF, in our understanding, upon the receiving the UE reader indicator in RRC Connection Setup Complete message, the gNB allocates the UE reader index to this UE and includes the UE reader index together with the UE reader indicator in the INITIAL UE MESSAGE message sent to AMF. Then the AMF can forward the UE reader index to AIOTF.

Proposal 3-1:	The UE reader can be selected by AIoTF or gNB.

Proposal 3-2:	AIOTF can provide the AIoT area ID(s) via Requested Service Area Information IE to the gNB for the gNB to perform UE reader selection, or it provides the selected UE reader list to the gNB.

Proposal 3-4:	gNB allocates index to each UE reader and includes the UE reader index in NGAP message sent to AMF.

## 2.4	Inventory procedures in Topology 2

### 2.4.1		Inventory procedure in Topology 2

A candidate inventory procedure for RRC-based architecture in D2T2-B was captured in TR 38.769 [2] based on the R19 SI discussion.

Figure 6.5.3.1.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

Proposal 4-1: 	RAN3 adopts the Inventory procedure captured in RAN TR 38.769 as baseline Inventory procedure in Topology 2 RRC-based solution.

It’s assumed that the relevant NGAP messages containing the AIoT information between gNB and AIoTF can be reused.

Proposal 4-2:	The INVENTORY REQUEST/RESPONSE/REPORT NGAP messages including the AIoT information between gNB and AIoTF in Topology 1 can be reused in Topology 2.

### 2.4.2		NG inter-working with Uu in Inventory Procedure

Inventory Request

In Topology 2 RRC based solution, after receiving the NGAP Inventory Request message from AMF, the gNB sends the RRC inventory request message to UE reader to trigger the UE reader performing the inventory procedure at the A-IoT radio interface. A new RRC message may be used. The potential content of the RRC inventory request message can be discussed.

Recalling the Topology 1, the NGAP INVENTORY REQUEST message towards the gNB includes the following information:

INVENTORY REQUEST IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. Inventory Request Transfer; M; OCTET STRING; Containing the Inventory Request Transfer IE specified in subclause 9.3.x.1; YES; reject. 9.3.x.1 Inventory Request Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. A-IoT Device Identification Requested; M; 9.3.3.xx5; YES; reject. Requested Service Area Information; M; 9.3.3.xx3; YES; reject. Inventory Assistance Information; M; 9.3.3.xx4; YES; reject. Follow on Command Indication; O; ENUMERATED (true, ...); This IE indicates that there is command(s) to be transmitted.; YES; reject.

Transaction Identifier

The issue is whether to support multiple on-going inventory procedures over Uu interface between gNB and a specific UE reader. Note that there is no parallel procedures over the A-IoT radio interface, it’s up to reader implementation to execute the service one by one.

If parallelized inventory procedures are supported over Uu interface, an identifier is needed for identifying different on-going procedures. In that case, when receiving the Inventory Request from CN, the gNB should generate a transaction ID used to identify over Uu interface the inventory procedure which is identified by the A-IoT Correlation Identifier and the AIOTF Identity over NGAP. Then gNB can include the transaction ID in the RRC Inventory Request to UE reader, so that the same transaction ID can be included in the following RRC Inventory Response and RRC Inventory Report.

A-IoT Device Identification Requested

It is straightforward to be aware by the UE reader, because the UE reader needs to page the targeted devices.

Requested Service Area Information

In R19, the requested service area information is used by the gNB to select the readers co-located within the gNB, because one gNB maybe have multiple readers. When considering R20, this information is useless to UE reader, because whether to perform the UE reader functionality is under the gNB/CN control.

Inventory assistance information

In R19, the CN may provide the assistance information to the reader, for example, the approximate number of target A-IoT devices that may be involved in the inventory procedure for the efficient RACH resource allocation by the reader, or the expected D2R message size to support not segmented D2R transmission of the device ID. Such assistance information would also be sent to UE reader.

Content related to A-IoT radio resource

Since the gNB is aware of A-IoT service for RRC based solution, after receiving the NGAP INVENTORY REQUEST message, the gNB can allocate and coordinate usage of A-IoT radio resources. Therefore, the A-IoT radio resource configuration can be included in the RRC inventory request message.

The exact information to be sent to the UE reader via the RRC Inventory Request will be discussed and concluded by RAN2.

In summary, we propose that,

Proposal 4-3: After receiving the NGAP Inventory Request message, the gNB sends the RRC Inventory Request to the selected UE reader to trigger the UE reader performing the inventory procedure at the A-IoT radio interface.

Observation 4-1: The content in the RRC Inventory Request can include the following information,

- Transaction Identifier;

- A-IoT Device Identification Requested information;

- Inventory assistance information;

- A-IoT radio resource configuration.

Inventory Response

Regarding Step 2a (i.e., RRC Inventory Response) in Figure 6.5.3.1.1-1 captured in RAN TR 38.769, when receiving the RRC inventory Request UE will always accept the inventory service and respond to gNB. Since it’s up to gNB whether to provide the A-IoT radio resources to the UE reader, if it is provided, the UE reader should follow the gNB control to perform the A-IoT service.

When gNB receiving the RRC Inventory Response from UE reader, it sends the NGAP Inventory Response message to CN. The gNB may still reject the A-IoT service request from CN due to that the A-IoT radio resources cannot be allocated to UE reader.

Proposal 4-4: After receiving the RRC Inventory Response message from UE reader, the gNB sends the NGAP INVENTORY RESPONSE/REJECT message to CN.

Inventory Report

After inventory over the AIoT radio interface, the UE reader sends RRC Inventory Report to gNB. After receiving inventory result reported from the A-IoT enabled UE, the A-IoT-enabled gNB can send one or multiple NGAP INVENTORY REPORT messages to AMF including the received inventory results.

Recalling the NGAP INVENTORY REPORT message in R19,

9.2.x.4 INVENTORY REPORT IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. Inventory Report Transfer; M; OCTET STRING; Containing the Inventory Report Transfer IE specified in subclause 9.3.x.4; YES; reject. 9.3.x.4 Inventory Report Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description. A-IoT Correlation Identifier; M; 9.3.3.xx1. Global gNB ID; M; 9.3.1.6. Reader Report List; 1. >Reader Report Item; 1..< maxnoofReaders>. >>Reader Index; M; 9.3.1.aa1. >>Device Report List; 1. >>>Device Report Item; 1..<maxnoofDevices>. >>>>A-IoT NAS PDU; M; OCTET STRING. >>>>RAN A-IoT Device NGAP ID; O; 9.3.3.yy1. Inventory Complete Indication; O; ENUMERATED (true, ...).

Transaction Identifier

The intention is to identify different on-going procedures over Uu interface if parallelized inventory procedures are supported over Uu interface.

Global gNB ID/Reader index

The purpose of introducing Global gNB ID/Reader index is to enable the device positioning at the granularity of reader level. This is meaningless for RRC Inventory Report message.

A-IoT NAS PDU (Device ID info)

The A-IoT NAS PDU needs to be included in the RRC inventory report, which is the response to the inventory service.

Inventory Complete Indication

For Topology 2, similar functionality should be supported, i.e., if the gNB provides Approximate Number of Target A-IoT Devices via RRC Inventory Request message to the UE reader, then the UE reader can estimate whether the on-going inventory procedure is completed. If the current inventory procedure is completed, the UE reader can provide the inventory complete indication in the RRC inventory report so that the gNB can forward this information to the AIoTF.

Based on above summary, it is proposed that,

Proposal 4-5: At receiving the RRC Inventory Report from the UE reader, the gNB sends NGAP INVENTORY REPORT messages to AMF.

Observation 4-2: The RRC Inventory Report can include the following information,

- Transaction Identifier;

- A-IoT NAS PDU (Device ID info);

- Inventory Complete Indication.

## 2.5	Command after Inventory procedures in Topology 2

### 2.5.1		A-IoT Command in Topology 2

A candidate Command after Inventory procedure for RRC-based architecture in D2T2-B was captured in TR 38.769 [2] based on the R19 SI discussion.

Figure 6.5.3.2-1: Message flow for A-IoT Command in Topology 2 (RRC-based solution)

Proposal 5-1: 	RAN3 adopts the Command after Inventory procedure captured in RAN TR 38.769 as baseline Command procedure in Topology 2 RRC-based solution.

It’s assumed that the relevant NGAP messages containing the AIoT information between gNB and AIoTF can be reused.

Proposal 5-2: 	The COMMAND REQUEST/RESPONSE NGAP messages including the AIoTF information between gNB and AIoTF can be reused in Topology 2.

### 2.5.2		A-IoT context for Command after Inventory procedure

In Command after Inventory procedure in Topology 2, the issue is how to establish per device context in gNB and in UE respectively. To identify each device, the UE reader should allocate the Device ID which is unique within the UE reader for each device and use it over RRC, while the gNB should allocate the NGAP ID for each device. And gNB should perform a transition between the device NGAP ID and the device ID used over RRC during the command procedure.

Observation 5-1:	In Command after Inventory procedure in Topology 2, the gNB should perform a transition between the device NGAP ID and the device RRC ID which is allocated by the UE reader..

For UE and gNB to allocate the device ID, similar to Topology 1, if the Follow on Command Indication IE set to "true" is included in the RRC Inventory Request message, the UE shall consider that there will be command to be transmitted to the device(s), and shall allocate the Device ID upon receiving the NAS PDU from device. The UE should include the Device ID in the RRC Inventory Report message(s) sent to gNB. Upon receiving the RRC inventory report message containing the Device ID, the gNB shall consider that there will be command to be transmitted to the device(s), and shall allocate NGAP ID for the device. The gNB includes the NGAP ID in the RAN A-IoT Device ID IE of Device Report Item IE for that device within the NGAP INVENTORY REPORT message(s).

The Device ID used over RRC shall also be included within the RRC Command Request message and RRC Command Response message. The NGAP ID allocated for the inventoried device is used in the following on NGAP Command Request message.

Proposal 5-3: 	Upon receiving the RRC inventory report containing the RRC Device ID, the gNB shall  allocate the NGAP ID for that device and include it in RAN A-IoT Device ID IE in the Device Report Item IE for that device within the NGAP INVENTORY REPORT message(s).

### 2.5.3		NG interworking with Uu in Command Procedure

After receiving COMMAND REQUEST NGAP message for a device, the gNB sends a RRC Command Request to the corresponding UE reader to trigger it deliver the AIoT NAS PDU to the device.

Command Request

Regarding what information that needs to be forwarded to UE reader by the RRC command request message, we can also take R19 as baseline,

9.2.x.5 COMMAND REQUEST IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1; YES; reject. Command Request Transfer; M; OCTET STRING; Containing the Command Request Transfer IE specified in subclause 9.3.x.5.; YES; reject. 9.3.x.5 Command Request Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1; YES; reject. A-IoT NAS PDU; M; OCTET STRING; YES; reject. Command Assistance Information; O; 9.3.3.yy3; YES; reject.

Transaction Identifier

The intention is to identify different on-going procedures over Uu interface if parallelized inventory procedures are supported over Uu interface.

A-IoT Device ID used by RRC

As described in section 2.1, a Device ID used over RRC needs to be aware by UE reader for which device the command is applied.

A-IoT NAS PDU

This is the upper layer command (i.e., read, write or disable) for UE reader to be sent to the corresponding device addressed by the associated AS ID.

Command Assistance Information

This command assistance information includes the expected D2R message size to assistance the efficient scheduling by the reader. So it’s better to be aware by the UE reader.

In summary, we propose that,

Proposal 5-4: After receiving COMMAND REQUEST NGAP message for a device, the gNB sends a RRC Command Request to the corresponding UE reader to trigger it deliver the AIoT NAS PDU to the device.

Observation 5-2: The RRC command request can include the following information,

- Transaction Identifier;

- A-IoT Device ID used by RRC;

- A-IoT NAS PDU (upper layer command);

- Command assistance information.

Command Response

If the command result is received from A-IoT device, the UE sends the command result to gNB via RRC Command Response message. Then, the gNB forwards the command result to AMF via the COMMAND RESPONSE NGAP message.

Regarding the RRC command response, according to the NGAP COMMAND RESPONSE message,

9.2.x.6 COMMAND RESPONSE IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1; YES; reject. Command Response Transfer; M; OCTET STRING; Containing the Command Response Transfer IE specified in subclause 9.3.x.6.; YES; reject. Criticality Diagnostics; O; 9.3.1.3; YES; ignore. 9.3.x.6 Command Response Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description. A-IoT Correlation Identifier; M; 9.3.3.xx1. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1. A-IoT NAS PDU; M; OCTET STRING. Criticality Diagnostics; O; 9.3.1.3.

Transaction Identifier

The intention is to identify different on-going procedures over Uu interface if parallelized inventory procedures are supported over Uu interface.

A-IoT Device ID used by RRC

A-IoT Device ID used by RRC needs to be included in the RRC command response to identify which device is responding the command.

A-IoT NAS PDU (upper layer response message)

This is the upper layer response to be included in the RRC command response message.

In summary, we propose that,

Proposal 5-5: After receiving the RRC Command Response with the AIoT NAS PDU for a device, the gNB sends the NGAP Command Response message to AMF.

Observation 5-5: The RRC command response can include the following information,

- Transaction Identifier;

- A-IoT Device ID used by RRC;

- A-IoT NAS PDU (upper layer response message).

## 2.6	Inter-gNB RRC-connected UE Reader Mobility

### 2.6.1	UE reader authorization status indication

As discussed in Section 2.2, for UE reader authorization, AMF can indicate the UE reader authoriation status during UE Context Setup to gNB over NGAP. There is a issue that whether the Target gNB (in case Handover procedure) or new gNB (in case RRC re-establishment procedure) needs to know UE reader authorization status.

In our understanding, the Target gNB (in case Handover procedure) or new gNB (in case RRC re-establishment procedure) also needs to know the UE reader authorization status, in order to determine whether AIOT radio resources should be allocated to the UE reader or not. In NG handover, AMF needs to provide the UE reader authorization status to target gNB during the Handover Resource Allocation procedure. In Xn handover, Source-gNB needs to provide the UE reader authorization status to target gNB during the Handover Preparation procedure. In RRC re-establishment, the old gNB needs to provide the UE reader indicator to new gNB during the Retrieve UE Context procedure.

Proposal 6-1: 	During inter-gNB UE reader mobility:

- In NG handover, AMF needs to provide the UE reader authorization status to target gNB.

- In Xn handover, Source-gNB needs to provide the UE reader authorization status indication to target gNB.

- In RRC re-establishment, the old gNB needs to provide the UE reader authorization status to new gNB.

### 2.6.2	UE reader indicator

As discussed in Section 2.2, for UE reader authorization, gNB can include a UE reader indictor within the Initial UE message to AMF over NGAP. There is a issue that whether the Target gNB (in case Handover procedure) or new gNB (in case RRC re-establishment procedure) needs to know a UE is an reader during inter-gNB mobility.

In our understanding, the Target gNB (in case Handover procedure) or new gNB (in case RRC re-establishment procedure) needs to know whether the relavant UE is an reader.  Because if the target gNB or new gNB does not support the functionality of AIoT, the target gNB or new gNB can reject the access of the UE reader. In NG handover, AMF needs to provide the UE reader indicator to target gNB during the Handover Resource Allocation procedure. In Xn handover, Source-gNB needs to provide the UE reader indicator to target gNB during the Handover Preparation procedure. In RRC re-establishment, the old gNB needs to provide the UE reader indicator to new gNB during the Retrieve UE Context procedure.

Proposal 6-2: 	During inter-gNB UE reader mobility:

- In NG handover, AMF needs to provide the UE reader indicator to target gNB.

- In Xn handover, Source-gNB needs to provide the UE reader indicator to target gNB.

- In RRC re-establishment, the old gNB needs to provide the UE reader indicator to new gNB.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

Proposal 1-1: 	RAN3 discuss indirect connectivity for Topology 2 with RRC-based architecture.

Proposal 1-2:	For Topology2 with RRC-based architecture, RAN3 confirms that A-IoT enabled gNB connects to AMF via NG-C interface and A-IoT specific information is included within NGAP.

Proposal 1-3:	The Protocol Stacks in Figure 1 is used for Topology 2 with RRC-based architecture.

Proposal 2-1:	The UE sends the reader indicator to gNB via RRC message.

Proposal 2-2:	Upon the receiving the UE reader indicator, the gNB includes the UE reader indicator in NGAP message sent to AMF.

Proposal 2-3:	AMF indicates the UE reader authorization status to the gNB via NGAP message after authorization to the UE reader.

Proposal 2-4:	Upon the gNB-CU receives the UE reader authorization status from AMF, the gNB-CU can indicate to the gNB-DU whether to allocate A-IoT radio resource via F1AP message.

Proposal 3-1:	The UE reader can be selected by AIoTF or gNB.

Proposal 3-2:	AIOTF can provide the AIoT area ID(s) via Requested Service Area Information IE to the gNB for the gNB to perform UE reader selection, or it provides the selected UE reader list to the gNB.

Proposal 3-4:	gNB allocates index to each UE reader and includes the UE reader index in NGAP message sent to AMF.

Proposal 4-1: 	RAN3 adopts the Inventory procedure captured in RAN TR 38.769 as baseline Inventory procedure in Topology 2 RRC-based solution.

Proposal 4-2:	The INVENTORY REQUEST/RESPONSE/REPORT NGAP messages including the AIoT information between gNB and AIoTF in Topology 1 can be reused in Topology 2.

Proposal 4-3:     After receiving the NGAP Inventory Request message, the gNB sends the RRC Inventory Request to the selected UE reader to trigger the UE reader performing the inventory procedure at the A-IoT radio interface.

Proposal 4-4:      After receiving the RRC Inventory Response message from UE reader, the gNB sends the NGAP INVENTORY RESPONSE/REJECT message to CN.

Proposal 4-5:    At receiving the RRC Inventory Report from the UE reader, the gNB sends NGAP INVENTORY REPORT messages to AMF.

Proposal 5-1: 	RAN3 adopts the Command after Inventory procedure captured in RAN TR 38.769 as baseline Command procedure in Topology 2 RRC-based solution.

Proposal 5-2: 	The COMMAND REQUEST/RESPONSE NGAP messages including the AIoTF information between gNB and AIoTF can be reused in Topology 2.

Proposal 5-3: 	Upon receiving the RRC inventory report containing the RRC Device ID, the gNB shall  allocate the NGAP ID for that device and include it in RAN A-IoT Device ID IE in the Device Report Item IE for that device within the NGAP INVENTORY REPORT message(s).

Proposal 5-4:      After receiving COMMAND REQUEST NGAP message for a device, the gNB sends a RRC Command Request to the corresponding UE reader to trigger it deliver the AIoT NAS PDU to the device.

Proposal 5-5:       After receiving the RRC Command Response with the AIoT NAS PDU for a device, the gNB sends the NGAP Command Response message to AMF.

Proposal 6-1: 	During inter-gNB UE reader mobility:

- In NG handover, AMF needs to provide the UE reader authorization status to target gNB.

- In Xn handover, Source-gNB needs to provide the UE reader authorization status indication to target gNB.

- In RRC re-establishment, the old gNB needs to provide the UE reader authorization status to new gNB.

Proposal 6-2: 	During inter-gNB UE reader mobility:

- In NG handover, AMF needs to provide the UE reader indicator to target gNB.

- In Xn handover, Source-gNB needs to provide the UE reader indicator to target gNB.

- In RRC re-establishment, the old gNB needs to provide the UE reader indicator to new gNB.

4. References

- RP-252894 Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2

- TR 23.700-30  Study on Architecture support of Ambient power-enabled Internet of Things (AIoT) - Phase 2  SA2

- SID SP-250834  NEW SID on Architecture support of Ambient power-enabled Internet of Things - Phase 2 SA2

- 3GPP TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR (Release 19)”, V19.0.0 (2024-12)