3GPP TSG RAN3 meeting #129bis   	         	                                R3-256776

Prague, Czech Republic, 13 – 17 October 2025

Agenda item: 	    11.2.2

Source: 	China Unicom

Title: 	Discussion on MRO enhancements for C-LTM

Document for:	Discussion

1	Introduction

At RAN#108, the new Work Item on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5 was agreed and the WID was revised at RAN#109 in[1] introducing the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] If needed, co-operate with RAN1, SA5.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-20.

2	Discussion

2.1	MRO for conditional LTM

Conditional LTM is supported by the UE when L1-based or L3-based LTM cell switch execution conditions are met. The procedure for C-LTM is as follows.

For C-LTM, when UE meets the configured conditions based on L1 or L3 measurements, it can perform the cell switch without the command from the gNB.

Both Conditional intra-CU LTM (Intra-gNB-DU) and Conditional intra-CU LTM (Inter-gNB-DU) are supported in Rel-19, which can be considered for Rel-20 MRO enhancements.

Proposal 1: Both Conditional intra-CU LTM (Intra-gNB-DU) and Conditional intra-CU LTM (Inter-gNB-DU) scenarios should be considered for MRO enhancements to conditional LTM.

Proposal 2: Both failure case and near failure case should be considered for MRO enhancements to conditional LTM.

Similar as CHO, the failure scenarios for C-LTM can use MRO for CHO as baseline.

Proposal 3: Rel-17 MRO for CHO scenarios can be considered as baseline for further study on conditional LTM failure scenarios.

The gNB-CU determines to initiate conditional LTM configuration with the candidate gNB-DU(s). If the candidate gNB-DU accepts the request of conditional LTM configuration, it provides a list of L1-based execution conditions generated for other candidate cells. The source gNB-DU is responsible for the L1-based execution condition. For failures due to inappropriate L1-based execution condition, CU initiates analysis and may forwards the RLF report to the source gNB-DU responsible for the failure.

Proposal 4:The source gNB-DU is responsible for the failure caused by inappropriate L1-based execution condition.

3	Conclusion

Proposal 1: Both Conditional intra-CU LTM (Intra-gNB-DU) and Conditional intra-CU LTM (Inter-gNB-DU) scenarios should be considered for MRO enhancements to conditional LTM.

Proposal 2: Both failure case and near failure case should be considered for MRO enhancements to conditional LTM.

Proposal 3: Rel-17 MRO for CHO scenarios can be considered as baseline for further study on conditional LTM failure scenarios.

Proposal 4:The source gNB-DU is responsible for the failure caused by inappropriate L1-based execution condition.

4   References

- RP-234038: Revised WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5; TSG RAN Meeting #109, Beijing, China, Sep 15-18, 2025