3GPP TSG-RAN WG3 Meeting #129bis	R3-256783

Prague, Czech Republic, Oct. 13th-17th, 2025

Source:				NEC

Agenda item: 	11.2.2 Intra-CU conditional LTM

Title: 	MRO for Intra-CU conditional LTM

Document for:				Discussion and decision

- Introduction

For Rel-20 Data collection for SON, the solution or the following objective is to be discussed and decided:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

In this paper, we would like to provide our initial thinking on the MRO enhancement for intra-CU conditional LTM.

- Discussion

In CHO, the UE stores and reports to the network information of CHO candidate cells and the execution condition configuration of the candidate cells. However, in Rel-19 MRO for intra-CU LTM, whether the UE needs to log candidate cell list was discussed, and it was agreed that network-based solution is used for LTM candidate cell list, because the cell list is going to be stored in the UE context in the network side for a long time due to the support of subsequent LTM, and therefore there is no need for the UE to store such information. Similarly, for CLTM, there is no need for the UE to log the cell list and execution condition associated with the LTM candidate cell.

Proposal 1. The UE does not log the execution condition (including L1 based and L3 based) associated with the LTM candidate cell, i.e., network-based solution is used.

For Rel-19 MRO for intra-CU LTM, for failures due to inappropriate cell switch triggering, CU initiates analysis and forwards the failure information to the DU responsible for the failure. For failure of CLTM, although it the UE triggered to LTM, but the execution condition is set by the source DU. Therefore, we think the same F1 signalling and procedure can be used for intra-CU LTM, such that the source DU can adjust the execution condition based on the failure information from the CU.

Proposal 2. The F1 signalling and procedure for intra-CU LTM failure (i.e., too early LTM, too late LTM and LTM to wrong cell) for can be reused for intra-CU CLTM.

In Rel-19 MRO for intra-CU LTM, the target gNB-DU can identify the BFR shortly after a successful LTM cell switch, and send the recovery beam information to the source gNB-DU via the gNB-CU. For intra-CU CLTM, BFR shortly after a successful LTM cell switch can also happen, the difference is that the beam selected during the CLTM is determined by the UE based on the fulfilled execution condition set by the source gNB-DU. We understand that this BFR shortly after successful CLTM may be also caused due to bad execution condition setting, and therefore MRO for this case can be supported. Since for CLTM, the source gNB-DU has no information of the beam selected by the UE, RAN3 should discuss whether any different scheme or enhancement should be supported for this case.

Proposal 3. Support MRO for BFR happened in the UE shortly after a successful intra-CU CLTM cell switch.

In Rel-19 MRO for intra-CU LTM, the target gNB-DU can identify that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell, and send the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU. For intra-CU CLTM, the same case also exist with the difference is that the TA is send and maintain to the UE by MAC CE before the execution condition is fulfilled, and the beam selected by the UE is based on the execution condition set by the source DU. And RAN3 should discuss whether any different scheme or enhancement should be supported for this case.

Proposal 4. Support MRO for the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell after intra-CU LTM cell switch.

In Rel-19 MRO of intra-CU LTM, the ping-pong issue of LTM cell switch was discussed and solved by sending from gNB-CU to gNB-DU the UE history information with a ping-pong indicator using CELL SWITCH NOTIFICATION message. For intra-CU CLTM, the ping-pong cell switch issue also exists. However, there is no CELL SWITCH NOTIFICATION message transmitted from the gNB-CU to the candidate gNB-DU, and RAN3 should discuss whether and how to indicate to the gNB-DU the UE history information and the ping-pong indicator.

Proposal 5. Support MRO for ping-pong issue of intra-CU CLTM.

# 3. Conclusion

In this contribution we made the following proposals:

Proposal 1. The UE does not log the execution condition (including L1 based and L3 based) associated with the LTM candidate cell, i.e., network-based solution is used.

Proposal 2. The F1 signalling and procedure for intra-CU LTM failure (i.e., too early LTM, too late LTM and LTM to wrong cell) for can be reused for intra-CU CLTM.

Proposal 3. Support MRO for BFR happened in the UE shortly after a successful intra-CU CLTM cell switch.

Proposal 4. Support MRO for the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell after intra-CU LTM cell switch.

Proposal 5. Support MRO for ping-pong issue of intra-CU CLTM.