3GPP TSG-RAN WG3 Meeting #129 bis	R3-256793

Prague, Czech Republic, 13-17 October 2025

Agenda Item:		12.2.1

Source:		NEC

Title:			(pCR for TR38.745) Multi-hop UE trajectory across gNBs

Document for:		Discussion and decision

# Introduction

A new Rel-20 SI on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3 [1] was approved for 5G-A and the following objectives are included.

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

- multi-hop UE trajectory across gNBs;

- intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

In this contribution, we provide our initial considerations on multi-hop UE trajectory across gNBs and the draft version TP to TR38.745.

# Discussion

2.1	Use case description

UE trajectory prediction has already been studied for two releases, and in Rel-18 and Rel-19, UE trajectory prediction and UE trajectory feedback are limited to the first-hop target NG-RAN node. There was some solution discussion on multi-hop UE trajectory across gNBs in Rel-19 SI, but this topic was ruled out due to no consensus.

UE trajectory prediction is an output of AI/ML-based mobility optimization and can be used as an input for AI/ML based load balancing.

Multi-hop UE trajectory prediction, which consists of a list of cells belonging to one or more gNBs where the UE is expected to connect, can provide gNB the information about further needed resource and may impact mobility decision. In order to generate multiple hop UE trajectory, gNB needs to collect multi-hop UE trajectory. To achieve this, the following two solutions were proposed:

- Hop by hop based on current mechanism with the effort to keep UE context for more time in RAN node

- End to End solution with the risk that the connection between the source and target is not available

However, two issues were identified: 1) gNB needs to keep large number of UE context with it, which may bring large signalling cost among gNBs and increase network complexity; 2) multi-hop UE trajectory may not be accurate enough to help improve mobility performance.

In this SI, we can continue to evaluate the potential solutions and standard impact with the aim of avoid increasing network complexity or signalling cost.

Besides, in order to assist mobility decision in finer granularity, beam level (multi-hop) UE trajectory can also be considered.

2.2	Solutions and standard impacts

2.2.1	UE trajectory collection

First, we recall the signalling procedure for one-hop UE trajectory:

Step 1. gNB1 triggers Data Collection Reporting Initiation procedure with gNB2, to request from gNB2 the reporting of UE trajectory. UE Trajectory Collection Configuration IE containing Collection Time Duration for UE Trajectory and Number of Visited Cells is present in the DATA COLLECTION REQUEST message

Step 2. During handover procedure (UE from gNB1 to gNB2), Data Collection ID IE is contained in the HANDOVER REQUEST message from gNB1 to gNB2.

Step 3. After UE handovers to gNB2, gNB2 performs UE trajectory collection and reporting via DATA COLLECTION UPDATE message. gNB1 still keeps UE context until Data collection completed.

Data Collection Reporting Initiation procedure and Data Collection Reporting procedure use non-UE-associated signalling and these procedures are not linked to a specific UE. After one-hop, both the Collection Time Duration for UE Trajectory and Number of Visited Cells need to be updated for the following handovers and the updates are UE specific, however, if legacy Data Collection Reporting Initiation procedure is triggered for each UEs, this will lead to large and unacceptable signalling cost. Therefore, we suggest to use UE-associate signalling for the UE trajectory collection afterwards. For example, source gNB in between updates the Collection Time Duration for UE Trajectory and Number of Visited Cells in UE Trajectory Collection Configuration, and sends it in the Handover Request message to target gNB for successive UE trajectory collection.

Besides, if all the involved gNBs need to be keep UE context after handover execution and UE context release needs per UE handling, this will increase network complexity. Therefore, we suggest to adopt the End to End solution that only requires the triggered gNB to keep UE context. To achieve this, gNB1 ID also needs to be sent together with UE Trajectory Collection Configuration.

The proposed procedures are showing as below:

Figure 1: Multi-hop UE trajectory collection

In addition, since continuous MDT has been specified in Rel-19 and therefore UE trajectory can be collected across RRC states, this can also facilitate multi-hop UE trajectory prediction.

Proposal 1: Adopt End to End solution for multi-hop UE trajectory collection, including:

- add Triggered gNB ID in the UE Trajectory Collection Configuration IE;

- add UE Trajectory Collection Configuration IE in Handover Request message.

2.2.2	Locations for AI/ML Model

In Rel-18 and Rel-19, high-level principles and functional framework for AI/ML for NG-RAN were agreed and two procedures, i.e., Data Collection Reporting Initiation procedure and Data Collection Reporting procedure were adopted for AI/ML data collection. In Rel-20, we need to take the agreements made in previous releases into account. For example, regarding to the locations for AI/ML Model Training and AI/ML Model Inference, the following deployment scenarios should still be applied:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB (gNB-CU(-CP) in case of split architecture).

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB (gNB-CU(-CP) in case of split architecture).

However, if beam level (multi-hop) UE trajectory as proposed above is agreed to be studied, AI/ML Model Inference located in gNB-DU should also be considered.

Proposal 2: Study beam level (multi-hop) UE trajectory with AI/ML Model Inference located in gNB-DU.

2.2.3	Input, output and feedback

To predict the multi-hop UE trajectory, a gNB may need the following information as input data:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

From the neighbouring RAN nodes:

-	UE Mobility History Information

-	Position, QoS parameters and the performance information of historical HO-ed UE (e.g., loss rate, delay, etc.)

-	UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism

From the local node:

-	UE trajectory prediction

The output generated by AI/ML model inference is the UE trajectory prediction, e.g., latitude, longitude, altitude, cell ID of UE over a future period of time.

Legacy UE performance feedback for those UEs handed over from the source gNB and SON reports (e.g., RLF, CEF, RA) can be used as feedback data.

2.2.4	Potential standard impacts

According to the analysis above, following standard impacts are foreseen for subsequent Rel-20 normative work:

Xn interface:

-	Enhanced existing procedure to collect information between gNBs:

-	Actual multi-hop UE trajectory

-	Predicted multi-hop UE trajectory

TP to TR38.745 which captures the above information is in Annex.

Proposal 3: Agree the TP to TR38.745 for AI/ML based intra-CU LTM in Annex.

# Proposals

In this contribution, we provide our initial considerations on multi-hop UE trajectory across gNBs, including the input/output/feedback and the potential standard impact, and the following proposals were drawn from the analysis.

Proposal 1: Adopt End to End solution for multi-hop UE trajectory collection, including:

- add Triggered gNB ID in the UE Trajectory Collection Configuration IE;

- add UE Trajectory Collection Configuration IE in Handover Request message.

Proposal 2: Study beam level (multi-hop) UE trajectory with AI/ML Model Inference located in gNB-DU.

Proposal 3: Agree the TP to TR38.745 for AI/ML based intra-CU LTM in Annex.

# Reference

[1] RP-252867, Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3

# Annex	TP for TR38.745

<Start of change>

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor Note: Capture the description of use case

In Rel-18 and Rel-19, UE trajectory prediction and UE trajectory feedback are limited to the first-hop target NG-RAN node.

Multi-hop UE trajectory prediction, which consists of a list of cells belonging to one or more gNBs where the UE is expected to connect, can provide gNB the information about further needed resource and may impact mobility decision. In order to generate multiple hop UE trajectory, gNB needs to collect multi-hop UE trajectory. To achieve this, the following two solutions were proposed:

- Hop by hop based on current mechanism with the effort to keep UE context for more time in RAN node

- End to End solution with the risk that the connection between the source and target is not available

However, two issues were identified: 1) gNB needs to keep large number of UE context with it, which may bring large signalling cost among gNBs and increase network complexity; 2) multi-hop UE trajectory may not be accurate enough to help improve mobility performance.

The potential solutions and standard impact need to be studied with the aim of avoid increasing network complexity or signalling cost.

Besides, in order to assist mobility decision in finer granularity, beam level (multi-hop) UE trajectory can also be considered.

### 4.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB (gNB-CU(-CP) in case of split architecture).

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB (gNB-CU(-CP) in case of split architecture).

AI/ML Model Inference located in gNB-DU can also be considered for beam level (multi-hop) UE trajectory.

#### 4.1.2.2	Input data of multi-hop UE trajectory

To predict the multi-hop UE trajectory, a gNB may need the following information as input data:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

From the neighbouring RAN nodes:

-	UE Mobility History Information

-	Position, QoS parameters and the performance information of historical HO-ed UE (e.g., loss rate, delay, etc.)

-	UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism

From the local node:

-	UE trajectory prediction

#### 4.1.2.3	Output data of multi-hop UE trajectory

The following information can be generated as output:

-	Multi-hop UE trajectory prediction, e.g., list of latitude, longitude, altitude, cell ID of UE over a future period of time

#### 4.1.2.4	Feedback of multi-hop UE trajectory

To optimize the performance of multi-hop UE trajectory prediction, the following feedback can be considered to be collected from gNBs:

-	Legacy UE performance feedback for those UEs handed over from the source gNB

-	SON reports (e.g., RLF, CEF, RA)

#### 4.1.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-20 normative work compared with what was specified during Rel-18 and Rel-19.

Xn interface:

-	Enhanced existing procedure to collect information between gNBs:

-	Actual multi-hop UE trajectory

-	Predicted multi-hop UE trajectory

<End of change>