3GPP TSG-RAN WG3 Meeting #129 bis	R3-256795

Prague, Czech Republic, 13-17 October 2025

Agenda Item:		12.2.3

Source:		NEC

Title:	(pCR for TR38.745) Other AI/ML based handover enhancements

Document for:		Discussion and decision

# Introduction

A new Rel-20 SI on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3 [1] was approved for 5G-A and the following objectives are included.

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

- multi-hop UE trajectory across gNBs;

- intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

In this contribution, we provide our initial considerations on other AI/ML based handover enhancements and the draft version TP to TR38.745.

# Discussion

Regarding other AI/ML based handover enhancements, we suggest to consider two topics: inter-CU LTM and enhanced NG handover.

2.1	Inter-CU LTM

LTM supports both intra-CU and inter-CU cases. Inter-CU LTM applies a similar procedure for intra-CU LTM as described in [2], with the following additional standard impacts on Xn interface and F1 interface:

- LTM predication needs to send from source gNB-CU to candidate gNB-CU(s) for LTM candidate cell preparation.

- Candidate gNB-CU needs to coordinate each candidate cells with its candidate gNB-DUs.

- UE performance feedback collected by target gNB-CU needs to send to source gNB-CU for model performance monitor.

Based on the above analysis, specifying inter-CU LTM case together with intra-CU LTM case can generate one complete solution for both LTM cases, but without spending much more standard effort. Therefore, we suggest to study AI/ML based inter-CU LTM in Rel-20 SI.

Proposal 1: Study AI/ML based inter-CU LTM in Rel-20 SI and consider the following standard impact on XN interface and F1 interface for normative work:

- Exchange LTM predication for LTM candidate cell preparation

- UE performance feedback, including beam level UE performance, after LTM execution for model performance monitor

2.2	AI/ML based NG handover

In Rel-18 and Rel-19, AI/ML based mobility optimization only focus on Xn handover. However, in the case of Xn interface not available, if we still intend to achieve mobility optimization using AI/ML, collection and reporting of input data, output or feedback information needs to be performed over NG interface. Therefore, we suggest to study data collection over NG interface to enrich 5G AI/ML feature and facility NG handover. For example, Data Collection Reporting Initiation procedure and Data Collection Reporting procedure as specified over Xn interface need to be supported over NG interface.

Proposal 2: Study AI/ML based NG handover in Rel-20 SI, including data collection and reporting over NG interface.

TP to TR38.745 which captures the above information is in Annex.

Proposal 3: Agree the TP to TR38.745 for other AI/ML based handover enhancements in Annex.

# Proposals

In this contribution, we provide our initial considerations on other AI/ML based handover enhancements, and the following proposals were drawn from the analysis.

Proposal 1: Study AI/ML based inter-CU LTM in Rel-20 SI and consider the following standard impact on XN interface and F1 interface for normative work:

- Exchange LTM predication for LTM candidate cell preparation

- UE performance feedback, including beam level UE performance, after LTM execution for model performance monitor

Proposal 2: Study AI/ML based NG handover in Rel-20 SI, including data collection and reporting over NG interface.

Proposal 3: Agree the TP to TR38.745 for other AI/ML based handover enhancements in Annex.

# Reference

[1] RP-252867, Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3

[2] R3-25xxxx, AI/ML based intra-CU LTM, NEC

# Annex	TP for TR38.745

<Start of change>

## 4.3	Other handover enhancements

Editor Note: Identify other handover enhancments via AI/ML, e.g., inter-CU LTM

### 4.3.1	Inter-CU LTM

Inter-CU LTM applies a similar procedure intra-CU LTM as described in section 4.2. Following additional standard impacts on Xn interface and F1 interface are listed for subsequent Rel-20 normative work.

- Exchange LTM predication for LTM candidate cell preparation

- UE performance feedback, including beam level UE performance, after LTM execution for model performance monitor

### 4.3.2	AI/ML based NG handover

In Rel-18 and Rel-19, AI/ML based mobility optimization only focus on Xn handover. However, in the case of Xn interface not available, collection and reporting of input data, output or feedback information needs to be performed over NG interface. Therefore, Data Collection Reporting Initiation procedure and Data Collection Reporting procedure as specified over Xn interface need to be supported over NG interface.

<End of change>