3GPP TSG-RAN WG3 Meeting #129-bis	R3-256830

Prague, Czech Republic, Oct 13th - Oct 17th, 2025

Agenda item:	9.2.5

Source:	Qualcomm Incorporated

Title:	[Draft TP for TS 38.473, TS 38.423] Remaining issues in Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #129 [1]), this topic was considered complete from RAN3 point of view, and from this meeting onwards, the maintenance phase work will be pursued. In the last RAN3 meeting, all the main open issues were resolved, and the following agreements were reached.

RAN3 #129 agreements:

Inter-CU LTM:

Confirm to have the explicit Request for CSI-RS Resource Configuration indicator in the Handover Request message.

The candidate gNB/gNB-DU provides the CSI-RS Report configuration for CSI Acquisition separately via a new IE (e.g., refers to ltm-CSI-ReportConfig-r19) during the preparation phase.

For deactivation of SP CSI-RS of candidate cell(s) after the UE’s successful cell switch, the CSI-RS Coordination procedure is re-used, triggered by the previous serving gNB-CU after successful cell switch is confirmed, and toward the relevant candidate gNB-DU(s) and gNB-CU(s).

Agree to reuse LTM Configuration Update message to transfer Rel-19 set ID per candidate cell to the candidate CUs.

Once the UE XnAP association is setup, the source gNB includes the target NG-RAN node UE XnAP ID in the handover request message for any follow-up preparation.

The old source gNB can deliver the old target UE XnAP ID(s) to the new serving gNB via Cell Switch Notification and LTM Configuration Update message.

The source gNB sends the Data Forwarding Information as per-PDU session level to the candidate gNBs in the LTM CONFIGURATION UPDATE message.

Include the Tag ID Pointer and RACH resource request ID in TA Information Transfer in XnAP.

Include the TA values in Cell Switch Notification message.

For Rel-19 Set ID assignment, source gNB-CU sends the Rel-19 Set ID list per-node to candidate gNB-CUs in Handover Request message.

Addition Request message from the MN to the candidate SN.

A per cell indicator indicating complete candidate configuration is included in the SN Addition Request Acknowledge message.

To clarify that the “old target UE XnAP ID” is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

For CSI-RS configuration in F1AP and XnAP, including the CSI-RS resource configuration and the CSI-RS resource type in the UE Context Setup Response, UE Context Modification Response and Handover Request Acknowledge message.

For TCI State List configuration over the XnAP, adopt the similar solution as TEI18.

Include the explicit request for CSI-RS Resource Configuration for CSI acquisition indicator in the Handover Request message.

The candidate gNB/gNB-DU provides the CSI-RS Report configuration for CSI Acquisition separately via a new IE (e.g., refers to ltm-CSI-ReportConfig-r19) in the LTM Configuration Update Acknowledge and the UE Context Modification Response message.

For CSI-RS coordination procedure, include the mandatory CSI Resource Config ID (i.e. LTM-CSI-ResourceConfigId) in the both request and response message in F1AP and XnAP.

The CSI-RS-based beam measurement results should be sent from the CU to the DU in the F1AP CU-DU Mobility Initiation Request message.

Inter-CU LTM in DC

Turn the WA into agreement: Different candidate PSCells in the same SN can have different Rel-19 set IDs.

Do not support LTM modification on the already prepared LTM candidate PScells in DC scenario in Rel-19.

Support both Source SN-initiated and Candidate SN-initiated LTM Cancel,

For security update for inter-CU SCG LTM, the list of {KSN, sk-counter} is provided on the granularity of Rel-19 set ID.

The MN provides a list of R19 set IDs to the candidate SN via the SN Addition Request message. MN sends R19 set ID for the candidate cell along with a list of {KSN, sk-counter} in the SN Modification Request message.

The MN sends the R19 set ID for each candidate PSCell to the source SN and other candidate SNs via the SN Modification Request message.

The candidate PSCells to be cancelled is included in the SN Modification Required message.

The MN may inform the source SN about the cancellation of the prepared candidate PSCells at a candidate SN using SN modification request message.

To support inter- or intra-CU MCG LTM with SCG, introduce a new XnAP IE to indicate to the candidate SN that the SN Addition/Modification preparation procedure is triggered as part of a MCG LTM procedure.

To support inter- or intra-CU MCG LTM with SCG, introduce a new F1AP IE to indicate to the DU of a candidate SN that the UE Context Setup/Modification procedure is triggered as part of a MCG LTM procedure.

Remove the FFS: FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

SN Change Confirm message is used to notify the list of data forwarding addresses to the source SN.

The SCG reference configuration is provided by an implicit way in the CG-Config RRC container in the SN Addition Request Acknowledge message from the candidate SN to the MN, no RAN3 impact is foreseen.

In this contribution, we discuss the following remaining issues in Inter-CU LTM:

- Remaining issues on activation/deactivation of semi-persistent (SP) CSI-RS resources on candidate cells,

- Remaining issues on support of CSI-RS resources for LTM CSI acquisition on candidate cells,

- Remaining issues in the Early TA RACH configuration procedure,

- Remaining aspects on transfer of TA information in the Cell Switch Notification procedure.

# 2 Remaining Issue#1: For activation/deactivation of SP CSI-RS resources on LTM candidate cells

In this section, we discuss an issue that we found with the current Rel-19 signalling procedure for SP CSI-RS activation/deactivation and propose two options for a solution to the issue.

The current Rel-19 signalling procedure for activation/deactivation of SP CSI-RS resources of an LTM candidate cell can be summarized in the following observation.

Observation 1. In Rel-19 LTM, RAN3/RAN2 has defined the following signalling procedure for activation/deactivation of SP CSI-RS resources of an LTM candidate cell:

- The source DU indicates in signalling messages over the F1 and Xn interfaces to a candidate DU activation/deactivation of SP CSI-RS resources of a candidate cell.

- The candidate DU confirms the activation/deactivation in its response message to the source DU.

- The source DU then transmits the activation/deactivation indication to the UE in the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE.

The issue that we found is captured in the following observation.

Observation 2. In the response message to the source DU, the candidate DU does not provide TCI state information for the activated (or deactivated) SP CSI-RS resources, so that the source DU cannot provide the TCI state information along with the SP CSI-RS resource indications in the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE (shown below) to the UE.

We propose two possible options for a solution to this issue:

- Option 1 (dynamic): The candidate DU provides the TCI state information for the activated or deactivated SP CSI-RS resources, during the SP CSI-RS activation/deactivation procedure, i.e., in the CSI-RS Coordination procedures over the F1 and Xn interfaces, respectively.

  - In this option, the TCI state information, i.e., the TCI state IDs, are dynamically assigned by the candidate DU.

- Option 2 (semi-static): The TCI states for SP CSI-RS resources of LTM candidate cells are configured during the LTM preparation procedure.

  - In this option, the TCI state IDs are assigned or configured by the candidate DU during the LTM preparation procedure in semi-static manner. The candidate DU does not need to include the TCI state information in the response message to the source DU during the SP CSI-RS activation/deactivation procedure. But this will require changes to RRC as well and it is a simple RRC change.

Regarding Option 1, the following proposals provide the details.

Proposal 1a (Option 1, dynamic). The source gNB-DU initiates the activation (or deactivation) of SP CSI-RS resources of a candidate cell and provides the L1 measurement results of the candidate cell received from the UE to the candidate gNB-DU, in the CSI-RS Coordination Request message for activation.

The L1 measurement results received from the UE need to be included in the activation request message since the SP CSI-RS resources to be activated (or deactivated) cannot be determined by the source gNB-DU based on the L1 measurement results, since it does not possess information regarding the TCI states of SP CSI-RS resources.

Proposal 1b (Option 1, dynamic). Upon receiving a request for activation (or deactivation) of a set of SP CSI-RS resources of a candidate cell, the candidate DU includes, in the CSI-RS Coordination Response messages transmitted over F1 and Xn, respectively, the TCI state IDs corresponding to the activated (or deactivated) SP CSI-RS resources. The candidate CU forwards the message received from an underlying candidate DU over Xn to the source CU, which in turn forwards the message over F1 to the source DU.

For Option 2, the proposal below describes the procedure to configure the TCI states for SP CSI-RS resources of LTM candidate cells during the LTM preparation procedure.

Proposal 2a (Option 2, semi-static). During LTM preparation, upon receiving a request from the source CU (via a candidate CU) for CSI-RS resource configuration, a candidate DU provides the configuration of TCI state IDs for SP CSI-RS resources configured for a candidate cell in its response message (UE Context Setup Response or UE Context Modification Response).

- The configuration of TCI state IDs for SP CSI-RS resources is forwarded to the source CU by the candidate CU in the Handover Request Acknowledgement message.

- The source CU includes the provided configuration in the LTM configuration signalled to the UE in an RRC reconfiguration message.

The source gNB-DU initiates the activation (or deactivation) of SP CSI-RS resources of a candidate cell based on L1 measurements received from the UE.

Proposal 2b (Option 2, semi-static). The source DU indicates through a signalling message transmitted over the F1 and Xn interfaces to a candidate DU activation/deactivation of SP CSI-RS resources of a candidate cell. The candidate DU provides confirmation in its response to the source DU.

Further details regarding the specific IEs in the F1 and Xn messages that can include the TCI state configuration for configured SP CSI-RS resources of candidate cells is described in the observation below. We note that the solution only has RRC specification impact and there is no RAN3 signalling impact, since the TCI state configuration information can be provided using the RRC IEs defined by RAN2.

Observation 3 (Option 2, semi-static). The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell is included in the CSI-RS Resource Configuration IE in the UE Context Setup Response, UE Context Modification Response, and Handover Request Acknowledgement messages. The CSI-RS Resource Configuration IE contains the ltm-CSI-ResourceConfigToAddModList IE, as defined in the running RRC CR for Rel-19 LTM.

As the running RRC CR for Rel-19 LTM indicates (see below), the ltm-NZP-CSI-RS-ResourceSet IE refers to the RRC IE NZP-CSI-RS-Resource, and we propose an enhancement to this basic IE in Observation 4 below to include the configuration of TCI state IDs for SP CSI-RS resources of an LTM candidate cell.

- LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations.

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18         LTM-SSB-ResourceSet-r18,

...,

[[

ltm-NZP-CSI-RS-ResourceSet-r19      LTM-NZP-CSI-RS-ResourceSet-r19                     OPTIONAL, -- Need R

ltm-CSI-IM-ResourceSet-r19          LTM-CSI-IM-ResourceSet-r19                         OPTIONAL, -- Need R

resourceType-r19                    ENUMERATED {periodic, semiPersistent}              OPTIONAL  -- Cond CSI-RS

]]

}

LTM-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-NZP-CSI-RS-ResourceSet-r19 ::=     SEQUENCE {

ltm-CSI-RS-ResourceList-r19         SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF NZP-CSI-RS-ResourceId,

ltm-CandidateIdList-r19             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-CSI-IM-ResourceSet-r19 ::=     SEQUENCE {

ltm-CSI-IM-ResourceSetId-r19        CSI-IM-ResourceSetId,

ltm-CandidateId-r19                 LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}       OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset      OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                           OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                    OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                        OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                OPTIONAL    -- Cond LTM

qcl-InfoSemiPersistentCSI-RS        TCI-StateId                          OPTIONAL,   -- Cond LTM

]]

}

Observation 4 (Option 2, semi-static). The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell is provided by enhancing the RRC NZP-CSI-RS-Resource IE, e.g., in the following:

Observation 5 (Option 2, semi-static). The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell as described in Observations 3 and 4, involves RRC specification impact, but no impact to RAN3 signalling.

Proposal 3. RAN3 should discuss the two possible options, Option 1 (dynamic), Option 2 (semi-static), as described in detail in the proposals 1a, 1b, 2a, 2b above, for specifying remaining aspects of the signalling procedure for activation/deactivation of SP CSI-RS resources on LTM candidate cells. The draft TPs to TS 38.473 and TS 38.423 describing the changes to the relevant messages for Option 1 are in Annexes A1 and A2.

# 3 Remaining issue#2: For support of CSI-RS resources for LTM CSI acquisition on candidate cells

During LTM preparation, when the source CU requests configuration of CSI-RS resources for CSI acquisition for a candidate cell, a candidate DU provides the configuration of CSI-RS resources in response.

RAN1 has defined a UE capability where the UE indicates support for CSI acquisition using only CSI-RS resources and without using CSI-IM resources, for an LTM candidate cell. Consequently, when the UE capabilities are forwarded to a candidate DU during LTM preparation, the candidate DU can determine whether it should provide the configuration of CSI-IM resources in addition to the configuration of CSI-RS resources, for an LTM candidate cell.

Proposal 4. During LTM preparation, based on UE capabilities regarding support for CSI acquisition using CSI-RS resources and/or CSI-IM resources that are forwarded to a candidate DU, the candidate DU determines whether to configure CSI-IM resources, in addition to CSI-RS resources, for the UE, and includes accordingly the configuration in its response message, for an LTM candidate cell.

Observation 6. There is no need for the source CU to explicitly request configuration of CSI-IM resources to a candidate gNB or a candidate DU, during LTM preparation.

If a candidate DU decides to configure CSI-IM resources for an LTM candidate cell, it can separately provide the configuration of periodic and SP CSI-RS resources, similarly as for CSI-RS resources.

Proposal 5. During LTM preparation, if a candidate DU decides to configure CSI-IM resources for an LTM candidate cell, it provides separately the configuration of periodic and SP CSI-IM resources in response to a request for configuration of CSI resources for CSI acquisition, similarly as for CSI-RS resources. The draft TPs to TS 38.473 and TS 38.423 describing the changes to the relevant messages are in Annexes A3 and A4.

# 4 Remaining issue#3:  For the Early TA RACH configuration procedure

In the RAN3 meetings so far, Early TA RACH configuration for initial LTM was covered. Below we discuss some aspects of the procedure for configuration for subsequent LTM.

For subsequent LTM, the serving gNB-DU needs the Early TA RACH configuration for all the configured LTM candidate cells.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

It was agreed in a previous meeting RAN3 #127 [2] that, to sync up the Early TA RACH configurations between the candidate gNBs, the LTM Configuration Update procedure may be triggered at any later point in time depending on network implementation.

For obtaining the Early TA RACH configuration for subsequent LTM, in Handover Request Ack, each candidate gNB-CU provides (candidate gNB ID, List of RACH resource request IDs).

Proposal 6. For obtaining the Early TA RACH configuration for subsequent LTM, each candidate gNB-CU provides the pair (candidate gNB ID, List of RACH resource request IDs) to the source gNB-CU in Handover Request Ack.

The source gNB-CU includes the information regarding the pair of (candidate gNB ID, List of RACH resource request IDs) provided by a candidate gNB-CU, in the LTM Configuration Update procedure with the other candidate gNB-CUs, for obtaining the Early TA RACH configuration for subsequent LTM.

# 5 Remaining aspect#4:  Related to transfer of TA information in the Cell Switch Notification procedure

Include the TA values in Cell Switch Notification message.

The following agreement was reached in the last RAN3 #129 meeting [1] on the transfer of TA information in the Cell Switch Notification procedure.

We think that the source gNB-DU should also provide the remaining TA validity times to the candidate gNB-DU along with the valid TA values, as it enables the candidate gNB-DU to track the validity of the TA values. Without providing remaining validity timer value to candidate cells, it does not help candidate DU/cell to determine how long source DU/cell provided TA value is valid.

Observation 7. If the source gNB-DU provides the remaining TA validity time to the candidate gNB-DU along with the valid TA values in the Cell Switch Notification procedure, it enables the candidate gNB-DU to track the validity of the TA values when the UE is in the target cell.

Proposal 7. The source gNB-DU provides the remaining TA validity time along with the TA values, to a candidate gNB-DU using the Cell Switch Notification messages over F1-AP and Xn-AP.  The draft TPs to TS 38.473 and TS 38.423 describing the changes to the relevant messages are in Annexes A5 and A6.

# 6 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

Remaining Issue#1: For activation/deactivation of SP CSI-RS resources on LTM candidate cells

Observation 1. In Rel-19 LTM, RAN3/RAN2 has defined the following signalling procedure for activation/deactivation of SP CSI-RS resources of an LTM candidate cell:

- The source DU indicates in signalling messages over the F1 and Xn interfaces to a candidate DU activation/deactivation of SP CSI-RS resources of a candidate cell.

- The candidate DU confirms the activation/deactivation in its response message to the source DU.

- The source DU then transmits the activation/deactivation indication to the UE in the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE.

Observation 2. In the response message to the source DU, the candidate DU does not provide TCI state information for the activated (or deactivated) SP CSI-RS resources, so that the source DU cannot provide the TCI state information along with the SP CSI-RS resource indications in the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE (shown below) to the UE.

Proposal 1a (Option 1, dynamic). The source gNB-DU initiates the activation (or deactivation) of SP CSI-RS resources of a candidate cell and provides the L1 measurement results of the candidate cell received from the UE to the candidate gNB-DU, in the CSI-RS Coordination Request message for activation.

Proposal 1b (Option 1, dynamic). Upon receiving a request for activation (or deactivation) of a set of SP CSI-RS resources of a candidate cell, the candidate DU includes, in the CSI-RS Coordination Response messages transmitted over F1 and Xn, respectively, the TCI state IDs corresponding to the activated (or deactivated) SP CSI-RS resources. The candidate CU forwards the message received from an underlying candidate DU over Xn to the source CU, which in turn forwards the message over F1 to the source DU.

Proposal 2a (Option 2, semi-static). During LTM preparation, upon receiving a request from the source CU (via a candidate CU) for CSI-RS resource configuration, a candidate DU provides the configuration of TCI state IDs for SP CSI-RS resources configured for a candidate cell in its response message (UE Context Setup Response or UE Context Modification Response).

- The configuration of TCI state IDs for SP CSI-RS resources is forwarded to the source CU by the candidate CU in the Handover Request Acknowledgement message.

- The source CU includes the provided configuration in the LTM configuration signalled to the UE in an RRC reconfiguration message.

Proposal 2b (Option 2, semi-static). The source DU indicates through a signalling message transmitted over the F1 and Xn interfaces to a candidate DU activation/deactivation of SP CSI-RS resources of a candidate cell. The candidate DU provides confirmation in its response to the source DU.

Observation 3 (Option 2, semi-static). The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell is included in the CSI-RS Resource Configuration IE in the UE Context Setup Response, UE Context Modification Response, and Handover Request Acknowledgement messages. The CSI-RS Resource Configuration IE contains the ltm-CSI-ResourceConfigToAddModList IE, as defined in the running RRC CR for Rel-19 LTM.

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}       OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset      OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                           OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                    OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                        OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                OPTIONAL    -- Cond LTM

qcl-InfoSemiPersistentCSI-RS        TCI-StateId                          OPTIONAL,   -- Cond LTM

]]

}

Observation 4 (Option 2, semi-static). The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell is provided by enhancing the RRC NZP-CSI-RS-Resource IE, e.g., in the following:

Observation 5 (Option 2, semi-static). The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell as described in Observations 3 and 4, involves RRC specification impact, but no impact to RAN3 signalling.

Proposal 3. RAN3 should discuss the two possible options, Option 1 (dynamic), Option 2 (semi-static), as described in detail in the proposals 1a, 1b, 2a, 2b above, for specifying remaining aspects of the signalling procedure for activation/deactivation of SP CSI-RS resources on LTM candidate cells. The draft TPs to TS 38.473 and TS 38.423 describing the changes to the relevant messages for Option 1 are in Annexes A1 and A2.

Remaining issue#2: For support of CSI-RS resources for LTM CSI acquisition on candidate cells

Proposal 4. During LTM preparation, based on UE capabilities regarding support for CSI acquisition using CSI-RS resources and/or CSI-IM resources that are forwarded to a candidate DU, the candidate DU determines whether to configure CSI-IM resources, in addition to CSI-RS resources, for the UE, and includes accordingly the configuration in its response message, for an LTM candidate cell.

Observation 6. There is no need for the source CU to explicitly request configuration of CSI-IM resources to a candidate gNB or a candidate DU, during LTM preparation.

Proposal 5. During LTM preparation, if a candidate DU decides to configure CSI-IM resources for an LTM candidate cell, it provides separately the configuration of periodic and SP CSI-IM resources in response to a request for configuration of CSI resources for CSI acquisition, similarly as for CSI-RS resources. The draft TPs to TS 38.473 and TS 38.423 describing the changes to the relevant messages are in Annexes A3 and A4.

Remaining issue#3:  For the Early TA RACH configuration procedure

Proposal 6. For obtaining the Early TA RACH configuration for subsequent LTM, each candidate gNB-CU provides the pair (candidate gNB ID, List of RACH resource request IDs) to the source gNB-CU in Handover Request Ack.

Remaining aspect#4:  Related to transfer of TA information in the Cell Switch Notification procedure

Observation 7. If the source gNB-DU provides the remaining TA validity time to the candidate gNB-DU along with the valid TA values in the Cell Switch Notification procedure, it enables the candidate gNB-DU to track the validity of the TA values when the UE is in the target cell.

Proposal 7. The source gNB-DU provides the remaining TA validity time along with the TA values, to a candidate gNB-DU using the Cell Switch Notification messages over F1-AP and Xn-AP.  The draft TPs to TS 38.473 and TS 38.423 describing the changes to the relevant messages are in Annexes A5 and A6.

# A1 Annex (Option 1: TP to TS 38.473)

Note that text procedure details are not provided and once a specific solution is agreed, these draft TPs can be converted into CRs with additional details.

#### 9.2.2.18	DU-CU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-DU to request the gNB-CU e.g. to activate/deactivate the SP CSI-RS transmissions from specific cells.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Resource Coordination List |  | 1 |  |  | YES | ignore |
| >CSI-RS ResourceCoordination Request Item |  | 1 .. <maxnoofLTMCSI-RSResourceConfig> |  |  | - |  |
| >>LTM CSI Resource Configuration ID | M |  | INTEGER (0..111) |  | - |  |
| >>Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | - |  |
| Measurement Quantities | M |  |  |  | - |  |
| >Candidate Cell ID List | M | 1 .. < maxnoofCandidateCells> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB List |  | 1..<maxnoofSSBs> |  |  |  |  |
| >>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |
| >>CSI-RS List |  | 0..<maxnoofCSI-RSs> |  |  | YES | ignore |
| >>>CSI-RS Resource ID | M |  | INTEGER (0.. 192) |  | - |  |
| >>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCSI-RSResourceConfig | Maximum number of LTM CSI-Resource Configurations. Value is 112. |

#### 9.2.2.20	CU-DU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-CU e.g. to coordinate the gNB-DU to activate/deactivate the SP CSI-RS transmissions from specific cells.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Coordination Request List |  | 1 |  |  | YES | ignore |
| >CSI-RS Coordination Request Item |  | 1 .. <maxnoofLTMCSI-RSResourceConfig> |  |  | - |  |
| >>LTM CSI Resource Configuration ID | M |  | INTEGER (0..111) |  | - |  |
| >>Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | - |  |
| Measurement Quantities | M |  |  |  | - |  |
| >Candidate Cell ID List | M | 1 .. < maxnoofCandidateCells> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB List |  | 1..<maxnoofSSBs> |  |  |  |  |
| >>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |
| >>CSI-RS List |  | 0..<maxnoofCSI-RSs> |  |  | YES | ignore |
| >>>CSI-RS Resource ID | M |  | INTEGER (0.. 192) |  | - |  |
| >>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCSI-RSResourceConfig | Maximum number of LTM CSI-Resource Configurations. Value is 112. |

#### 9.2.2.21	CU-DU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-DU e.g. to coordinate the gNB-CU about the SP CSI-RS transmissions activation/deactivation result.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | reject |  |
| gNB-CU UE F1AP ID | M |  |  | 9.3.1.4 |  | YES | reject |  |
| gNB-DU UE F1AP ID | M |  |  | 9.3.1.5 |  | YES | reject |  |
| CSI-RS Coordination Result List |  |  | 0..1 |  |  | YES | ignore |  |
| >CSI-RS Coordination Result Item IEs |  |  | 1 .. < maxnoofLTMCSI-RSResourceConfig > |  |  | - |  |  |
| >>LTM CSI Resource Configuration ID | M |  |  | INTEGER (0..111) |  | - |  |  |
| >>Transmission Status | M |  |  | ENUMERATED(activated, deactivated, …) |  | - |  |  |
| TCI States List |  |  | 0..1 |  |  | YES | ignore |  |
| > TCI State Item IEs |  |  | 1..<maxnoofTCIStates> |  |  | - |  |  |
| >> TCI State ID | M |  |  | INTEGER (0…127) | This field contains TCI-StateId, as specified in TS 38.331 [8], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [8]. | - |  |  |
| Range bound |  | Explanation |  |  |  |  |  |
| maxnoofLTMCSI-RSResourceConfig |  | Maximum number of LTM CSI-Resource Configurations. Value is 112. |  |  |  |  |  |

#### 9.2.2.19	DU-CU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-CU e.g. to inform the gNB-DU about the SP CSI-RS transmissions activation/deactivation result.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Coordination Result List |  | 0..1 |  |  | YES | ignore |
| >CSI-RS Coordination Result Item IEs |  | 1 .. < maxnoofLTMCSI-RSResourceConfig > |  |  | - |  |
| >>LTM CSI Resource Configuration ID | M |  | INTEGER (0..111) |  | - |  |
| >>Transmission Status | M |  | ENUMERATED(activated, deactivated, …) |  | - |  |
| TCI States List |  | 0..1 |  |  | YES | ignore |
| > TCI State Item IEs |  | 1..<maxnoofTCIStates> |  |  | - |  |
| >> TCI State ID | M |  | INTEGER (0…127) | This field contains TCI-StateId, as specified in TS 38.331 [8], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCSI-RSResourceConfig | Maximum number of LTM CSI-Resource Configurations. Value is 112. |

# A2 Annex (Option 1: TP to TS 38.423)

#### 9.1.5.7	CSI-RS COORDINATION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to coordinate the activation and deactivation of CSI-RS transmission for a UE at NG-RAN node2.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| CSI-RS Coordination Request | M |  |  |  | YES | reject |
| >L1 Measurements List |  | 0..1 |  |  | – |  |
| >>L1 Measurements Item |  | 1 .. <maxnoofCSIResourceConfigurationsPlus1> |  |  | – |  |
| >>>Request For Semi Persistent CSI-RS Resources | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the LTM-CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |
| >CSI-RS Acquisition List |  | 0..1 |  |  | – |  |
| >>CSI-RS Acquisition Item |  | 1 .. < maxnoofCSIResourceConfigurationsPlus1> |  |  | – |  |
| >>>Request For Semi Persistent CSI-RS Resources | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |
| Measurement Quantities | M |  |  |  | - |  |
| >Candidate Cell ID List | M | 1 .. < maxnoofCandidateCells> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB List |  | 1..<maxnoofSSBs> |  |  |  |  |
| >>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |
| >>CSI-RS List |  | 0..<maxnoofCSI-RSs> |  |  | YES | ignore |
| >>>CSI-RS Resource ID | M |  | INTEGER (0.. 192) |  | - |  |
| >>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIResourceConfigurationsPlus1 | Maximum number of CSI Resource Configurations Plus 1. Value is 112. |

#### 9.1.5.8	CSI-RS COORDINATION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to coordinate the activation and deactivation of CSI-RS transmission for a UE at NG-RAN node2.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| CSI-RS Activation Deactivation Result | M |  |  |  | YES | reject |
| >L1 Measurements List |  | 0..1 |  |  | – |  |
| >>L1 Measurements Item |  | 1 .. <maxnoofCSIResourceConfigurationsPlus1> |  |  | – |  |
| >>>Semi Persistent CSI-RS Resource Response | M |  | ENUMERATED(activated, deactivated, …) |  | – |  |
| >>>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |
| >CSI-RS Acquisition List |  | 0..1 |  |  | – |  |
| >>CSI-RS Acquisition Item |  | 1 .. < maxnoofCSIResourceConfigurations Plus1> |  |  | – |  |
| >>>Semi Persistent CSI-RS Resource Response | M |  | ENUMERATED(activated, deactivated, …) |  | – |  |
| >>>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |
| TCI States List |  | 0..1 |  |  | YES | ignore |
| > TCI State Item IEs |  | 1..<maxnoofTCIStates> |  |  | - |  |
| >> TCI State ID | M |  | INTEGER (0…127) | This field contains TCI-StateId, as specified in TS 38.331 [8], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIResourceConfigurationsPlus1 | Maximum number of CSI Resource Configurations Plus 1. Value is 112. |

# A3 Annex (TP to TS 38.473)

#### 9.3.1.360		CSI-RS Resource Configuration

This IE contains the CSI-RS resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodic CSI-RS Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [8]. |
| Semi Persistent CSI-RS Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [8]. |
| CSI-RS Resource Configuration To Release List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [8]. |
| Periodic CSI-IM Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-CSI-IM-ResourceToAddModList as defined in TS 38.331 [8]. |
| Semi Persistent CSI-IM Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-CSI-IM-ResourceToAddModList as defined in TS 38.331 [8]. |

# A4 Annex (TP to TS 38.423)

#### 9.2.3.224	CSI-RS Resource Configuration

This IE contains the CSI resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodic NZP CSI-RS Resource Configuration | O |  | NZP CSI-RS Resource Configuration 9.2.3.225 |  |
| Semi-Persistent NZP CSI-RS Resource Configuration | O |  | NZP CSI-RS Resource Configuration 9.2.3.225 |  |
| Periodic NZP CSI-RS Resource Set Configuration | O |  | NZP CSI-RS Resource Set Configuration 9.2.3.226 |  |
| Semi-Persistent NZP CSI-RS Resource Set Configuration | O |  | NZP CSI-RS Resource Set Configuration 9.2.3.226 |  |
| Periodic CSI-IM Resource Configuration | O |  | CSI-IM Resource Configuration 9.2.3.xxx |  |
| Semi-Persistent CSI-IM Resource Configuration | O |  | CSI-IM Resource Configuration 9.2.3.xxx |  |
| Periodic CSI-IM Resource Set Configuration | O |  | CSI-IM Resource Set Configuration 9.2.3.yyy |  |
| Semi-Persistent CSI-IM Resource Set Configuration | O |  | CSI-IM Resource Set Configuration 9.2.3.yyy |  |

#### 9.2.3.225	NZP CSI-RS Resource Configuration

This IE contains the CSI-RS resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI-RS Resource to AddMod List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [10]. |
| CSI-RS Resource to Release List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToReleaseList as defined in TS 38.331 [10]. |

#### 9.2.3.226	NZP CSI-RS Resource Set Configuration

This IE contains the CSI-RS resource set configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI-RS Resource Set to AddMod List | O |  | OCTET STRING | Includes ltm-NZP-CSI-RS-ResourceSetToAddModList contained in the LTM-Config IE as defined in TS 38.331 [10]. |
| CSI-RS Resource Set to Release List | O |  | OCTET STRING | Includes ltm-NZP-CSI-RS-ResourceSetToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [10]. |

#### 9.2.3.xxx	CSI-IM Resource Configuration

This IE contains the CSI-IM resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI-IM Resource to AddMod List | O |  | OCTET STRING | Includes the ltm-CSI-IM-ResourceToAddModList as defined in TS 38.331 [10]. |
| CSI-IM Resource to Release List | O |  | OCTET STRING | Includes the ltm-CSI-IM-ResourceToReleaseList as defined in TS 38.331 [10]. |

#### 9.2.3.yyy	CSI-IM Resource Set Configuration

This IE contains the CSI-IM resource set configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI-IM Resource Set to AddMod List | O |  | OCTET STRING | Includes ltm-CSI-IM-ResourceSetToAddModList as defined in TS 38.331 [10]. |
| CSI-IM Resource Set to Release List | O |  | OCTET STRING | Includes ltm-CSI-IM-ResourceSetToReleaseList as defined in TS 38.331 [10]. |

# A5 Annex (TP to TS 38.473)

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >> TAT Remaining Time | O |  | INTEGER (0..10240) | Indicates the remaining time of the TA timer, in ms. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >> TAT Remaining Time | O |  | INTEGER (0..10240) | Indicates the remaining time of the TA timer, in ms. | YES | ignore |
| Last Visited LTM Cells | O |  | 9.3.1.358 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

# A6 Annex (TP to TS 38.423)

#### 9.1.5.5	CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

This message is also sent by the source S-NG-RAN node to the M-NG-RAN node or from the M-NG-RAN node to the target S-NG-RAN node to inform the target S-NG-RAN node about the initiation of the cell switch command to the UE.Direction: source NG-RAN node  target NG-RAN node.

Direction: M-NG-RAN node  target S-NG-RAN node or source S-NG-RAN node  M-NG-RAN node (Dual Connectivity).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Include the TCI-StateId IE as defined in TS 38.331 [10]. | – |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE as defined in TS 38.331 [10]. | – |  |
| LTM UE Association Information List |  | 0..1 |  |  | YES | ignore |
| >LTM UE Association Item |  | 1.. <maxnoofLTMCells> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Last Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Cell Switch TA Information List |  | 0..1 |  |  | YES | ignore |
| >Cell Switch TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | – |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [10]. | – |  |
| >> TAT Remaining Time | O |  | INTEGER (0..10240) | Indicates the remaining time of the TA timer, in ms. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

# References

[1] Chair notes, RAN3 #129 meeting.

[2] Chair notes, RAN3 #127 meeting.