3GPP TSG-RAN WG3 Meeting #129bis														R3-256835

Prague, CZ, 13th-17th October, 2025

Agenda item:		10.5.1

Source:				CMCC

Title:					Discussion on 6G AIML use cases

Document for:		Discussion

# Introduction

In the 6G Radio SID [1], the following objectives about AI are captured, and it can be observed that AI/ML is a crucial component of 6G:

(8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this paper, we provide our consideration on the AI/ML related use cases.

# Discussion

In 5G, RAN3 has studied various AI/ML related use cases to optimize network performance, including AI/ML based energy saving, load balancing, mobility optimization, slicing, coverage and capacity optimization, etc. The detailed solutions and procedures are captured in TR 37.817 and TR 38.743.

Taking existing solutions and procedures as baseline, AI/ML related use cases for 6G can be further studied and enhanced to adapt the evolving of 6G network architecture.

Observation 1: 5G has studied various AI/ML related use cases including AI/ML based energy saving, load balancing, mobility optimization, slicing, coverage and capacity optimization, etc., to optimize network performance.

Proposal 1: The study of 6G AI/ML use cases propose to take the existing 5G use cases solutions as starting point, and enhanced to adapt 6G network architecture.

Based on the use cases discussed in 5G, we believe that 6G Day 1 needs to select some typical use cases for high priority support. From the perspective of operators, network operation and ensuring user experience are two comprehensive consideration dimensions. For network operation and maintenance, with the increasing deployment of base stations, energy consumption issues increase significantly, which may become one of the core challenges in 6G. Utilizing AI function to achieve intelligent energy efficiency optimization is a typical use case that needs to be supported. For user experience guarantee aspect, maintaining stable service quality in high-speed mobility scenario is a key task for operators. To enable the UE experience smooth and reliable services in different mobility scenarios, it’s necessary to optimize mobility management by utilizing AI function. Therefore, for 6G AI/ML use cases, it’s proposed to prioritize AI/ML based mobility optimization and network energy saving in 6G Day 1.

Proposal 2: AI/ML based mobility optimization and network energy saving propose to be supported with high priority in 6G Day 1, since the network operation and user experience guarantee are two critical aspects for operators.

## AI/ML based mobility optimization

Mobility Optimization is one of the typical scenarios optimized by means of AI/ML. The solutions and procedures for AI/ML based mobility optimization has been discussed in both R18 and R19, and R20 will continuous discussing AI/ML based mobility use cases, such as multiple-hop UE trajectory prediction and LTM.

In 5G, the AI/ML based mobility optimization mainly focused on two aspects: UE trajectory prediction and NR-DC scenario. Considering that 6G has not been determined whether DC scenario will be supported, it’s proposed that for AI/ML based mobility optimization in 6G, SA scenario should be prioritized and other scenarios can be discussed based on the progress of 6G migration option(s) discussion.

From RP-252912: TSG#113 (September/2026): RAN plenary to make a decision on additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G). This includes decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

Observation 2: 6G has not determined whether additional migration option(s) will be supported other than standalone.

For UE trajectory prediction, R18 only supported single-hop UE trajectory prediction which created a dependency chain: if the previous hop lacks prediction capabilities, subsequent hop may lose the critical UE trajectory prediction information. The multi-hop UE trajectory prediction will be discussed in R20, which enables multiple hops obtaining trajectory prediction in advance no matter whether the previous hop has the prediction capability or not, thereby facilitating proactive resource reservation and significantly improving the robustness of mobility. Given the above advantages, the multi-hop UE trajectory prediction should be supported in the 6G Day 1.

Observation 3: R18 only supported single-hop UE trajectory prediction, where the availability of prediction information for subsequent hops strictly depends on the prediction capability of previous hop. Multi-hop UE trajectory prediction will be discussed in R20 to enable the subsequent hops obtain the prediction in advance and reserve resource accordingly.

Therefore, based on the above analysis, for 6G AI/ML based mobility optimization, multi-hop UE trajectory prediction and SA scenario should be supported with high priority.

Proposal 3: AI/ML based mobility optimization in 6G proposes to support SA scenario first and discuss multi-hop UE trajectory prediction with high priority.

Besides, the following mobility features are most likely to be supported in 6G Day 1:

- L3 handover: CMCC has deployed R15 L3 measurement based HO in the commercial network, and based on the real network data, the handover success probability is almost 99%, which can ensure UE’s experience very well. Therefore, it seems that the L3 handover can be used as baseline mobility mechanism in 6G.

- L1/L2-triggered mobility (LTM): LTM was first introduced in 5G to reduce the handover latency. It is applicable to high frequency scenarios with frequently handover, like FR2. Though it is not commercialized or tested since it is introduced in the late releases of 5G, considering in the future 6G, more high frequency spectrum may be used, LTM should be supported from 6G day 1 for high frequency scenarios.

- Conditional handover (CHO): CHO is defined as a handover that is executed by the UE when one or more handover execution conditions are met. It is applicable to high speed scenario and CMCC has performed field trial on CHO for high speed train scenario and achieves good handover performance. As high speed scenario also exists in 6G, it’s necessary to be supported in 6G Day 1.

If the above mobility features are supported in 6G Day 1, the AI/ML based mobility enhancement can be considered to improve the mobility performance. For example, for L3 handover, as the handover accuracy is relative high, the enhancement by AI/ML can be considered from other aspects, such as how to save measurement resources, etc. For LTM, as AI/ML based LTM will be first discussed in R20 AI/ML for NG-RAN SI/WI, the 5G-A discussion can be taken as starting point and further enhance the solution with 6G RAN architecture and 6G AI/ML framework, the potential enhancements may including target cell/beam prediction, TA prediction, etc. For CHO, the existing AI/ML enabled handover enhancement could be taken as starting point, and adjust the solution for conditional handover, such as candidate cell prediction, etc.

Proposal 4: For AI/ML based mobility optimization, L3 handover, LTM and CHO can be considered to be supported in 6G Day 1.

## AI/ML based energy saving

In R18, the solution for AI/ML based energy saving has been discussed with limited progress, i.e. the measured energy cost is only limited to node level and there is no consensus on prediction information. Then, during the study item phase of R19, energy saving use case also been discussed with no consensus reached and, as a result, dropped for the R19 WI phase. From our perspective, AI/ML based energy saving can further enhance energy efficiency and achieve a balance between performance and energy consumption, which is worth to be studied in 6G.

During the discussion in 5G, the predicted energy cost had been discussed for several meetings with no consensus achieved. Also, for the cell level energy cost, although it’s beneficial to have this information, it remains challenges to be obtained as many cells share the same hardware infrastructure which is difficult to calculate the energy cost per cell. Therefore, at the beginning of 6G, it’s proposed not to discuss the enhancements that have been discussed before but is difficult to reach some consensus.

Observation 4: AI/ML based energy saving has been discussed in 5G with limited progress. Some enhancements, such as energy cost prediction and finer granularity of energy cost, had already discussed for several meetings with no consensus achieved.

Proposal 5: At the beginning of 6G, it’s proposed not to discuss the enhancements which have been discussed before but with no consensus achieved.

From our perspective, AI/ML based energy saving is a valuable use case for operators. But when discussing 6G AI/ML based energy saving, the scope of the discussion should be clarified first. Two potential aspects may be evaluated:

- Whether the 6G network remains almost similar to that of 5G. If so, the finer granularity of energy cost and other enhancements with the same issue may still not be achievable.

- Whether the features supported in 5G will still be supported in 6G. For the 5G NES features, such as Cell DTX/DRX, Cell (de)Activation, OD-SSB, and OD-SIB1, if 6G will not support these features, the discussion of the related enhancements will be meaningless.

Therefore, it is proposed that a clear discussion scope should first be carefully deliberated‌, and the solutions and procedures should be carried out on this basis.

Proposal 6: For AI/ML based energy saving, the enhancement scope proposes to be discussed and clarified first. The solutions and procedures should be carried out on this basis.

## Others

Network Slicing:

Network slicing can provide customized network services for diversified services/UEs. As 6G has diversified services/applications, network slicing becomes essential to meet the distinct services requirements of different UEs or vertical industries.

In 6G, network slicing is explicitly supported in 6G RAN architecture in [2].

## 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

Observation 5: Network slicing is explicitly supported in 6G RAN architecture to meet the diversified services/ applications requirements.

AI/ML based network slicing has been discussed in R19 and several agreements achieved. However, considering that even though 6G already supports the network slicing feature, the working mechanism and technical details of network slicing still unclear. Therefore, it’s proposed that the supporting of AI/ML based network slicing can be postponed until the technical details of 6G network slicing are determined.

Proposal 7: AI/ML based network slicing can be supported until the working mechanism and technical details of 6G network slicing are determined.

QoE Optimization:

QoE (Quality of Experience) refers to the user experience at application level of the network services. It not only includes subjective scores (such as MOS), but can also be measured with objective metrics such as delay and buffer. Current studies mainly focus on the latter one. For objective metrics, legacy QoE metrics are configured by OAM, and the QoE report will be transferred to the QoE server, which may not parse by gNB. To enable RAN to perform optimizations based on QoE metrics, RVQoE was introduced including buffer level and playout delay for Media Start-up.

Figure 2.3-1 RAN-visible QoE information reporting

In 6G networks, the diverse device types and services lead to diversified QoE from different users and applications. To guarantee the user experience from different services/UEs, AI/ML function is needed to assist optimization. With the support of AI/ML function, the network can adjust the resources dynamically to fulfill the real-time requirements and the guarantee the QoE performance, especially for mobility handover.

Observation 6: AI/ML based QoE optimization is beneficial for guarantee user experience during handover with various services requirements.

In the previous meetings, AI/ML based QoE optimization has been proposed in R19 and R20, but was down-selected in favour of slice and CCO in R19 and mobility enhancement in R20, since this feature has not been commercialized in 5G.

In 6G, as it’s not determined whether QoE will be supported yet, AI/ML based QoE optimization proposes to be considered with secondary priority, and discusses only after 6G explicitly supports QoE features and clarifies the QoE working mechanisms.

Proposal 8: AI/ML based QoE optimization proposes to be discussed after 6G explicitly supports QoE features and determines the QoE working mechanisms.

# Conclusion

In this contribution, the following observations and proposals are provided on 6G AIML use cases:

Observation 1: 5G has studied various AI/ML related use cases including AI/ML based energy saving, load balancing, mobility optimization, slicing, coverage and capacity optimization, etc., to optimize network performance.

Observation 2: 6G has not determined whether additional migration option(s) will be supported other than standalone.

Observation 3: R18 only supported single-hop UE trajectory prediction, where the availability of prediction information for subsequent hops strictly depends on the prediction capability of previous hop. Multi-hop UE trajectory prediction will be discussed in R20 to enable the subsequent hops obtain the prediction in advance and reserve resource accordingly.

Observation 4: AI/ML based energy saving has been discussed in 5G with limited progress. Some enhancements, such as energy cost prediction and finer granularity of energy cost, had already discussed for several meetings with no consensus achieved.

Observation 5: Network slicing is explicitly supported in 6G RAN architecture to meet the diversified services/ applications requirements.

Observation 6: AI/ML based QoE optimization is beneficial for guarantee user experience during handover with various services requirements.

Proposal 1: The study of 6G AI/ML use cases propose to take the existing 5G use cases solutions as starting point, and enhanced to adapt 6G network architecture.

Proposal 2: AI/ML based mobility optimization and network energy saving propose to be supported with high priority in 6G Day 1, since the network operation and user experience guarantee are two critical aspects for operators.

Proposal 3: AI/ML based mobility optimization in 6G proposes to support SA scenario first and discuss multi-hop UE trajectory prediction with high priority.

Proposal 4: For AI/ML based mobility optimization, L3 handover, LTM and CHO can be considered to be supported in 6G Day 1.

Proposal 5: At the beginning of 6G, it’s proposed not to discuss the enhancements which have been discussed before but with no consensus achieved.

Proposal 6: For AI/ML based energy saving, the enhancement scope proposes to be discussed and clarified first. The solutions and procedures should be carried out on this basis.

Proposal 7: AI/ML based network slicing can be supported until the working mechanism and technical details of 6G network slicing are determined.

Proposal 8: AI/ML based QoE optimization proposes to be discussed after 6G explicitly supports QoE features and determines the QoE working mechanisms.

# References

- RP-251881, “New SID: Study on 6G Radio”, RAN#108, June 2025.

- 3GPP TR 38.914 V0.2.0, Study on 6G Scenarios and Requirements