3GPP TSG-RAN WG3 Meeting #129-bis	R3-256841

Prague, Czech, 13th – 17th Oct 2025

Title: 	Discussion on AI/ML based intra-CU LTM use cases

Source: 	CMCC

Agenda item: 	12.2.2

Document Type:	Discussion and Decision

# Introduction

A new R20 study item has been agreed in RAN#109:  RP-252867, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3.

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

This discussion paper will focus on the AI/ML based intra-CU LTM use cases.

# Discussion

This discussion paper will focus on the AI/ML based intra-CU LTM use cases. And since we shall base on the principles of AI/ML for NG-RAN captured in TS 38.300 and TS 38.401, we assume that:

For CU-DU split architecture, the following locations are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

There are three scenarios for intra-CU LTM as described in TS38.401 we considered as below.

## 2.1 Intra-DU LTM

The procedures of Intra-DU LTM is described in TS38.401 as below:

When gNB-DU makes LTE Cell Switch Decision, it needs the TA report. According to TS38.300, the TA can be

When configured by the network, it is possible to initiate UL TA acquisition (called early TA) procedure of one or multiple cells that are different from the current serving cells. If the cell has the same NTA as the current serving cells or NTA=0, early TA acquisition procedure is not required. The network may request the UE to perform early TA acquisition of a candidate cell before a cell switch. The early TA acquisition procedure is triggered by PDCCH order as specified in clause 9.2.6 or realized through UE-based TA measurement as configured by RRC. In the former case, the gNB/gNB-DU to which the candidate cell belongs calculates the TA value and sends it to the gNB/gNB-DU to which the serving cell belongs via gNB-CU. The serving cell sends the TA value in the LTM cell switch command MAC CE when triggering LTM cell switch. In the latter case, the UE performs TA measurement for the candidate cells after being configured by RRC but the exact time the UE performs TA measurement is up to UE implementation. The UE applies the TA value measured by itself and performs RACH-less LTM upon receiving the cell switch command, if it does not include any valid TA value. The network may also send a TA value in the LTM cell switch command MAC CE without early TA acquisition.

In current procedure, for the candidate cell, if NTA=0 or NTA is same as the serving cell, acquisition procedure is not required. Otherwise, the early TA acquisition procedure will be triggered by gNB-DU to UE. And all the cells need to calculate the TA respectively.

Another way to get TA is to predicate it on gNB-CU through AI/ML, based on the historical TA measurements gNB-CU collected, and some other information such as the UE trajectory, UE location information. And gNB-CU can transfer the predication TA measurement to gNB-DU for each candidate cells. Then gNB-DU can use this value to decide which cell to switch over to.

With the predicated TA value for each candidate cells, gNB-DU can decide the target cell quickly, prepare more for the future switchover, and avoid the messages transferred between UE. It can further improve the efficiency of LTM.

And since the TA value is a predicated value, it shall be with a validate time. gNB-DU can use it to select the cell, or use it as an assistant information to decide the cell switchover. The early TA acquisition procedure may still be triggered, up to the implementation of gNB-DU.

So the following Output shall be considered as AI/ML based intra-CU LTM enhancement：

- Predicated TA value for each candidate cells

Proposal 1: For intra-DU LTM, it is suggested to introduce predicated TA value for each candidate cells as the output of AI/ML model. The predicated TA value can be used by gNB-DU to decide a cell switch over, or as assistance information to decide a cell switch over.

And for intra-DU switch over, if early TA acquisition is triggered, the gNB/gNB-DU to which the candidate cell belongs calculates the TA value and make LTE Cell Switch Decision. However, if gNB-CU can report get TA value from gNB-DU, it can help AI/ML to predicate the future TA, and verify the TA predication.

So to predicate the TA value for each candidate cells, gNB-CU can use the following information from gNB-DU as input for AI/ML inference model:

- Historical TA value measured by UE for candidate cells

gNB-CU can also use the local input to predicate TA value as below:

- UE trajectory predication

- UE location predication, this location includes the geo, coordination information, not the same granularity as UE trajectory to cells, gNB can use the location to predicate TA value from UE to each candidate cells

Proposal 2: In intra-DU LTM, it is suggested to use the local information such as UE trajectory predication and UE location predication as input to predicate the TA value in AI/ML model.

Proposal 3: In intra-DU LTE, it is also suggested to use the historical TA value measured by UE for all the candidate cells as input to predicate the TA value in AI/ML model.

And after cell switch over, it is better to provide the feedback information back to AI/ML model for verification and evaluation:

- UE performance

- Cell switch over time

- TA value measured by UE for target cell and other candidate cells

Proposal 4: In intra-DU LTM, it is suggested to introduce the following feedback information for AI/ML model:

- UE performance

- Cell switch over time

- TA value measured by UE for target cell and candidate cells

## 2.2 Inter-DU LTM

The inter-DU LTM procedures are very similar with intra-DU LTM. So before early TA acquisition is triggered, gNB-CU can predicate TA through AI/ML, based on the historical TA measurements gNB-CU collected, and some other information such as the UE trajectory. Then gNB-CU can transfer the predication TA measurement to gNB-DU for each candidate cell. And gNB-DU can use this value to decide which cell to switch over to.

With the predicated TA value for each candidate cells, gNB-DU can decide the target cell quickly, prepare more for the future switchover, and avoid the messages transferred between UE. It can further improve the efficiency of LTM.

And since the TA value is a predicated value, it shall be with a validate time. gNB-DU can use it to select the cell, or use it as an assistant information to decide the cell swithover. The early TA acquisition procedure may still be triggered, up to the implementation of gNB-DU.

And for intre-DU swith over, if early TA acquisition is triggered, the gNB/gNB-DU to which the candidate cell belongs calculates the TA value and make LTE Cell Switch Decision. And the source gNB-DU can report the TA value to target gNB-DU through gNB-CU, so in current procedures, gNB-CU can know the TA value to do future predication, or verify its predication. No enhancement here.

It is also beneficial to collect UE performance feedback (including DL/UL packet delay and DL/UP packet loss) after cell switch over succeed. And the UE CONTEXT MODIFICATION REQUEST procedures can be reused to initialize the UE performance feedback from DU to CU-UP.

Proposal 5: For inter-DU LTM, it is suggested to use the same input, output and feedback for AI/ML model as intra-DU LTM.

Observation 1: Some difference may exist between inter and intra DU LTM procedures that the TA value for target cell and Cell switch over time have already been sent to gNB-CU in current inter-DU procedure.

## 2.3 Intra-CU LTM with gNB-CU-UP change

In TS38.401, also LTM with gNB-CU-UP change procedure is described. However, we don’t think it is a common scenario and introduce extra complexity through E1. So it is suggested to exclude this scenario in this study.

Proposal 6: It is suggested that gNB-CU-UP change scenario is out of scope of this SI.

# 3	Summary

In this contribution, we provide additional input considerations on AI/ML intra-CU LTM use cases, following proposals are made:

Observation 1: Some difference may exist between inter and intra DU LTM procedures that the TA value for target cell and Cell switch over time have already been sent to gNB-CU in current inter-DU procedure.

Proposal 1: For intra-DU LTM, it is suggested to introduce predicated TA value for each candidate cells as the output of AI/ML model. The predicated TA value can be used by gNB-DU to decide a cell switch over, or as assistance information to decide a cell switch over.

Proposal 2: In intra-DU LTM, it is suggested to use the local information such as UE trajectory predication and UE location predication as input to predicate the TA value in AI/ML model.

Proposal 3: In intra-DU LTE, it is also suggested to use the historical TA value measured by UE for all the candidate cells as input to predicate the TA value in AI/ML model.

Proposal 4: In intra-DU LTM, it is suggested to introduce the following feedback information for AI/ML model:

- UE performance

- Cell switch over time

- TA value measured by UE for target cell and candidate cells

Proposal 5: For inter-DU LTM, it is suggested to use the same input, output and feedback for AI/ML model as intra-DU LTM.

Proposal 6: It is suggested that gNB-CU-UP change scenario is out of scope of this SI.

# 4	Reference

[1] RP-252867, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3

[2] TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2

[3] TS 38.401, NG-RAN Architecture description, Stage 2