3GPP TSG-RAN WG3 #129bis                                  	R3-256857

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	9.2.5

Source:	Samsung

Title:	Essential Corrections for Inter-CU (SCG) LTM

Document for:	Discussion & Decision

# 1 Introduction

RAN3 have completed the R19 NR mobility work, and all submitted CRs were approved at RAN#109 for Release 19 specification.

In this paper, we will discuss some issues regarding the procedure on inter-gNB-CU LTM based on the all agreed CRs in [1]~[6], and propose the essential corrections for the specification of TS 38.423/473 V19.0.0.

# 2 Discussion

## Issue 1: The acquisition of Early RACH Resources Requester ID info for Early TA RACH resource request based on gNB-DU

In RAN3#127 meeting, the following agreement was reached on this issue:

- The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration. - The format of the new introduced IE is same with the gNB-DU ID.

From the final CR in [4] as shown below, we note that early RACH configuration of each candidate cell is requested/allocated based on gNB-DU and corresponding gNB-DU ID is replaced with a newly defined ID (Early RACH Resources Requester ID) in XnAP Handover Request procedure in order to avoid exposing the internal architecture of gNB. This ID is used to allocate the dedicated early RACH resource differently to the gNB-DU, as RACH configuration could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different gNB-DU.

Below IEs are from BLCR in [4] and captured in TS 38.423 V19.0.0, in the HO REQ message for early RACH resources request. #### 9.2.3.217 Early Sync Information Request This IE contains a request for the early synchronization information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Early Sync Configuration Request; M; ENUMERATED (true, …). Early RACH Resources Providers List; 0..1. >Early RACH Resources Item; 1..< maxnoofEarly RACHResourcesID>. >>Global gNB ID; M; 9.2.2.1. >>Early RACH Resources Requester ID; M; 9.2.3.xx4; Identifies the entity requesting Early RACH resources.. Range bound; Explanation. maxnoofEarlyRACHResourcesID; Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8.. #### 9.2.3.220 Early RACH Resources Requester ID This IE indicates the identity of the Early RACH resources requester. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Early RACH Resources Requester ID; M; INTEGER (0 .. 236-1); Identifies requester of the Early RACH resources..

After reviewing the whole inter-CU LTM procedure, in case of supporting subsequent inter-CU LTM, we can note that in the Handover Request message, source gNB-CU sends the Early RACH Resources Providers List which includes multiple pairs of (gNB ID, Early RACH Resources Requester ID) to the candidate gNB-CU to request early RACH configuration for other candidate gNB-DUs besides source gNB-DU.

However, the issue is that source gNB does not know corresponding Early RACH Resources Requester ID (i.e., gNB-DU ID) information which candidate gNB-CUs have before Handover Request is initiated.

In current specification as below highlight from TS 38.473, when receiving the L3 Measurement Report from UE, source gNB can know the gNB information to which the cell belongs from Cell ID information because gNB ID is equal to the leftmost bits of the NR Cell Identity. However, source gNB cannot achieve the corresponding Early RACH Resources Requester ID (i.e., gNB-DU ID) information for each candidate cell as there is no connection between gNB-DU ID and cell identifiers.

Below IEs are from TS 38.473 V19.0.0 #### 9.3.1.305 Global gNB ID This IE is used to globally identify a gNB (see TS 38.300 [6]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.3.1.14. CHOICE gNB ID; M. >gNB ID. >>gNB ID; M; BIT STRING (SIZE(22..32)); Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB.. 9.3.1.9 gNB-DU ID The gNB-DU ID uniquely identifies the gNB-DU at least within a gNB-CU. IE/Group Name; Presence; Range; IE type and reference; Semantics description. gNB-DU ID; M; INTEGER (0 .. 236-1); The gNB-DU ID is independently configured from cell identifiers, i.e. no connection between gNB-DU ID and cell identifiers..

Therefore, RAN3 should discuss this issue on how source gNB-CU acquires the gNB-DU ID (Early RACH Resources Requester ID) information under the candidate gNB-CU before handover request is initiated.

Proposal 1-1: RAN3 should discuss how source gNB-CU acquires the gNB-DU ID (Early RACH Resources Requester ID) information under the candidate gNB-CU before handover request is initiated.

Regarding this issue, we propose below alternative solutions for discussion.

Option#1. Through Xn setup or NG-RAN node configuration update procedure between two neighbour NG-RAN nodes (also including source node and each candidate node), the Early RACH Resources Requester ID (corresponding gNB-DU ID) info can be exchanged in case of split gNB.

Option#2. If source decides to configure a subsequent inter-CU LTM, it triggers a new non-UE associated procedure to retrieve the gNB-DU ID info for candidate cell before HO REQ is initiated; The response message includes Early RACH Resources Requester ID (corresponding gNB-DU-ID) info of candidate cells in case of split gNB.

Option#3. Introduce an indicator for the Early RACH Resources Requester ID retrieval in the HO REQ message.

The HO REQ ACK message includes Early RACH Resources Requester ID (corresponding gNB-DU-ID) info of candidate cell in case of split gNB. In the second round, LTM Configuration Update procedure can be used to request the early RACH resource for each pair of (gNB ID, Early RACH Resources Requester ID).

Regarding Option#2, since a new non-UE associated procedure is introduced, it has more impact on the specification than Option#1 and Option#3. Therefore, Option#2 may not be the best choice. For Option#3, compared to Option#1, since early RACH resource is requested and allocated in the second round via XnAP LTM Configuration Update and F1AP UE Context Modification procedure, more delay is incurred even when subsequent LTM is not considered.

To minimize delay, early RACH resources should be requested and allocated within the first round by HO REQ and F1AP UE Context Setup procedure.

Therefore, we prefer Option#1 as a solution as it has the least impact on the specification by reusing the existing XnAP procedure.

Based on the above observations, we suggest RAN3 to agree Option#1 as a solution to fix this issue.

Proposal 1-2: RAN3 agrees to introduce the Early RACH Resources Requester ID IE in Served Cell Information NR IE for serving cells in case of split gNB in XnAP Xn Setup/NG-RAN node Configuration Update procedure, so as to let the source gNB-CU acquires the Early RACH Resources Requester ID information belongs to the candidate gNB-CU.

Moreover, considering the same issue in the inter-CU MCG LTM scenario, above solution is also applicable.

Regarding the scenario of inter-CU SCG LTM, early RACH configuration of each candidate PSCell is requested/allocated based on gNB-DU of SN and corresponding gNB-DU ID of SN is replaced with a new ID (Early RACH Resources Requester ID) in XnAP SN Addition Request procedure for the same reason, i.e., preventing exposure of internal architecture of the SN.

Therefore, same issue also exists here and Option#1 can be the solution. Before MN triggers the SN addition request to each candidate SN to request the Early TA RACH resource allocation for each gNB-DU of SN, MN can know the Early RACH Resources Requester ID information under the inter-SN candidate PSCell via Xn Setup/NG-RAN node Configuration Update procedure between MN and each candidate SN.

Observation 1: Option#1 is also applicable for the scenario of inter-CU SCG LTM and inter-CU MCG LTM.

## Issue 2: TA Info Transfer inter-CU SCG LTM

Consider the scenario regarding the inter-CU SCG LTM as below in which LTM is requested to be configured for the same candidate PSCell for different UE that is currently connected to the different MN and/or S-SN.

- UE1 in S-SN1 (DU1) and MN1

- UE2 in S-SN2 (DU2) and MN1

- UE3 in S-SN3 (DU3) and MN2

Respective LTM configuration for the same candidate PSCell in C-SN can be initiated for UE1, UE2 and UE3.

Early RACH resource of candidate PSCell is also requested/allocated for DU1 in S-SN1, DU2 in S-SN2 and DU3 in S-SN3

Since early RACH resource is requested and allocated for each DU of SN based on at least one pair of <SN ID, gNB-DU ID> as indicated by <LTM gNB ID, LTM gNB-DU ID> in Early Sync Information Request IE in SN Addition Request message in step2 and UE Context Setup Request message in step3. When candidate SN-DU receives the RACH resource sent by UE for early TA acquisition, it sends TA information via the DU-CU TA Information Transfer message in step10 to the candidate SN-CU only indicating one pair of <source SN ID, source gNB-DU ID> as indicated by <Source gNB ID, Source gNB-DU ID>. Since the TA information transfer procedure is non-UE associated signalling, by receiving only information of a pair of <Source gNB ID, Source gNB-DU ID>, the candidate SN-CU does not know to which MN this TA information should be forwarded as different MN for different UE (e.g., MN1 for UE1 and UE2, MN2 for UE3) can trigger the SN addition request to this same candidate PSCell in candidate SN.

Moreover, in step11, even the Candidate SN-CU sends XnAP TA Information Transfer to the correct MN somehow, by indicating only Early RACH Resources Requester ID in current specification, MN does not know to which SN it should forward this TA information, since different SN for different UE (UE1 in SN1, and UE2 in SN2) with one same MN can trigger the SN addition request procedure to the same candidate PSCell in candidate SN.

Based on the observation above, as current TA information transfer procedure needs to be enhanced, RAN3 should discuss this issue on how to ensure that TA Information is correctly transferred from the candidate SN-DU to the source SN-DU for inter-CU SCG LTM scenario.

Proposal 2-1: RAN3 to discuss the issue on how to ensure that TA Information is correctly transferred from the candidate SN-DU to source SN-DU in the scenario of inter-CU SCG LTM.

Let’s first recall how TA related information is transferred from the candidate DU to the source DU in non-DC scenario. The TA value of the cell with the candidate DU is included in the DU-CU TA INFORMATION TRANSFER message, and then transmitted to the source gNB-CU via TA INFORMATION TRANSFER message. Afterwards, the value will be finally passed to the source DU included in the CU-DU TA INFORMATION TRANSFER message.

In F1AP messages, gNB-DU ID is contained to distinguish candidate DUs for Early RACH resources. With the same philosophy, the pair <gNB ID, Early RACH Resources Requester ID> is included in the XnAP message.

Below messages are from BLCR in [5] and released in TS 38.473 V19.0.0 ### 9.2.1.24 DU-CU TA INFORMATION TRANSFER This message is sent by the gNB-DU to inform the gNB-CU about TA information. Direction: gNB-DU gNB-CU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. Transaction ID; M; 9.3.1.23; YES; reject. DU to CU TA Information List; 0..1; YES; ignore. >DU to CU TA Information Item IEs; 1 .. <maxnoof TAList>; EACH; ignore. >>Candidate Cell ID; M; NR CGI 9.3.1.12; -. >>TA Value; M; INTEGER (0..4095); Indicates the TA value as defined in TS 38.213 [31].; -. >>Preamble Index; M; INTEGER (0..63); -. >>RA-RNTI; M; INTEGER (0..65535, ...); RA-RNTI as defined in TS 38.321 [16].; -. >>Source gNB-DU ID; M; gNB-DU ID 9.3.1.9; -. >>Tag ID Pointer; O; OCTET STRING; Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8].; -. >>Source gNB ID; O; Global gNB ID 9.3.1.305; YES; ignore. Below message/IE is from BLCR in [4] and released in TS 38.423 V19.0.0 #### 9.1.5.4 TA INFORMATION TRANSFER This message is sent by the NG-RAN node1 to inform the NG-RAN node2 about the Timing Advance value and related information. Direction: NG-RAN node1  NG-RAN node2. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. TA Information List; 1; YES; reject. >TA Information Transfer Item; <1… maxnoof TAList>; –. >>Early RACH Resources Requester ID; M; 9.2.3.220; Identifies the entity requesting Early RACH resources.; –. >>Candidate Cell ID; M; NR CGI 9.2.2.7; –. >>TA Value; M; INTEGER (0..4095); Indicates the TA value as defined in TS 38.213 [40].; –. >>Preamble Index; M; INTEGER (0..63); –. >>RA-RNTI; M; INTEGER (0..65535, ...); RA-RNTI as defined in TS 38.321 [35].; –. >>Tag ID Pointer; O; OCTET STRING; Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [10].; –. Range bound; Explanation. maxnoofTAList; Maximum no. of TA values to be sent, the maximum value is 8.. #### 9.2.3.220 Early RACH Resources Requester ID This IE indicates the identity of the Early RACH resources requester. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Early RACH Resources Requester ID; M; INTEGER (0 .. 236-1); Identifies requester of the Early RACH resources..

We think the same mechanism could be adopted to the inter-CU SCG LTM, by introducing MN ID on top of the Source gNB-DU ID and Source gNB ID IE in the DU-CU TA Information Transfer message (in step10) which can be transmitted from the candidate SN-DU to the candidate SN-CU. When receiving the IE, SN-CU will forward the TA relevant information to the indicated MN. Furthermore, when the Source gNB ID is introduced in the XnAP message, to indicate SN to which TA relevant information should be transmitted, MN can use it transferring corresponding information to the indicated SN.

The above figure depicts the procedure including new IE. In step3, when Early RACH resource is requested, MN ID is included in addition to the existing node IDs, i.e., <SN ID, gNB-DU ID>, and candidate SN-DU stores the RACH resource for early sync based on <MN ID, SN ID, gNB-DU ID>. Then, when the UE accesses to the candidate SN-DU for early TA acquisition, Candidate SN-DU can determine corresponding MN to which TA information should be transmitted and indicate MN ID besides <Source gNB ID, Source gNB-DU ID> in the DU-CU TA Information Transfer message.

Below message is from BLCR in [5] and released in TS 38.473 V19.0.0 ### 9.2.2.1 UE CONTEXT SETUP REQUEST This message is sent by the gNB-CU to request the setup of a UE context. Direction: gNB-CU gNB-DU. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. gNB-CU UE F1AP ID; M; 9.3.1.4; YES; reject. gNB-DU UE F1AP ID; O; 9.3.1.5; YES; ignore. SpCell ID; M; NR CGI 9.3.1.12; Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell.; YES; reject. ServCellIndex; M; INTEGER (0..31,...); YES; reject. <skip unchanged part>. Early Sync Information Request; 0..1; YES; ignore. >Request for RACH Configuration; O; ENUMERATED (true, …); -. >LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).; YES; reject. >>LTM gNB-DUs Item IEs; 1..< maxnoofLTM gNBDUs>. >>>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>>LTM gNB ID; O; Global gNB ID 9.3.1.305; YES; reject. Path Addition Information; O; 9.3.1.296; This IE contains either the Indirect Path Addition IE or the N3C Indirect Path Addition IE.; YES; reject. #### 9.3.1.328 Early UL Sync Configuration This IE indicates the early UL sync configurations for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RACH Configuration; M; OCTET STRING; Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8].. LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).. >LTM gNB-DUs Item IEs; 1..< maxnoofLTMg NBDUs>. >>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>Preamble Index List; O; 9.3.1.329. Range bound; Explanation. maxnoofLTMgNBDUs; Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8..

Based on above discussion, new IEs can be introduced as below for the issue on how to ensure that TA Information is correctly transferred from the candidate SN-DU to the source SN-DU.

Proposal 2-2: RAN3 agrees to introduce new IEs as below for the issue on how to ensure that TA Information is correctly transferred from the candidate SN-DU to the source SN-DU in inter-CU SCG LTM.

- Introduce MN ID in Early Sync Information Request IE in F1AP UE Context Setup/Modification Request message

- Introduce MN ID in the F1AP DU-CU TA Information Transfer message from candidate SN-DU to candidate SN-CU to inform candidate SN-CU to which MN it should forward this TA information.

- Introduce Source gNB ID (indicating source SN) in the XnAP TA Information Transfer message from candidate SN-CU to MN to inform MN to which SN (source SN-CU) TA information should be forwarded.

In addition, in the current specification of TS 38.473 V19.0.0, regarding 9.3.1.328 Early UL Sync Configuration IE, LTM gNB ID IE is missing. Since RACH resource request is based on each pair of <LTM gNB ID, LTM gNB-DU ID> in the Early Sync Information Request IE in F1AP UE Context Setup Request message, the Early UL Sync Configuration should also include the pair of <LTM gNB ID, LTM gNB-DU ID>.

Below IE is from TS 38.473 V19.0.0, LTM gNB ID IE is newly added on top of current specification #### 9.3.1.328 Early UL Sync Configuration This IE indicates the early UL sync configurations for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RACH Configuration; M; OCTET STRING; Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8].. LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).. >LTM gNB-DUs Item IEs; 1..< maxnoofLTMg NBDUs>. >>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>Preamble Index List; O; 9.3.1.329. >>LTM gNB ID; O; Global gNB ID 9.3.1.305. Range bound; Explanation. maxnoofLTMgNBDUs; Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8..

Proposal 2-3: Add LTM gNB ID IE in the Early UL Sync Configuration IE.

## Issue 3: UE Based TA Measurement ID assignment mechanism

As we know, the co-existence of inter-CU LTM and intra-CU LTM is possible. Source gNB can configure intra-CU (including inter/intra-DU) LTM candidate cell, furthermore, source gNB can configure more than one LTM candidate cells within one candidate gNB via respective handover request procedure per-cell.

Suppose cell1 and cell2 is in the same candidate gNB1, regarding the configuration of UE Based TA Measurement ID for each LTM cell, it is reasonable for candidate gNB1 to decide whether corresponding UE Based TA Measurement ID for cell1 and cell2 is different or same. Source gNB can only decide whether UE Based TA Measurement ID is same or different between source cell0 and other candidate cells (including cell1 and cell2), but it cannot decide whether UE Based TA Measurement ID is same or different between cell1 and cell2 with one candidate gNB1. Otherwise, it violates the principles of R18 intra-CU LTM, the gNB-CU configures its own cells on the UE Based TA Measurement ID which is same or different between any two cells.

Now we revisit the agreement from RAN3#128.

UE Based TA Measurement − Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message.

As we know, last RAN3#129 meeting, we continued to discuss Rel-19 Set IDs assignment mechanism and finally agreement is achieved by down selction.

RAN3#128 meeting - FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2: - Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message. - Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different. - FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB. RAN3#129 meeting - For Rel-19 Set ID assignment, source gNB-CU sends the Rel-19 Set ID list per-node to candidate gNB-CUs in Handover Request message. - Agree to reuse LTM Configuration Update message to transfer Rel-19 set ID per candidate cell to the candidate CUs.

From our understanding, for the UE Based TA Measurement ID assignment for LTM candidate cells in inter-CU (SCG) LTM, we can reuse the same solution on Rel-19 set IDs assignment among CUs (including the inter-CU SCG LTM), so we suggest RAN3 to agree below proposal:

Proposal 3: For UE Based TA Measurement ID assignment for LTM candidate cells, REUSE the same solution on Rel-19 set IDs assignment among CUs.

Based on above discussion, corresponding CR to TS38.423 has been provided in Annex and CR to TS38.473 has been provided in [7].

Proposal 4: RAN3 to agree the provided CRs for corresponding corrections on aforementioned essential issue.

# 3 Conclusion

This contribution discusses the essential issues on inter-CU (SCG) LTM procedure first, and invite RAN3 to agree below proposals.

## Issue 1: The acquisition of Early RACH Resources Requester ID info for Early TA RACH resource request based on gNB-DU

Proposal 1-1: RAN3 should discuss how source gNB-CU acquires the gNB-DU ID (Early RACH Resources Requester ID) information under the candidate gNB-CU before handover request is initiated.

Proposal 1-2: RAN3 agrees to introduce the Early RACH Resources Requester ID IE in Served Cell Information NR IE for serving cells in case of split gNB in XnAP Xn Setup/NG-RAN node Configuration Update procedure, so as to let the source gNB-CU acquires the Early RACH Resources Requester ID information belongs to the candidate gNB-CU.

Observation 1: Option#1 is also applicable for the scenario of inter-CU SCG LTM and inter-CU MCG LTM.

## Issue 2: TA Info Transfer inter-CU SCG LTM

Proposal 2-1: RAN3 to discuss the issue on how to ensure that TA Information is correctly transferred from the candidate SN-DU to source SN-DU in the scenario of inter-CU SCG LTM.

Proposal 2-2: RAN3 agrees to introduce new IEs as below for the issue on how to ensure that TA Information is correctly transferred from the candidate SN-DU to the source SN-DU in inter-CU SCG LTM.

- Introduce MN ID in Early Sync Information Request IE in F1AP UE Context Setup/Modification Request message

- Introduce MN ID in the F1AP DU-CU TA Information Transfer message from candidate SN-DU to candidate SN-CU to inform candidate SN-CU to which MN it should forward this TA information.

- Introduce Source gNB ID (indicating source SN) in the XnAP TA Information Transfer message from candidate SN-CU to MN to inform MN to which SN (source SN-CU) TA information should be forwarded.

Proposal 2-3: Add LTM gNB ID IE in the Early UL Sync Configuration IE.

## Issue 3: UE Based TA Measurement ID assignment mechanism

Proposal 3: For UE Based TA Measurement ID assignment for source cell and each candidate cell(s), REUSE the same solution on Rel-19 set IDs assignment among CUs.

Proposal 4: RAN3 to agree the provided CRs for corresponding corrections on aforementioned essential issue.

# 4 Reference

[1] R3-256003 (BLCR for 37.340) stage 2 DC

[2] R3-256004 (BL CR to 38.300) Support for Inter-CU LTM

[3] R3-256005 (BL CR to TS38.401) Support for Inter-CU LTM

[4] R3-256006 (BL CR to TS 38.423) XnAP support for inter-CU LTM

[5] R3-256007 (BL CR to TS38.473) F1AP support for inter-CU LTM

[6] R3-256023 (BL CR to 38.423) Xn LTM DC

[7] R3-256858 (CR to 38.473) Essential correction for inter-CU SCG LTM

# 5 Annex: CR to TS 38.423

3GPP TSG-RAN WG3 #129-bis										R3-25xxxx

# Prague, Czech Republic, 13 – 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | xxxx | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (CR to 38.423) Essential correction for Inter-CU (SCG) LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on discussion paper in R3-256857, the below issues are acknowledged by RAN3 for Inter-CU (SCG) LTM scenario. - Acquisition of Early RACH Resources Requester ID info by source gNB for Early TA RACH resource request - TA Info Transfer only for inter-CU SCG LTM - UE Based TA Measurement ID assignment mechanism |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the Early RACH Resources Requester ID IE in Served Cell Information NR IE for serving cells in case of split gNB in XnAP Xn Setup/NG-RAN node Configuration Update procedure. Introduce the Source gNB ID IE in the XnAP TA Information Transfer message from candidate SN-CU to MN to inform MN to forward the TA information to the corresponding source SN-CU indicated by the IE. Introduce the Proposed LTM UE Based TA Measurement ID List IE and UE Based TA Measurement Configuration IE in the relevant handover request procuedure and SN addition/modification request procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The defact is existing and function of Early RACH Resources Request,TA Information transfer and UE based TA measurement configuration can not work for inter-CU (SCG) LTM scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2; 8.3.1.2; 8.4.1.2; 8.4.2.2; 8.6.2.2; 9.1.1.1; 9.1.1.2; 9.1.2.1; 9.1.2.5; 9.1.5.1; 9.1.5.4; 9.2.2.11; 9.2.3.243; 9.2.3.xxx, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

If the procedure concerns an LTM, parallel transactions are allowed only when using the same source UE AP ID. Possible parallel requests are identified by the target cell ID.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Target NG-RAN node UE XnAP ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node shall remove the existing prepared conditional HO identified by the Target NG-RAN node UE XnAP ID IE and the Target Cell Global ID IE. It is up to the implementation of the target NG-RAN node when to remove the HO information.

<skip unchanged part>

If the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the LTM information for the UE in the target NG-RAN node and include the LTM Handover Information Request Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Reference Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for generating the LTM configuration. If the Request for CSI-RS Resource Configuration for L1 Measurements IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for L1 Measurements IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for Early CSI Acquisition IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM Candidate Configuration IE for the requested candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Proposed LTM No Security Change ID List IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM No Security Change ID IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the Target NG-RAN node UE XnAP ID IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, consider that the source NG-RAN node requests LTM preparation for an already established UE association.

If the Early Sync Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the early synchronization information for the UE in the target NG-RAN node.

If the Early Sync Information Response IE is contained in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, store it and use it as defined in TS 38.300 [9].

If the LTM CFRA Resource Information IE is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [35].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the target NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, and the Geographical Area IE contains the MDT PLMN List IE, the target NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

If the Proposed LTM UE Based TA Measurement ID List IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM UE Based TA Measurement Configuration IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

<skip unchanged part>

If the LTM Candidate PSCell Addition Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use the information to prepare for LTM candidate PSCell(s) configuration, and include the LTM Candidate PSCell Addition Information Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration.

If the SCG Reference Configuration Request IE set to "request" is contained in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, provide the SCG reference configuration for LTM.

If the LTM Configuration ID Mapping List IE is contained in in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider this as the mapping information for the suggested LTM candidate PSCell(s).

If the LTM Information SN Addition IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the S-NG-RAN node Addition Preparation procedure has been triggered as part of an MCG LTM.

If the Proposed LTM UE Based TA Measurement ID List IE is contained in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use this information for preparing LTM candidate PSCells, as described in TS 37.340 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE MODIFICATION REQUEST message, it shall start the timer TXnDCprep.

<skip unchanged part>

If the LTM Candidate PSCell Information Update Request IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the included information provides the SCG LTM related information of the prepared candidate PSCells and may include the LTM Candidate PSCell Information Update Acknowledge IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, as described in TS 37.340 [8].

If the CSI Resource Configuration IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration.

If the LTM Configuration ID Mapping List IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider this as the mapping information for the prepared LTM candidate PSCell(s).

If the Proposed LTM No Security Change IDs IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use this information for preparing LTM candidate PSCells, as described in TS 37.340 [8].

If the LTM SCG Security Configuration IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use this information to apply security during SCG LTM.

If the LTM Candidate PSCell To be Cancelled List IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the request concerns the cancellation of LTM candidate PSCell(s) indicated, and may provide the updated information for SCG LTM in the LTM Candidate PSCell Information Update Acknowledge IE of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

If the LTM Information SN Modification IE set to "intra-MN-LTM" is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure has been triggered as part of an MCG LTM.

If the LTM Inter-SN Execution Notification IE set to "executed" is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the UE has been successfully accessed to another candidate SN and may stop data transmission to the UE. If the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Modification Info – SN terminated IE is also included for some PDU session in the PDU Session Resources To Be Modified List IE of the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node may include the Data Forwarding Info from target NG-RAN node IE within the PDU Session Resource Modification Response Info – SN terminated IE of the corresponding PDU sessions in the PDU Session Resources Admitted To Be Modified List IE of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to provide the new data forwarding address information for inter-SN SCG LTM.

If the Proposed LTM UE Based TA Measurement ID List IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use this information for preparing LTM candidate PSCells, as described in TS 37.340 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.1	Xn Setup

#### 8.4.1.1	General

The purpose of the Xn Setup procedure is to exchange application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE 1:	If Xn-C signalling transport is shared among multiple Xn-C interface instances, one Xn Setup procedure is issued per Xn-C interface instance to be setup, i.e. several Xn Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE 2:	Exchange of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2: Xn Setup, successful operation

The NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to the candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

The AMF Region Information IE in the XN SETUP REQUEST message shall contain a complete list of Global AMF Region IDs to which the NG-RAN node1 belongs. The AMF Region Information IE in the XN SETUP RESPONSE message shall contain a complete list of Global AMF Region IDs to which the NG-RAN node2 belongs.

The List of Served Cells NR IE and the List of Served Cells E-UTRA IE, if contained in the XN SETUP REQUEST message, shall contain a complete list of cells served by NG-RAN node1 or, if supported, a partial list of served cells together with the Partial List Indicator IE. The List of Served Cells NR IE and the List of Served Cells E-UTRA IE, if contained in the XN SETUP RESPONSE message, shall contain a complete list of cells served by NG-RAN node2 or, if supported, a partial list of served cells together with the Partial List Indicator IE.

<skip unchanged part>

If the RedCap Broadcast Information IE is included in the Served Cell Information NR IE in the XN SETUP REQUEST message or the XN SETUP RESPONSE message, the receiving NG-RAN node may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the TAI NSAG Support List IE is contained in the XN SETUP REQUEST or in the XN SETUP RESPONSE message, the receiving NG-RAN node shall, if supported, take this IE into account for slice aware cell reselection.

If the TAI Slice Unavailable Cell List IE is contained in the XN SETUP REQUEST or in the XN SETUP RESPONSE message, the receiving NG-RAN node shall, if supported, take this IE into account to deduce slice resource allocation.

If the eRedCap Broadcast Information IE is included in the Served Cell Information NR IE in the XN SETUP REQUEST message or the XN SETUP RESPONSE message, the receiving NG-RAN node may use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the Mobile IAB Cell IE is included in the Served Cell Information NR IE in the XN SETUP REQUEST message or in the XN SETUP RESPONSE message, the receiving NG-RAN node may use it accordingly.

If the XR Broadcast Information IE is included in the Served Cell Information NR IE in the XN SETUP REQUEST message or the XN SETUP RESPONSE message, the receiving NG-RAN node shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information NR IE in the XN SETUP REQUEST message or the XN SETUP RESPONSE message, the receiving NG-RAN node may use this information to determine a suitable target in case of subsequent outgoing mobility during emergency call.

If the WAB-MT Identifier IE is included in the XN SETUP REQUEST message or in the XN SETUP RESPONSE message, the receiving NG-RAN node shall, if supported, consider that the transmitting NG-RAN node is a WAB-gNB, and conclude that the WAB-MT identified by the WAB-MT Identifier IE is co-located with the transmitting NG-RAN node, if needed.

If the Early RACH Resources Requester ID IE is included in the Served Cell Information NR IE in the XN SETUP REQUEST message or the XN SETUP RESPONSE message, the receiving NG-RAN node may use this information to determine an entity requesting early RACH resources when inter-CU LTM preparation is initiated.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

If the TAI Support List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the receiving node shall replace the previously provided TAI Support List IE by the received TAI Support List IE.

If the Cell Assistance Information NR IE is present, the NG-RAN node2 shall, if supported, use it to generate the Served NR Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

<skip unchanged part>

Update of Served Cell Information NR:

-	If Served Cells NR To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old NR-CGI IE according to the information in the Served Cell Information NR IE.

-	When either served cell information or neighbour information of an existing served cell in NG-RAN node1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information NR IE or the Neighbour Information E-UTRAN IE or both. The NG-RAN node2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If the Deactivation Indication IE set to "deactivated" is contained in the Served Cells NR To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption.

-	If Served Cells NR To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old NR-CGI IE.

-	If the Intended TDD DL-UL Configuration NR IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 should take this information into account for cross-link interference management and/or NR-DC power coordination with the NG-RAN node1. The NG-RAN node2 shall consider the received Intended TDD DL-UL Configuration NR IE content valid until reception of a new update of the IE for the same NG-RAN node2.

-	If the NR Cell PRACH Configuration IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

-	If the SFN Offset IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE shall, if supported, use this information to update the SFN0 time offset of the reported cell.

-	If the Supported MBS FSA ID List IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use it according to TS 38.300 [9].

-	If the RedCap Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

-	If the eRedCap Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

-	If the Mobile IAB Cell IE is included in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message or the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node may use it accordingly.

-	If the XR Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

-	If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message or the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node may use this information to determine a suitable target in case of subsequent outgoing mobility during emergency call.

-	If the Early RACH Resources Requester ID IE is included in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message or the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node may use this information to determine an entity requesting early RACH resources when inter-CU LTM preparation is initiated.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.6.2	TA Information Transfer

#### 8.6.2.1	General

The purpose of the TA Information Transfer procedure is to enable the NG-RAN node1 to send the TA related information to the NG-RAN node2. The procedure uses non UE-associated signalling.

#### 8.6.2.2	Successful Operation

Figure 8.6.2.2-1: TA Information Transfer procedure. Successful operation.

The NG-RAN node1 initiates the procedure by sending a TA INFORMATION TRANSFER message.

If the Source gNB ID IE is included, the NG-RAN node2 shall, if supported, consider to forward the associated TA information to source SN indicated by Source gNB ID IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Handover Information Request | O |  |  |  | YES | reject |
| >Proposed LTM No Security Change ID List | O |  | LTM No Security Change ID List 9.2.3.231 |  | – |  |
| >Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | – |  |
| >Reference Configuration | O |  | OCTET STRING | Includes the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >Request for CSI-RS Resource Configuration for L1 Measurements | O |  | ENUMERATED (true, …) |  | – |  |
| >Request for CSI-RS Resource Configuration for Early CSI Acquisition | O |  | ENUMERATED (true, …) |  | – |  |
| >Proposed LTM UE Based TA Measurement ID List | O |  | LTM UE Based TA Measurement ID List 9.2.3.xxx | Indicates the LTM UE Based TA Measurement IDs to be assigned during the preparation of candidate cells. | YES | ignore |
| Early Sync Information Request | O |  | 9.2.3.217 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Handover Information Request Acknowledge | O |  |  |  | YES | ignore |
| >SSB Information | M |  | 9.2.3.228 |  | – |  |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. | – |  |
| >LTM Candidate Configuration | M |  | OCTET STRING | Includes the ltm-CandidateConfig as defined in TS 38.331 [10]. | – |  |
| >LTM No Security Change ID | M |  | INTEGER(1..9,…) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10]. | – |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | – |  |
| >CSI-RS Resource Configuration for L1 Measurements | O |  | 9.2.3.224 |  | – |  |
| >CSI-RS Resource Configuration for Early CSI Acquisition | O |  | 9.2.3.224 |  | – |  |
| >LTM CFRA Resource Information | O |  | 9.2.3.232 |  | – |  |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the candidate cell. | YES | ignore |
| Early Sync Information Response | O |  | Early Sync Information 9.2.3.218 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.1.2	Messages for Dual Connectivity Procedures

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 | This IE is ignored if the S-CPAC Request Information IE is present in the Conditional PSCell Addition Information Request IE or the LTM Candidate PSCell Addition Information Request IE is present. | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Candidate PSCell Addition Information Request | O |  |  |  | YES | reject |
| >LTM Request Indication | M |  | ENUMERATED (request, ...) |  | – |  |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the candidate SN may prepare. | – |  |
| >Suggested LTM Candidate PSCell List | M |  | LTM Candidate PSCell Request List 9.2.3.242 |  | – |  |
| >Proposed LTM No Security Change IDs | M |  | LTM No Security Change ID List 9.2.3.231 | Indicates the LTM No Security Change IDs to be assigned during the preparation of candidate PSCells. | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >SCG Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for LTM is requested. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >Proposed LTM UE Based TA Measurement ID List | O |  | LTM UE Based TA Measurement ID List 9.2.3.xxx | Indicates the LTM UE Based TA Measurement IDs to be assigned during the preparation of candidate PSCells. | YES | ignore |
| LTM Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Candidate PSCell Information Update Request | O |  |  |  | YES | ignore |
| >Multiple S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>LTM Candidate PSCell Prepared List | O |  | 9.2.3.243 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >Proposed LTM No Security Change IDs | O |  | LTM No Security Change ID List 9.2.3.231 | Indicates the LTM No Security Change IDs to be assigned during the preparation of candidate PSCells. | – |  |
| >LTM SCG Security Configuration | O |  | 9.2.3.241 |  | – |  |
| >LTM Candidate PSCell To be Cancelled List | O |  | LTM Candidate PSCell Request List 9.2.3.242 |  | – |  |
| >Proposed LTM UE Based TA Measurement ID List | O |  | LTM UE Based TA Measurement ID List 9.2.3.xxx | Indicates the LTM UE Based TA Measurement IDs to be assigned during the preparation of candidate PSCells. | YES | ignore |
| LTM Information SN Modification | O |  |  |  | YES | ignore |
| >LTM Reconfiguration | M |  | ENUMERATED (intra-MN-LTM, ...) |  | – |  |
| LTM Inter-SN Execution Notification | O |  | ENUMERATED (executed, …) | Indicates that inter-SN SCG LTM was executed. | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.1.5	Messages for L1/L2 Triggered Mobility

#### 9.1.5.4	TA INFORMATION TRANSFER

This message is sent by the NG-RAN node1 to inform the NG-RAN node2 about the Timing Advance value and related information.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TA Information List |  | 1 |  |  | YES | reject |
| >TA Information Transfer Item |  | <1… maxnoofTAList> |  |  | – |  |
| >>Early RACH Resources Requester ID | M |  | 9.2.3.220 | Identifies the entity requesting Early RACH resources. | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | – |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | – |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [35]. | – |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [10]. | – |  |
| >>Source gNB ID | O |  | Global gNB ID 9.2.2.1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| <skip unchanged part> |  |  |  |  |  |  |
| NZP CSI-RS Resources Configuration | O |  | 9.2.2.107 |  | YES | ignore |
| SRS Resource Configuration | O |  | 9.2.2.108 |  | YES | ignore |
| Early RACH Resources Requester ID | O |  | 9.2.3.220 | Identifies the entity requesting Early RACH resources. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.243	LTM Candidate PSCell Prepared List

This IE provides the information related to the prepared LTM candidate PSCell(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| LTM Candidate PSCell Prepared List |  | 1 |  |  |  |  |
| >LTM Candidate PSCell Prepared Item |  | 1 .. < maxnoofLTMCells> |  |  |  |  |
| >>PSCell ID | M |  | NR CGI 9.2.2.7 |  |  |  |
| >>LTM No Security Change ID | M |  | INTEGER (0.. 9, …) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10] |  |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. |  |  |
| >>Early Sync Information | O |  | 9.2.3.218 |  |  |  |
| >>LTM CFRA Resource Information | O |  | 9.2.3.232 |  |  |  |
| >>SSB Information | O |  | 9.2.3.228 |  |  |  |
| >>CSI-RS Resource Configuration for Layer 1 Measurements | O |  | CSI-RS Resource Configuration 9.2.3.224 |  |  |  |
| >>CSI-RS Resource Configuration for Early CSI Acquisition | O |  | CSI-RS Resource Configuration 9.2.3.224 |  |  |  |
| >>Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  |  |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate PSCell identified by the PSCell ID IE. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.xxx	LTM UE Based TA Measurement ID List

This IE contains the UE Based TA Measurement Configuration information for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM UE Based TA Measurement ID List |  | 1.. <maxnoofLTMCells> |  |  |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

===================<skip unchanged part>====================

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

===================<skip unchanged part>====================

id-SRS-Resource-Configuration,

id-SliceToReportForDataCollection-List,

id-PredictedSliceAvailableCapacity,

id-SliceMeasurementInitiationResult,

id-SliceUEPerformance,

id-FiveGProSeLayer3MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay,

id-FiveGProSeLayer2MHRemote,

id-SourceGNB-ID,

id-EarlyRACHResourcesRequesterID,

id-UeBasedTAMeasurementConfiguration,

id-LTM-UEBasedTAMeasurementID-List,

id-ProposedLTM-UEBasedTAMeasurementID-List,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

#### -- L

===================<skip unchanged part>====================

LTMHandoverInformationRequest ::= SEQUENCE {

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List		OPTIONAL,				-- This IE may need to be refined.

targetNG-RANnodeUEXnAPID				NG-RANnodeUEXnAPID				OPTIONAL,

referenceConfiguration					ReferenceConfiguration			OPTIONAL,

lTMConfigurationIDMappingList			LTMConfigurationIDMappingList	OPTIONAL,

cSIResourceConfiguration				CSIResourceConfiguration		OPTIONAL,

requestForCSI-RSResourceConfigForLayer1Measurements		RequestForCSI-RSResourceConfigForLayer1Measurements		OPTIONAL,

requestForCSI-RSResourceConfigForEarlyCSIAcquisition	RequestForCSI-RSResourceConfigForEarlyCSIAcquisition		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMHandoverInformationRequest-ExtIEs} } OPTIONAL,

...

}

LTMHandoverInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ProposedLTM-UEBasedTAMeasurementID-List  CRITICALITY ignore	EXTENSION LTM-UEBasedTAMeasurementID-List		PRESENCE optional },

...

}

LTMConfigurationIDMappingList ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMConfigurationIDMapping-Item

LTMConfigurationIDMapping-Item::= SEQUENCE{

lTMCellID			NR-CGI,

lTMConfigurationID	LTMConfigurationID,

iE-Extensions		ProtocolExtensionContainer {{ LTMConfigurationIDMapping-Item-ExtIEs}}		OPTIONAL

}

LTMConfigurationIDMapping-Item-ExtIEs 	XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMConfigurationID ::=  INTEGER (1..8)

LTM-NoSecurityChangeID-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTM-NoSecurityChangeID

LTM-NoSecurityChangeID ::= INTEGER (1..9,...)

LTMHandoverInformationRequestAcknowledge ::= SEQUENCE {

sSBInformation								SSBInformation,

tCIStateConfigurationList					OCTET STRING,

lTMCandidateConfiguration					OCTET STRING,

lTM-NoSecurityChangeID						LTM-NoSecurityChangeID,

completeCandidateConfigurationIndicator		CompleteCandidateConfigurationIndicator 	OPTIONAL,

cSI-RSResourceConfigurationForLayer1Measurements		CSI-RSResourceConfiguration		OPTIONAL,

cSI-RSResourceConfigurationForEarlyCSIAcquisition		CSI-RSResourceConfiguration		OPTIONAL,

lTMCFRAResourceInformation								LTMCFRAResourceInformation 		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMHandoverInformationAcknowledge-ExtIEs} } OPTIONAL,

...

}

LTMHandoverInformationAcknowledge-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-UeBasedTAMeasurementConfiguration	CRITICALITY ignore	EXTENSION UeBasedTAMeasurementConfiguration		PRESENCE optional },

...

}

LTMCellSwitchInformation	::= SEQUENCE {

jointorDLTCIStateID				JointorDLTCIStateID,

uLTCIStateID					ULTCIStateID 				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { LTMCellSwitchInformation-ExtIEs } }	OPTIONAL,

...

}

LTMCellSwitchInformation-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUEAssociationInformation-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTMUEAssociationInformation-Item

LTMUEAssociationInformation-Item ::= SEQUENCE {

candidateCellID									NR-CGI,    -- This IE needs to be refined

lastTarget-NG-RANnodeUEXnAPID					NG-RANnodeUEXnAPID,

iE-Extensions		ProtocolExtensionContainer { { LTMUEAssociationInformation-Item-ExtIEs} } OPTIONAL,

...

}

LTMUEAssociationInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUpdatesToCandidateNodeInformation ::= SEQUENCE {

referenceConfiguration					ReferenceConfiguration			OPTIONAL,

lTMConfigurationIDMappingList			LTMConfigurationIDMappingList	OPTIONAL,

cSIResourceConfiguration				CSIResourceConfiguration		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMUpdatesToCandidateNodeInformation-ExtIEs} } OPTIONAL,

...

}

LTMUpdatesToCandidateNodeInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUpdatesToCandidateCellInformation-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMUpdatesToCandidateCellInformation-Item

LTMUpdatesToCandidateCellInformation-Item ::= SEQUENCE {

candidateCellID							NR-CGI,

earlySyncInformation					EarlySyncInformation				OPTIONAL,

lTMCFRAResourceInformation				LTMCFRAResourceInformation 			OPTIONAL,

ueBasedTAMeasurementConfiguration		OCTET STRING 						OPTIONAL,

aSSecurityInformation					AS-SecurityInformation				OPTIONAL,

lTM-NoSecurityChangeID					LTM-NoSecurityChangeID				OPTIONAL,

lTMUEAssociation						LTMUEAssociationInformation-List	OPTIONAL,

dataForwardingInfoForLTM-List			DataForwardingInfoForLTM-List		OPTIONAL,

lastTarget-NG-RANnodeUEXnAPID			NG-RANnodeUEXnAPID					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMUpdatesToCandidateCellInformation-Item-ExtIEs} } OPTIONAL,

...

}

LTMUpdatesToCandidateCellInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUpdatesFromCandidateCellInformation-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMUpdatesFromCandidateCellInformation-Item

LTMUpdatesFromCandidateCellInformation-Item ::= SEQUENCE {

candidateCellID										NR-CGI,

lTMCandidateConfiguration							OCTET STRING,

completeCandidateConfigurationIndicator				CompleteCandidateConfigurationIndicator 	OPTIONAL,

cSI-RSReportConfigurationForEarlyCSIAcquisition		OCTET STRING								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMUpdatesFromCandidateCellInformation-Item-ExtIEs} } OPTIONAL,

...

}

LTMUpdatesFromCandidateCellInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMPSCellInformation-AddReq ::= SEQUENCE {

lTM-RequestIndication					ENUMERATED {request, ...},

maxNrofPSCellsToPrepare					MaxNrofPSCellsToPrepare,

suggestedLTMCandidatePSCellList			LTM-PSCell-Request-List,

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List,

cSI-ResourceConfiguration				CSIResourceConfiguration													OPTIONAL,

sCG-ReferenceConfigRequest				ENUMERATED {request, ...} 													OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList												OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-AddReq-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-AddReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-ProposedLTM-UEBasedTAMeasurementID-List  CRITICALITY ignore	EXTENSION LTM-UEBasedTAMeasurementID-List		PRESENCE optional },

...

}

LTMPSCellInformation-AddReqAck ::= SEQUENCE {

lTM-CandidatePSCellPreparedList		LTM-PSCell-Prepared-List												OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { LTMPSCellInformation-AddReqAck-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-AddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-UpdateReq ::= SEQUENCE {

multipleSN-List							MultipleCandidateSN-List,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List											OPTIONAL,

lTM-SCG-SecurityConfiguration			LTM-SCG-SecurityConfiguration-List									OPTIONAL,

lTM-CandidatePSCellToBeCancelled		LTM-PSCell-Request-List												OPTIONAL,

-- reference to the LTM-PSCell-Request-List to be refined

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReq-ExtIEs} }				OPTIONAL,

...

}

LTMPSCellInformation-UpdateReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-ProposedLTM-UEBasedTAMeasurementID-List  CRITICALITY ignore	EXTENSION LTM-UEBasedTAMeasurementID-List		PRESENCE optional },

...

}

LTMPSCellInformation-UpdateReqAck ::= SEQUENCE {

lTM-CandidatePSCellPreparedList			LTM-PSCell-Prepared-List											OPTIONAL,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReqAck-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-UpdateReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTM-PSCell-Prepared-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTM-PSCell-Prepared-Item

LTM-PSCell-Prepared-Item ::= SEQUENCE {

pscell-id											NR-CGI,

lTM-NoSecurityChangeID								LTM-NoSecurityChangeID,

tCI-StatesConfigurationsList						OCTET STRING											OPTIONAL,

earlySyncInformation								EarlySyncInformation									OPTIONAL,

lTMCFRAResourceInformation							LTMCFRAResourceInformation 								OPTIONAL,

sSBInformation										SSBInformation											OPTIONAL,

cSI-RSResourceConfigurationForLayer1Measurements	CSI-RSResourceConfiguration								OPTIONAL,

cSI-RSResourceConfigurationForEarlyCSIAcquisition	CSI-RSResourceConfiguration								OPTIONAL,

complete-CandidateConfigurationIndicator			ENUMERATED {complete, ...}								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTM-PSCell-Prepared-Item-ExtIEs} } 						OPTIONAL,

...

}

LTM-PSCell-Prepared-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-UeBasedTAMeasurementConfiguration	CRITICALITY ignore	EXTENSION UeBasedTAMeasurementConfiguration		PRESENCE optional },

...

}

LTM-UEBasedTAMeasurementID-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF UeBasedTAMeasurementConfiguration

#### -- U

UeBasedTAMeasurementConfiguration	::= OCTET STRING

#### -- S

===================<skip unchanged part>====================

-- Served Cells NR IEs

ServedCellInformation-NR ::= SEQUENCE {

nrPCI								NRPCI,

cellID								NR-CGI,

tac									TAC,

ranac								RANAC						OPTIONAL,

broadcastPLMN						BroadcastPLMNs,

nrModeInfo							NRModeInfo,

measurementTimingConfiguration		OCTET STRING,

connectivitySupport					Connectivity-Support,

iE-Extensions						ProtocolExtensionContainer { {ServedCellInformation-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-NR				CRITICALITY ignore	EXTENSION BPLMN-ID-Info-NR						PRESENCE optional }|

{ ID id-ConfiguredTACIndication			CRITICALITY ignore	EXTENSION ConfiguredTACIndication				PRESENCE optional }|

{ ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst					PRESENCE optional }|

{ ID id-NRCellPRACHConfig				CRITICALITY ignore	EXTENSION NRCellPRACHConfig						PRESENCE optional }|

{ ID id-NPN-Broadcast-Information		CRITICALITY reject	EXTENSION NPN-Broadcast-Information			PRESENCE optional }|

{ ID id-CSI-RSTransmissionIndication	CRITICALITY ignore	EXTENSION CSI-RSTransmissionIndication		PRESENCE optional }|

{ ID id-SFN-Offset						CRITICALITY ignore	EXTENSION SFN-Offset							PRESENCE optional }|

{ ID id-Supported-MBS-FSA-ID-List		CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List			PRESENCE optional }|

{ ID id-NR-U-ChannelInfo-List			CRITICALITY ignore	EXTENSION NR-U-ChannelInfo-List					PRESENCE optional }|

{ ID id-Additional-Measurement-Timing-Configuration-List		CRITICALITY ignore	EXTENSION Additional-Measurement-Timing-Configuration-List		PRESENCE optional }|

{ ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information			PRESENCE optional }|

{ ID id-eRedcap-Bcast-Information		CRITICALITY ignore	EXTENSION ERedcap-Bcast-Information			PRESENCE optional }|

{ ID id-MobileIABCell					CRITICALITY ignore	EXTENSION MobileIABCell							PRESENCE optional }|

{ ID id-XR-Bcast-Information			CRITICALITY ignore	EXTENSION XR-Bcast-Information					PRESENCE optional }|

{ ID id-BarringExemptionforEmerCallInfo	CRITICALITY ignore	EXTENSION BarringExemptionforEmerCallInfo	PRESENCE optional }|

{ ID id-NZP-CSI-RS-Resources-Config		CRITICALITY ignore	EXTENSION NZP-CSI-RS-Resources-Config		PRESENCE optional }|

{ ID id-SRS-Resource-Configuration		CRITICALITY ignore	EXTENSION SRS-Resource-Configuration			PRESENCE optional }|

{ ID id-EarlyRACHResourcesRequesterID	CRITICALITY ignore	EXTENSION EarlyRACHResourcesRequesterID		PRESENCE optional },

...

}

#### -- T

===================<skip unchanged part>====================

TAInformation-List ::= SEQUENCE (SIZE(1..maxnoofTAList)) OF TAInformation-Item

TAInformation-Item ::= SEQUENCE {

earlyRACHResourcesRequesterID	EarlyRACHResourcesRequesterID,

candidateCellID					NR-CGI,

tAValue							TAValue,

preambleIndex					PreambleIndex,

rA-RNTI							RA-RNTI,

tagIDPointer					TagIDPointer	OPTIONAL,

ie-Extension 					ProtocolExtensionContainer { { TAInformation-Item-ExtIEs} } OPTIONAL,

...

}

TAInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SourceGNB-ID	CRITICALITY ignore	EXTENSION GlobalGNB-ID		PRESENCE optional },

...

}

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

===================<skip unchanged part>====================

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

===================<skip unchanged part>====================

id-LTMInterSNExecutionNotification																	ProtocolIE-ID ::= 535

id-LTMPSCellInformation-AddReq																		ProtocolIE-ID ::= 536

id-LTMPSCellInformation-AddReqAck																	ProtocolIE-ID ::= 537

id-LTMPSCellInformation-UpdateReq																	ProtocolIE-ID ::= 538

id-LTMPSCellInformation-UpdateReqAck																ProtocolIE-ID ::= 539

id-LTMPSCellInformation-UpdateReqired																ProtocolIE-ID ::= 540

id-LTMPSCellInformation-UpdateConfirm																ProtocolIE-ID ::= 541

id-LTMPSCellInformation-ChangeRequired																ProtocolIE-ID ::= 542

id-LTMPSCellInformation-ChangeConfirm																ProtocolIE-ID ::= 543

id-LTM-DC-DataForwarding-Indicator																	ProtocolIE-ID ::= 544

id-SourceGNB-ID                                                                                   ProtocolIE-ID ::= XXX

id-EarlyRACHResourcesRequesterID                                                                  ProtocolIE-ID ::= XXX

id-UeBasedTAMeasurementConfiguration                                                              ProtocolIE-ID ::= XXX

id-LTM-UEBasedTAMeasurementID-List                                                                ProtocolIE-ID ::= XXX

id-ProposedLTM-UEBasedTAMeasurementID-List                                                        ProtocolIE-ID ::= XXX

END

-- ASN1STOP