3GPP TSG-RAN WG3 #129bis                                                R3-256861

Prague, CZ, 13th – 17th Oct, 2025

Agenda Item:	12.2.1

Source:	CATT

Title:	Support of multi-hop UE trajectory prediction/feedback

Document for:	Discussion

Introduction

According to the WI [1], the following objective is captured:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

-	Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

-	multi-hop UE trajectory across gNBs;

-	intra-CU LTM;

-	handover enhancements, e.g. inter-CU LTM.

Here we provide some analysis on Multi-hop UE trajectory across gNBs.

Discussion

  - General

In Release 18, due to limited time and complexity of collecting measured UE trajectory from multiple hop gNBs, gNB only collect UE trajectory from one hop gNBs.

In Release 19, the following agreement is achieved in RAN3 #124 meeting:

Support multiple-hop UE trajectory collection and reporting reusing the current mechanisms, which has limited specification impacts. Other solutions can be further discussed.

Multiple-hop UE trajectory is not supported in R19.

Currently source gNBs can only collect one hop measured UE trajectory in R18. Accordingly source gNB can only predict UE trajectory within one hop target gNB because for multiple hop gNBs, source gNB cannot request measured UE trajectory to verify the prediction. As a necessary input of the use cases captured in R18 and R19, Only one hop UE trajectory prediction may degrade the performance of AI/ML module.

As discussed in R18, there are two solutions on transferring UE trajectory feedback: serial mode and parallel mode.

The following figure describes the workflow on how to collect UE trajectory feedback from multi-hop gNBs in serial mode.

Figure: UE trajectory feedback from multi-hop gNBs in serial mode

Step 1: UE connects to gNB1;

Step 2: Data Collection Reporting Initiation procedure is used to configure UE trajectory feedback.

Step 3: UE is handed over to gNB2 and UE trajectory prediction is transferred to gNB2.

Step 4: UE connects to gNB2, and gNB1 keep UE context to collect UE trajectory feedback.

Step 5: gNB2 should trigger Data Collection Reporting Initiation procedure to configure UE trajectory feedback before UE handover to gNB3; The UE Trajectory Collection Configuration in step 5 should be the same with the one in step 2 except the Collection Time Duration. Since there may be many UEs handover from gNB1 to gNB2 and then to gNB3, a common Collection Time Duration could be configured first and then updated later according to the time duration that UE stayed in gNB2. Details could be discussed in normative phase

Step 6: UE is handed over to gNB3 and UE trajectory prediction is transferred to gNB3.

Step 7: UE connects to gNB3, but gNB2 still keep UE context to collect UE trajectory feedback.

Step 8: gNB3 collects measured UE trajectory and send UE trajectory feedback to gNB2.

Step 9: gNB2 collect measured UE trajectory in gNB2 and send UE trajectory feedback to gNB1 which includes measured UE trajectory in both gNB3 and gNB2.

However, there are gNB implementation impacts if serial feedback is adopted for multi-hop UE trajectory feedback. UE context has to be kept in all of the visited gNBs in order to collect necessary UE trajectory feedback until the timer for collection time duration expires.

Observation 1: UE context has to be kept by all of the gNBs that UE connected to during handover in order to collect necessary UE trajectory feedback.

Once one of the gNBs fails to send UE trajectory feedback, the measured UE trajectory collected after the failed gNB would be missing.

Observation 2: The drawback of serial mode is that once one of the gNB in the feedback chain fails to send UE trajectory feedback, all of UE trajectory feedback collected after the failed gNB would be missing.

Another approach is the measured UE trajectory feedback sent to the source node directly, i.e. parallel feedback mode, the above issues on keeping UE contexts may be solved. The figure below illustrates the workflow of parallel feedback.

Figure: UE trajectory feedback in parallel mode

Step 1: UE connects to gNB1;

Step 2: Data Collection Reporting Initiation procedure is used to configure UE trajectory feedback. gNB2 as handover target gNB should be configured by data Collection Reporting Initiation procedure as in step 2a. If gNB3 is within the UE trajectory prediction, data Collection Reporting Initiation procedure may be also triggered towards gNB3 as in step 2b in order to collect UE trajectory feedback if UE could handover to gNB3 in the future.

Step 3: UE is handed over to gNB2 and UE trajectory prediction is transferred to gNB2.

Step 4: UE connects to gNB2, and gNB1 keeps UE context to collect UE trajectory feedback.

Step 5: gNB2 may trigger Data Collection Reporting Initiation procedure to configure UE trajectory feedback before UE handover to gNB3, but if gNB3 is within UE trajectory prediction, it is not needed for gNB2 to perform step5 because gNB1 has done that.

Step 6: UE is handed over to gNB3 during which UE trajectory prediction is transferred to gNB3.

Step 7: UE connects to gNB3.

Step 8: gNB3 collects measured UE trajectory and sends UE trajectory feedback directly to gNB1, so it is not needed for gNB2 to keep UE context.

Step 9: gNB2 collects measured UE trajectory in gNB2 and sends UE trajectory feedback to gNB1.

For parallel UE trajectory feedback, one issue to be discussed is how to identify UE at the initial node, e.g., gNB1 configured data collection to gNB2 and gNB3, when UE1 HO to gNB2 and further HO to gNB3, and the gNB3 send feedback to gNB1 directly, how to enable the gNB1 identify the feedback is from UE1.This issue can be solve by signalling enhancement, solutions FFS.

Observation 3: For parallel feedback mode, the issue to be discussed is how to identify UE at the initial node when it receives the UE feedback.

According to above observations, for serial mode solution, observation 1 and 2 are insuperable problems. While for parallel mode solution, these problems could be solved by enhancing XN interface message. Since UE trajectory feedback is crucial for mobility enhancement, reliable feedback should be considered. Therefore, parallel mode solution is preferred.

Proposal 1: It is proposed for RAN3 to consider the serial mode solution and parallel mode solution on multi-hop UE trajectory feedback.

  - Remaining time

Collection Time Duration for UE Trajectory in UE Trajectory Collection Configuration is included in Data Collection Reporting Initiation procedure. It starts at the first successful handover. For the example illustrated in above serial mode and parallel mode figures, it starts when UE successfully handover from gNB1 to gNB2, but gNB3 is not aware of the starting time point and does not know the accurate collecting time.

There is an issue on how to provide accurate Collection Time Duration to multi-hop target gNBs.

Observation 4: There is an issue on how to provide accurate Collection Time Duration for subsequent handover procedures. One feasible solution is to update Collection Time Duration via Handover request messages.

Proposal 2: Agree that Collection Time Duration could be updated via UE associated message during subsequent handover procedures if serial multi-hop UE trajectory feedback is adopted.

Conclusions

Observation 1: UE context has to be kept by all of the gNBs that UE connected to during handover in order to collect necessary UE trajectory feedback.

Observation 2: The drawback of serial mode is that once one of the gNB in the feedback chain fails to send UE trajectory feedback, all of UE trajectory feedback collected after the failed gNB would be missing.

Observation 3: For parallel feedback mode, the issue to be discussed is how to identify UE at the initial node when it receives the UE feedback.

Proposal 1: It is proposed for RAN3 to consider the serial mode solution and parallel mode solution on multi-hop UE trajectory feedback.

Observation 4: There is an issue on how to provide accurate Collection Time Duration for subsequent handover procedures. One feasible solution is to update Collection Time Duration via Handover request messages.

Proposal 2: Agree that Collection Time Duration could be updated via UE associated message during subsequent handover procedures if serial multi-hop UE trajectory feedback is adopted.

Reference

[1] RP-252867 “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” 3GPP TSG RAN Meeting #109