3GPP TSG-RAN WG3 #129bis                                     R3-256863

Prague, CZ, 13th – 17th Oct, 2025

Title:	Discussion on support of AI/ML enabled inter-CU LTM

Agenda Item:	12.2.3

Source:	CATT

Document for:	Discussion

# 1	Introduction

In RP-251868[1], New SI for AI for NG-RAN lists the following objective:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

  - handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

These scenarios aim to improve handover reliability, reduce latency, and optimize mobility-related resource allocation through predictive and data-driven approaches. In this contribution, we will focus on inter-CU LTM.

- Discussion

2.1	 Avoid Overlapping With Intra-CU LTM

Similar as intra-CU LTM, the objectives for inter-CU LTM by the use of AI/ML also including:

-	Efficient early TA acquisition;

-	beam level UE Location prediction;

-	Optimization on candidate cell selection;

-	Optimization on execution conditions;

This above parts will also be discussed in intra-CU LTM part, and the logic is no much difference. To avoid overlapping, we suggest to discuss the above parts in intra-CU case, and further check whether conclusions are also apply to inter CU LTM. And enhance XnAP accordingly..

Proposal 1: For inter-CU LTM, supports the same objectives as intra-CU LTM, including

-	Efficient early TA acquisition;

-	Beam level UE Location prediction;

-	Optimization on candidate cells selection;

-	Optimization on execution conditions;

Proposal 2: Discuss the above objectives in intra-CU LTM first and check whether similar enhancement shall be introduce to Xn interface.

  - Latency Reduce in Inter-CU LTM

During execution phase, it is up to the gNB-DU implementation when will the gNB-DU signal to the CU. This does not mean that the gNB-DU is “allowed” to signal to the gNB-CU before LTM command is sent to the UE.

Follow LTM procedure, during execution phase, the source DU shall first transmit the Cell Switch Command MAC CE to the UE, And then source DU inform source CU via DU-CU CELL SWICH NOTIFICATION message. Upon receiving the corresponding CU-DU CELL SWITCH NOTIFICATION message, the candidate DU shall initiate UE access detection procedures..

In contrast to intra-CU LTM, the inter-CU scenario exhibits significantly longer interface delays. Source DU shall transmit the DU-CU CELL SWITCH NOTIFICATION to the source CU, after it, source CU transmit it to candidate CU via Xn, and then to the candidate DU. The long latency may decrease LTM performance, i.e., UE finish processing, but the candidate DU still not received the CU-DU CELL SWITCH NOFTIVATION, and cannot detect UE access.

Observation 1: In contrast to intra-CU LTM, the inter-CU scenario exhibits significantly longer interface delays.

It is considering to use AI minimize the drawback bring by longer interface latency, and try to obtain similar UE performance as intra-CU LTM. Possible solutions state as below:

- Source node send the predicted cell/beam in advance, then the candidate cell can start UE detect early

- Candidate node predicted UE access, and start UE detect before receive CU-DU CELL SWITCH NOTIFICATION message.

Proposal 3: For inter-CU LTM, RAN3 support using AI to mitigate the drawback bring by longer interface latency, and increase UE performance. Solutions can be further studied.

3	Conclusion

Proposal 1: For inter-CU LTM, supports the same objectives as intra-CU LTM, including

-	Efficient early TA acquisition;

-	Beam level UE Location prediction;

-	Optimization on candidate cells selection;

-	Optimization on execution conditions;

Proposal 2: Discuss the above objectives in intra-CU LTM first and check whether similar enhancement shall be introduce to Xn interface.

Observation 1: In contrast to intra-CU LTM, the inter-CU scenario exhibits significantly longer interface delays.

Proposal 3: For inter-CU LTM, RAN3 support using AI to mitigate the drawback bring by longer interface latency, and increase UE performance. Solutions can be further studied.

4 Reference

[1] RP-252867 “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” 3GPP TSG RAN Meeting #109