3GPP TSG-RAN WG3 Meeting #129-bis                      R3-256865		\\

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	Completion of Continuous management-based MDT in RAN3

Agenda Item:	9.2.2

Source:	CATT, ZTE, Samsung, China Telecom, CMCC

Document for:	Discussion

# 1	Introduction

In S5-254110, SA5 provides a reply LS regarding continuous MDT, with the proposed solution reflected in the corresponding CRs. The SA5 approach is summarized as follows:

For the Measurement Continuity:

- OAM shall provide a specific TR to the gNBs among the collection area for C-MDT.

- Upon UE RRC state transition (from RRC_idle to RRC_connected), the gNB identify the UE is configured with C-MDT via the specific TR.

For Trace Correlation:

- OAM shall provide a TRSR range per TR for each gNB to avoiding TRSR collision.

- When the new TRSR is assigned, the gNB shall report the correlation between the old and new TRSRs to the TCE for measurement continuity.

In this paper, we will address the remaining details and finalize this topic in RAN3.

2	Discussion

2.1 Stage 3 Impact

Considering the HO scenario, to support continuous MDT, the target gNBs within the configured area shall be capable of identifying the UE has been selected for C-MDT, and the target gNBs re-configured the immediate MDT to the UE as soon as possible. Besides, the target gNB shall report the association between the old TRSR (assigned by the source gNB) and new TRSR (assigned by the target gNB).

To realize C-MDT, a recommended approach is including the (TR, TRSR) in the Handover Request message.

If the target node is configured with C-MDT by the OAM, upon receiving the TR of UE, the target node shall identify the UE is configured with C-MDT, and will re-configuration the immediate and logged MDT configuration to the UE. Also, the target gNB will report the association between the old TRSR (assigned by the source gNB) and new TRSR (assigned by the target gNB) to the OAM.

If the target node is not configured C-MDT by OAM, then the C-MDT shall be terminated upon the UE moving out of the collection area.

This above approach is also applicable for UE resume from RRC_idle, and (TR, TRSR) need to be introduced to the Retrieve UE Context Response message.

Proposal 1: Introduce a (TR, TRSR) as Continuous MDT indicator in Handover Request message to inform target node the UE is configured with C-MDT.

Proposal 2: Introduce a (TR, TRSR) as Continuous MDT indicator in retrieve UE Context Response message to inform new serving node the UE is configured with C-MDT.

2.2 Stage 2 Impact

For Measurement Continuity Problem, when UE transitions to RRC_connected form RRC_idle, the new gNB shall identify the UE is selected to execute C-MDT. Unlike HO and Resume scenarios, TR information can not be obtained form the network side. The new gNB need to obtain TR information from the logged MDT report.

TS 37.320

##### 5.1.1.3.2 Report retrieval For Logged MDT the measurement reporting is triggered by an on-demand mechanism, i.e. the UE is asked by the network to send the collected measurement logs via RRC signalling. UE Information procedure defined in TS 25.331 [4] and TS 36.331 [5] and TS 38.331 [15] is used to request UE to send the collected measurement logs. The reporting may occur in different cells than which the logged measurement configuration is signalled.

According to TS 37.320, logged MDT reporting is triggered by an on-demand mechanism. When a UE transitions to RRC_connected and logged MDT measurement is available, UE shall include an available indicator in RRCSetupcomplete message. It is depend on the gNB to decide when to trigger logged MDT measurement reporting.

Observation 1: In legacy MDT, logged MDT reporting is triggered by an on-demand mechanism, i.e., the network decide when to triggered UE sends the collected measurement.

To avoid UE impact and reduce service interruption, when the gNB receives C-MDT configurations from the OAM, it is propose the gNB to trigger MDT report immediately when UE back from RRC_idle and the available indicator is present in RRCSetupcomplete message.

Proposal 3: If the gNB is supported C-MDT, it is propose the gNB to trigger MDT report immediately when the available indicator is present in RRCSetupcomplete, in order to check whether UE is selected for C-MDT. Capture this requirement in TS 37.320.

Besides, SA5 also defined a new job type, TRSR range, and additional TRSR in Trace Record Header for C-MDT, as these features are exclusively used in management-based MDT scenarios with no identified Stage 3 impact, and the relevant descriptions will be captured in TS 37.320..

Proposal 4: Considering capturing the continuous MDT procedure in TS 37.320, including new job type, TRSR range, and additional TRSR in Trace Record Header for C-MDT.

In TS38.300, it is proposed to further clarify that continuous management-based MDT can be used for AI data collection.

Proposal 5: Suggest adding clarification in TS 38.300 to indicate that continuous Management-based MDT can be used for AI data collection.

According to the incoming LS R3-256523, SA5 has captured in TS 32.422 that for split RAN architecture, the C-MDT data collection activation mechanism is only applicable if a gNB-CU-CP receives C-MDT activation. Corresponding OAM requirements for continuous MDT in split architecture shall be specified in TS 38.401.

Proposal 6: Suggest adding clarification in TS 38.401 about OAM requirements for continuous MDT in split architecture.

3	Conclusion

Proposal 1: Introduce a (TR, TRSR) as Continuous MDT indicator in Handover Request message to inform target node the UE is configured with C-MDT.

Proposal 2: Introduce a (TR, TRSR) as Continuous MDT indicator in retrieve UE Context Response message to inform new serving node the UE is configured with C-MDT.

Observation 1: In legacy MDT, logged MDT reporting is triggered by an on-demand mechanism, i.e., the network decide when to triggered UE sends the collected measurement.

Proposal 3: If the gNB is supported C-MDT, it is propose the gNB to trigger MDT report immediately when the available indicator is present in RRCSetupcomplete, in order to check whether UE is selected for C-MDT. Capture this requirement in TS 37.320.

Proposal 4: Considering capturing the continuous MDT procedure in TS 37.320, including new job type, TRSR range, and additional TRSR in Trace Record Header for C-MDT.

Proposal 5: Suggest adding clarification in TS 38.300 to indicate that continuous Management-based MDT can be used for AI data collection.

Proposal 6: Suggest adding clarification in TS 38.401 about OAM requirements for continuous MDT in split architecture.