3GPP TSG-RAN WG3 Meeting #129bis	R3-256866

Prague, CZ, 13-17 Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1546 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on AI/ML for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For the follow description on Data Collection Reporting Initiation procedure: If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and one or more of the UE performance metrics are requested, the UE Performance Collection Configuration IE shall be included.The NG-RAN node2 shall take the UE Performance Collection Configuration IE into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - the time since S-NG-RAN node addition successfully completed is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell; - the NR-DC with the S-NG-RAN node for the UE is released. For the last terminating condition, it is introduced for NR-DC case, but for a handovered over UE with SCG, after successful handover, if SN is released, UE performance collection is also terminated. NR-DC case should not impact handover case. 2. ASN.1 and tabular is not aligned for Global NG-RAN Cell Identity IE in Future Coverage Modification Item. It is proposed to align ASN.1 with tabular because only NG-RAN Cell should be included. 3. Add reference for Slice Average Packet Loss UL IE in Semantics description according to LS S5-253899. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modification the terminating condition for data collection to only include NR-DC case. - Align ASN.1 with tabular for Global NG-RAN Cell Identity IE in Future Coverage Modification Item. - Add reference for Slice Average Packet Loss UL IE in Semantics description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The terminating condition for NR-DC impacts handover case. ASN.1 and tabular is not aligned. The reference for Slice Average Packet Loss UL IE is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.13, 9.2.3.238, 9.3.5 |  |  |  |  |  |  |  |  |
| .4 |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

8.4.13	Data Collection Reporting Initiation

8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and slice-specific information reporting is requested, the Slice To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and predicted information is requested, the Requested Prediction Time IE shall be included. The Requested Prediction Time IE indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the measured UE trajectory is requested, the UE Trajectory Collection Configuration IE shall be included. The NG-RAN node2 shall take the UE Trajectory Collection Configuration IE into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and one or more of the UE performance metrics are requested, the UE Performance Collection Configuration IE shall be included.The NG-RAN node2 shall take the UE Performance Collection Configuration IE into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	the time since S-NG-RAN node addition successfully completed is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	NG-RAN node2 previously configured as the S-NG-RAN node for the UE is released.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.3.238	Slice Based UE Performance

This IE represents UE performance metrics per S-NSSAI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per S-NSSAI as specified in TS 28.558 [58] clause 6.3.1.4.1. |
| Slice Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per S-NSSAI as specified in TS 28.558 [58] clause 6.3.1.4.2. |
| Slice Average Packet Delay | O |  | Average Packet Delay 9.2.3.187 | Corresponds to the Average Packet Delay per S-NSSAI as specified in TS 28.558 [58], clause 6.3.1.1. |
| Slice Average Packet Loss DL | O |  | INTEGER (0..1000000, ...) | Corresponds to DL PDCP SDU Drop Rate per S-NSSAI. |
| Slice Average Packet Loss UL | O |  | INTEGER (0..1000000, ...) | Corresponds to UL PDCP SDU Loss Rate per S-NSSAI as specified in TS 28.558 [58], clause 6.3.1.3.1. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.5	Information Element definitions

#### -- F

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

globalNG-RANCell-ID						GlobalNG-RANCell-ID,

future-cellCoverageState				INTEGER (0..63, ...),

future-SSB-Coverage-Modification-List	Future-SSB-Coverage-Modification-List		OPTIONAL,

predicted-Coverage-Modification-Cause	Predicted-CoverageModificationCause			OPTIONAL,

timeForFutureCoverageState				INTEGER (1..60, ...)						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>