3GPP TSG-RAN WG3 Meeting #129bis	R3-256882

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	9.2.7

Source:	Nokia

Title:	Open issues of Ambient IoT Release 19

WID/SID:	Ambient_IoT_Solutions-Core

Document for:	Discussion and Decision

# 1	Introduction

The AIoT release 19 work item of AIoT for topology 1 was agreed in [4] with the following scope:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

At RAN3#129 the work item was officially completed. However, a few points remain to be corrected, mostly related to the encoding of IEs, or presence of IEs.

This paper addresses these points to correct and complete the work.

# 2	Discussion

Need of command type

The Command Type would allow the gNB to discriminate between different types of command, namely write, read or disable. This may be useful for the gNB to determine “whether” and “how” the response will be.

However, the “whether” seems solved because RAN2 agreed to always send back an answer and the “how” seems also solved because the expected D2R message size included in the request can already indicate an estimate of the uplink need.

Therefore, we tend to conclude that the need of a “command type” field is not proven at this stage, pending new different decision from RAN2/SA2.

Proposal 1: The “command type” seems not needed at this stage.

Presence of the Reader List in inventory report transfer container

It has been agreed to include the complete indicator in the inventory report message.

The reader report list is currently in the inventory report message and presence was left FFS.

It was clarified at last RAN3#129 that the “estimated number of readers” will be estimated by the AIOTF per gNB but obviously this number will not be very accurate when several gNBs are involved. For example, devices may be all located in another gNB. It is therefore possible that some gNBs will report AIOT devices at multiple intervals, while another gNB reports none. It therefore seems logical to set the presence of the Reader List as optional.

Proposal 2: set the presence of reader list as “optional” in the Inventory Report.

Encoding of the AIoT Device Identification Requested

This IE was encoded as a choice at RAN3#129 with the branch for the group devices encoded as octet string. This was waiting some CT4 clarification. Since then CT4 made some progress. They revised their earlier CR in [5] into [6] and sent the LS in [7] to inform. It turns out that the size can be bit string which needs some alignment.

Besides, another open point was whether to have yet another branch to encode the “all devices”. This one was commented at RAN3#129 as dependent on some security issue. Nevertheless, the “all devices” can be considered as a special case of the group. Therefore, the identification may work without this branch.

Proposal 3: Encode the device identification as “bitstring” without “all devices”.

Correlation ID and AIOTF Identifier

At RAN3#129 both were encoded as octet string, waiting also for some CT4 progress.

Similarly, CT4 made progress and the AIOT correlation ID was agreed to be an integer of size 65535. This is a constrained size and therefore we propose to align.

Also, CT4 agreed that the AIOTF ID can be a pointer to NFInstance ID defined in TS 29.571. Even though this is a “string” it is beneficial to have constrained size in our protocol. Taking the GUAMI as an example, which has 3 octets for PLMN ID, and 3 octets for AMF set and pointer, we propose to define an OCTET STRING (SIZE (6)).

Proposal 4: Align the coding of Correlation ID to integer size 65535 and AIOTF ID to octet string (size (6)) referring to NFinstance ID.

Encoding of Time Interval

The time interval was encoded at RAN3#129 as INTEGER with a range up to 256 and a granularity of 0.1 second. This enables to encode the time interval from 0.1 second up to more than 25 seconds. We don’t see the need to have a finer granularity nor to have a range exceeding the 25 seconds, therefore we propose to leave the value unchanged.

Proposal 5: Agree the Time Interval with granularity of 0.1 second and range up to 25.6 seconds.

Additional signalling

The QoS aspect has not been really discussed in the study phase. Of course, we are not discussing here the QoS of QoS flows but at least some QoS related information can help the AIoT RAN node. For example, the Latency or Max time to Respond is one QoS aspect that can also help the AIoT RAN node determining when to complete the AIoT inventory transaction i.e. it waits for responses and possible manages trigger message repetitions until this latency time is reached.

While this maximum time to respond can have the same granularity as the Time Interval with a step of 0.1 second, its range should be larger than the Time Interval. However, a range up to 10.24 seconds for a total inventory time seems sufficient.

Proposal 6: Add “maximum time to respond” as assistance information in the Inventory Request.

# 3	Conclusion

This paper has provided a review of the remaining issues mostly related to encoding of the AIOT information elements and made the following proposals to complete the work before asn1 gets frozen:

Proposal 1: The “command type” seems not needed at this stage.

Proposal 2: set the presence of reader list as “optional” in the Inventory Report.

Proposal 3: Encode the device identification as bitstring without all devices.

Proposal 4: Align the coding of Correlation ID to integer size 65535 and AIOTF ID to octetstring referring to NFinstance ID.

Proposal 5: Agree the Time Interval with granularity of 0.1 second and range up to 25.6 seconds.

Proposal 6: Add “maximum time to respond” as assistance information in the Inventory Request.

The corresponding CR is available in [8].

# 4	References

- RP-234058, New Study Item: Study on Solutions for Ambient IOT (Internet of Things), Huawei

- RP-240826, Revised Study Item: Study on Solutions for Ambient IOT (Internet of Things), CMCC

- RP-243280, Way Forward for Ambient IoT for release 19 and release 20

- RP-243326, Work Item “Solutions for AIoT in NR”

- C4-252464, Definition of AIoT Device Permanent Identifier, CR0713, TS 23.003

- C4-253570, Updates on AIoT Device Permanent Identifier and Filtering Information

- C4-253575, LS on Structure updates of AIoT Identifiers

- R3-256883, Correction of Ambient IoT