3GPP TSG-RAN WG3 Meeting #129bis	R3-256889

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0491 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on AULI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current TS 38.301 states: For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the core network with Additional ULI, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s broadcasted PLMN/SNPN. The WAB-gNB has same functionality as the gNB defined in TS 38.300, which can suppprt network sharing. When the WAB-gNB is shared, it can broadcast multiple PLMNs/SNPNs (e.g. PLMN A and B). For a specific UE from PLMN A, it is unclear which PLMN/SNPN is used in AULI. - In case the WAB-gNB sends the AULI related to PLMN B, the UE’s CN (PLMN A) cannot use the AULI since it does not have the information of PLMN B. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the PLMN/SNPN in AULI is the UE’s serving PLMN/SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AULI is incorrect for a shared WAB-gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

## 12.5	User Location Information for UEs served by a WAB-gNB

For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the core network with Additional ULI, which includes a TAI and a NR CGI pertinent to the UE’s serving PLMN/SNPN.

If the PLMN/SNPN broadcasted by a WAB-gNB is the same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s broadcasted PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB is determined by the WAB-gNB, based on the WAB-node’s geo-location.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the Additional ULI for UEs served by WAB-gNB is determined by the WAB-gNB, based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is the same as, or different than, the WAB-gNB’s broadcasted PLMN/SNPN.

In case Additional ULI for UEs served by a WAB-gNB changes, e.g., due to WAB-node movement, the WAB-gNB derives the new Additional ULI and reports it via legacy procedures, if required by the core network.

End of Change