3GPP TSG-RAN WG3 Meeting #129bis	R3-256896

# Prague, CZ, 13 – 17 Oct 2025

Agenda item:	13.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RAN-CN procedures and signaling for ISAC

Document for:	Discussion and Decision

# 1	Introduction

The ISAC SID ([1]) agrees following RAN3 related objectives:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution discusses the RAN-CN procedures and signaling for ISAC.

# 2	Functions of the RAN-CN interface for ISAC

The RAN-CN interface should support following functionalities for ISAC:

- Start Sensing

This function allows the CN to start a sensing operation in the RAN. This function is supported by a CN initiated Class-1 procedure.

- Modify Sensing

This function allows the CN to modify an ongoing sensing operation in the RAN.  This function is supported by a CN initiated Class-1 procedure.

- SF-initiated Stop Sensing

This function allows the CN to stop an ongoing sensing operation in the RAN. This function is supported by a CN initiated Class-2 procedure.

The Stop function is usually used to cancel/stop an ongoing sensing operation.

- gNB-initiated Stop Sensing

This function allows the gNB to stop an ongoing sensing operation in the RAN.  This function is supported by a gNB initiated Class-1 procedure.

In some scenarios, the gNB may not have resource to continue the sensing operation, so the gNB request to stop an existing sensing.

- Sensing Data  Report.

This function allows the RAN to report the sensing result. This function is supported by a RAN initiated Class-2 procedure.

Proposal 1: agree the following functionalities for RAN-CN interface.

- Start Sensing

- Modify Sensing

- SF-initiated Stop Sensing

- gNB-initiated Stop Sensing

- Sensing Data  Report.

# 3	RAN-CN procedures for ISAC

To support the above ISAC functionalities over NGAP, the following procedures are required.

## 3.1	Start Sensing procedure

The Start Sensing procedure is to start a sensing operation in the gNB. The SF initiates the procedure by sending the START SENSING REQUEST message to the gNB, including following parameters:

a) Sensing ID, which is used to uniquely identify the session in the gNB.

b) Sensing service type, e.g. detection of an object, tracking of an object, etc.

c)	Sensing target area; indicates the geographical area where the sensing service needs to be performed and may contain, TAI, cell ID, Universal Geographical Area Description (GAD) as specified in TS 23.032 [11], etc.

d)	Sensing time parameters; indicates the time period over which the sensing is to be performed e.g. start time and end time and/or total duration.

e)	Sensing contextual information:

-	target object type (e.g. drone, car, human, etc.): indicates the type of objects and its corresponding properties (e.g. object size, Radar Cross Section) that shall be detected in the specific sensing service request,

-	target object mobility profile (e.g. static, low mobility, etc.): indicates whether fixed objects or objects with a specific mobility profile shall be detected in the specific sensing service request,

f)	Requested Sensing QoS parameters (SQoS); The QoS parameters include positioning accuracy, velocity accuracy and confidence level for the detected object(s), sensing range resolution for object detection, maximum latency to obtain the sensing result, missed detection probability, false alarm probability etc (see also TS 22.137 [2]).

g)	Type of event reporting, e.g. one-time report, periodic report, or event-triggered report, etc.

## 3.2	Modify Sensing procedure

The Modify Sensing procedure is to modify an existing sensing operation in the gNB. The SF initiates the procedure by sending the MODIFY SENSING REQUEST message including the similar parameters as the START SENSING REQUEST message.

## 3.3	SF-initiated Stop Sensing procedure

The SF-initiated Stop Sensing procedure is to stop an existing sensing operation in the gNB. The SF initiates the procedure by sending the STOP SENSING REQUEST message including the Sensing ID.

## 3.4	gNB-initiated Stop Sensing procedure

The gNB-initiated Stop Sensing procedure is to request the stop an existing sensing operation in the SF. The gNB initiates the procedure by sending the STOP SENSING REQUIRED message including the Sensing ID.

## 3.5	Sensing Data Report procedure

The Sensing Data Report procedure is to report the sensing data to the SF. The gNB initiates the procedure by sending the SENSING DATA REPORT message including the Sensing ID and sensing data.

There are different options about the type of sensing data that a SE (i.e., gNB) can report to the SF, such as objects map, point clouds and periodograms. This issue needs to consider the discussion in RAN1, but the usage of point cloud seems to be a reasonable option that provides enough information to the SF for the generation of sensing results and allows their reusability for multiple sensing service requests without reporting a large volume of data from the SE to the SF.

Proposal 2: agree following RAN-CN procedures for ISAC.

- Start Sensing procedure

- Modify Sensing procedure

- SF-initiated Stop Sensing procedure

- gNB-initiated Stop Sensing procedure

- Sensing Data  Report procedure

# 4	Conclusion

In this contribution, we discussed the RAN-CN signaling for ISAC. Our proposals are:

Proposal 1: agree the following functionalities for RAN-CN interface.

- Start Sensing

- Modify Sensing

- SF-initiated Stop Sensing

- gNB-initiated Stop Sensing

- Sensing Data  Report.

Proposal 2: agree following RAN-CN procedures for ISAC.

- Start Sensing procedure

- Modify Sensing procedure

- SF-initiated Stop Sensing procedure

- gNB-initiated Stop Sensing procedure

- Sensing Data  Report procedure

# References

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- TR 23.700-14