3GPP TSG-RAN WG3 Meeting #129-bis	R3-256908

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	10.4

Source: 	Lenovo

Title: 	6G RAN Internal Functional Split and Interfaces

Document for:	Discussion and Approval

1. Introduction

A new study on 6G radio was approved at the RAN#108 meeting and then revised at the RAN#109 meeting with the following objectives [1].

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items. From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4] - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML - Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN NOTE: Coordination with TSG-SA and SA WGs will be done, as necessary. 4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the Rel-20 RAN Plenary 6G Study (FS_6G_RAN_Scen_Req). TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026. TSG#113 (September/2026): RAN plenary to make a decision on additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G). This includes decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

In this contribution, we discuss the 6G RAN internal functional split and interfaces.

2. Discussion

In 5G, RAN architecture introduced CU-DU functional split and CU-CP/CU-UP functional split to improve scalability and flexibility. There was a long discussion on the detailed functional split between the CU and the DU, and 8 options were proposed. Depending on some factors related to radio network deployment scenarios, constraints and intended supported services, the option 2 was finally agreed, where the CU hosts the RRC and PDCP layers while the DU hosts the RLC, MAC and PHY layers. The benefits of the option 2 are [2]:

-	This option will allow traffic aggregation from NR and E-UTRA transmission points to be centralized. Additionally, it can facilitate the management of traffic load between NR and E-UTRA transmission points.

-	This option enables centralization of the PDCP layer, which may be predominantly affected by UP process and may scale with UP traffic load.

-	This option allows a separate U-plane while having a centralised RRC/RRM.

Furthermore, in 5G, the separation of CP and UP for the CU was agreed for the increasing use of very dense deployments where user terminals will be able to connect to multiple transmission points simultaneously. The benefits of the CP-UP separation are [2]:

-	A centralization of CP functions, controlling different transmission points, has the potential to achieve enhanced radio performance.

-	Flexibility to operate and manage complex networks, supporting different network topologies, resources and new service requirements.

-	Alignment with SDN concept that would result in a functional decomposition of the radio access, based on a partial de-coupled architecture, between user and control plane entities and on network abstractions [10].

-	For functions purely handling with CP or UP processes, independent scaling and realization for control and user plane functions operation.

-	Support of multi-vendor interoperability.

With the benefits shown as above, the CU-DU functional split and the separation of CU-CP and CU-UP can be supported in 6G. The 5G RAN internal function split and interfaces can be taken as a baseline for 6G.

Proposal 1: The 5G CU-DU functional split and CU-CP/CU-UP separation can be taken as a baseline for 6G RAN internal functional split.

However, the CU-DU split 5G RAN architecture did not fully realize its intended benefits, and challenges remained in efficiency supporting new services and minimizing operational complexity. As a result, there is no successful large-scale commercial 5G CU-DU split architecture deployment so far. Therefore, some enhancements of the CU-DU functional split can be taken in 6G.

One possible enhancement is to reconsider the entity hosting the RRC layer. The main drawback of the 5G CU-DU split architecture is that the RRC layer is located in the CU, which brings the following disadvantages:

- 	All procedures (e.g., mobility, UE context setup) cause extra control plane latency between the CU and the DU.

-	Reconfiguration of RRC parameters needs the involvement of the CU, even the RRC parameters related to radio resource are generated by the DU. For example, the DU related L1/L2 configurations needs to be configured via the CU, where the CU generates the RRC signalling sent to the UE via the DU transfer. Therefore, the signalling shall go through the path of DU -> CU -> DU -> UE, which results significant latency compared with direct transmission from the DU to the UE.

-	RRC pooling gains are not realized as the DU still owns most of lower layer functionalities.

-	Joint optimization of the CU and the DU procedures are not easily achievable due to too many inter-dependencies.

-	Unnecessary control information over MAC layer due to slow RRC procedure.

Therefore, a possible approach is to move the RRC layer from the CU to the DU. With this enhancement, the drawback of the legacy CU-DU split architecture can be avoided.

Proposal 2: RAN3 to discuss the 6G CU-DU functional split architecture where the DU hosts the (partial) RRC layer.

If the legacy CU-DU functional split in 5G is used, i.e., the DU hosts the RLC, MAC and PHY layers while the CU hosts the PDCP, RRC and SDAP layers, another possible enhancement is to enable direct interface between DUs. With this enhancement, the two DUs can coordinate with each other, especially for the L1/L2 coordination, without the involvement of the CU. Therefore, the signalling latency can be reduced.

Proposal 3: RAN3 to discuss the direct inter-DU interface to facilitate L1/L2 coordination.

In 6G, there are some proposals to revisit the L1/L2 functionalities provided by many companies. For example, the PDCP and RLC/MAC functionalities may be merged into one common protocol layer to provide one common SN and reduced processing. This will impact the CU-DU functional split architecture.

Proposal 4: The 6G RAN internal functional split architecture should take into account the L1/L2 functionalities revisited by RAN2.

Compared with the 5G RAN, the 6G RAN should support some new services, e.g., AI, sensing.

For the AI service, it typically requires high computing demand to guarantee the quality, low latency to support the real-time communication, and high security on the exposed AI data. To satisfy these requirements, the 6G RAN can provide a new function to support the AI service. For example, the new function supports the AI computing resource management, online AI model training, AI model inference, AI data collection and exposure, etc.

For the sensing service, it typically requires data privacy, low latency and flexible deployment (e.g., TRP mono-static, TRP-TRP bi-static, TRP-UE bi-static, UE-TRP bi-static, UE mono-static and UE-UE bi-static). To satisfy these requirements, the 6G RAN can provide a new function to support the sensing service. For example, the new function supports the sensing service initiation/termination to obtain sensing data, sensing data handling to obtain the cloud of points or sensing results, etc.

For the 6G RAN internal functional split architecture, the group should discuss how to support these new RAN functions, e.g., the new RAN functions are in the CU, the DU, or a separate node different from the CU and the DU.

Proposal 5: The 6G RAN internal functional split architecture should be designed to enable the new type of services, e.g., data collection, AI, and sensing.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: The 5G CU-DU functional split and CU-CP/CU-UP separation can be taken as a baseline for 6G RAN internal functional split.

Proposal 2: RAN3 to discuss the 6G CU-DU functional split architecture where the DU hosts the RRC layer.

Proposal 3: RAN3 to discuss the direct inter-DU interface to facilitate L1/L2 coordination.

Proposal 4: The 6G RAN internal functional split architecture should take into account the L1/L2 functionalities revisited by RAN2.

Proposal 5: The 6G RAN internal functional split architecture should be designed to enable the new type of services, e.g., data collection, AI, and sensing.

4. Reference

- RP-252912, Revised SID: Study on 6G Radio.

- TR 38.801: Study on new radio access technology: radio access architecture and interfaces.

- RP-252881: Way forward on 6G fronthaul.