3GPP TSG-RAN WG3 #129bis	R3-256909

Prague, Czech Republic, 13-17 October 2025

Agenda Item:	10.5.1

Source:	Lenovo

Title: 		Consideration on 6G AI use cases

Document for:	Discussion and Approval

- Introduction

In this paper, we discuss the usage of AIML in RAN, considering possible use cases.

- Discussion

  - Recap of 5G AIML use cases

In Rel18, the following RAN3-led AI for RAN use cases have been studied and captured in TR37.817:

- AILM based load balancing to improve:

  - Currently the load balancing decisions relying on the current/past-state cell load status are insufficient. The traffic load and resource status of the network changes rapidly, especially in the scenarios with high-mobility and large number of connections, which may lead to ping-pong handover between different cells, cell overload and degradation of user service quality.

  - It is difficult to guarantee the overall network and service performance when performing load balancing. For the load balancing, the UEs in the congested cell may be offloaded to the target cell, by means of handover procedure or adapting handover configuration. For example, if the Ues with time-varying traffic load are offloaded to the target cell, the target cell may be overloaded with new-arrival heavy traffic. It is difficult to determine whether the service performance after the offloading action meets the desired targets.

- AIML based Network Energy Saving to improve:

  - Inaccurate cell load prediction. Currently, energy-saving decisions rely on current traffic load without considering future traffic load.

  - Conflicting targets between system performance and energy efficiency. Maximizing the system’s key performance indicator (KPI) is usually done at the expense of energy efficiency. Similarly, the most energy efficient solution may impact system performance. Thus, there is a need to balance and manage the trade-off between the two.

  - Conventional energy-saving related parameters adjustment. Energy-saving related parameters configuration is set by traditional operation, e.g., based on different thresholds of cell load for cell switch on/off which is somewhat a rigid mechanism since it is difficult to set a reasonable threshold.

  - Actions that may produce a local (e.g., limited to a single RAN node) improvement of Energy Efficiency, while producing an overall (e.g., involving multiple RAN nodes) deterioration of Energy Efficiency.

- AIML based Mobility Optimization to improve:

  - -	Reduction of the probability of unintended events

  - -	UE Location/Mobility/Performance prediction

  - -	Traffic Steering

In Rel19, the following RAN3-led AI for RAN use cases have been studied and captured in TR38.743:

- AIML based Network Slicing to improve:

  - The fulfilment of target requirements derived from the SLA of each supported network slice.

  - AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements

- AIML based Coverage and Capacity Optimization to improve:

  - The reactive CCO approach used in non-AIML based solution: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

  - With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

- Other miscellaneous use cases: split architecture, continuous MDT.

The following RAN3-led AI for RAN use cases have been studied in 5G supported by RAN-side AIML model: AILM based load balancing, AIML based Network Energy Saving, AIML based Mobility Optimization, AIML based Network Slicing, AIML based CCO.

Besides, AI for RAN use cases, RAN1/RAN2 have also studied the following use cases.

In Rel18, the following RAN1-led AI for air interface use cases supported by UE or RAN-side AIML model have been studied and captured in TR38.843:

- AIML based beam management to improve:

  - Beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement

- AIML based CSI compression to improve:

  - overhead reduction, improved accuracy

- AIML based UL positioning to improve:

  - Positioning accuracy for those with heavy NLOS conditions

The following RAN1-led AI for Air Interface use cases have been studied in 5G supported by UE or RAN-side AIML model: AIML based beam management, AIML based CSI compression, AIML based UL positioning.

In Rel19, the following RAN2-led AI for mobility use cases supported by RAN-side AIML model have been studied and captured in TR38.744:

- AIML based RRM measurement/event prediction to improve:

  - Reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements

  - The handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption)

The following RAN2-led AI for Mobility use case have been studied in 5G supported by UE or RAN-side AIML model: AIML based RRM measurement/event prediction.

From the work organization point of view, we have noticed that both RAN2 and RAN3 have studied and specified solutions for AI for mobility use cases separately.

- RAN2-led AI for Mobility: UE/RAN can predict RRM measurement result (e.g., of serving and neighbor cells) to select the appropriate target cell and trigger the handover proactively.

- RAN3-led AI based Mobility Optimization: RAN can predict the future UE trajectory to select the appropriate target cell.

Without close coordination between RAN2/RAN3, the specified solution tries to minimize the impact on the other group, which leads to suboptimal situation, e.g.,

- RAN3 assumed the RAN can predict the UE trajectory while didn’t consider the UE side AIML capability to predict UE’s trajectory (which could be even more precise)

- RAN2 assumed the RAN can predict RRM measurement result for neighbor cells which triggers the handover while didn’t consider the possible feedback over Xn interface for the sake of monitoring.

This will also result in fragmented feature implementation in the network for the same purpose.

In 5G, RAN2 and RAN3 have separately studied and specified solutions for AI for mobility use cases without close coordination which leads to suboptimal/incomplete design.

  - Consideration of initial RAN3-led AI for RAN use cases

Considering the recap and observation above, we believe the initial RAN3-led study on AI for RAN use cases should focus on those are not overlapped with possible RAN1/RAN2 use cases, e.g., not AI for mobility.

Other RAN3-led AI for RAN use cases can be considered as the start point for the 6G study. In the meanwhile, the relevant non-AIML feature discussion in parallel should be taken into account, e.g., DTX/DRX at RAN and SSB-less cell in NES discussion. In addition, we believe the AI based slicing use case can be considered as part of the AI based load balancing use case, because they may end up with the network load prediction of different granularity or metric.

RAN3 is suggested to initially consider AI for RAN use cases that are not overlapped with RAN1/RAN2, e.g.,

  - AI based load balancing including different granularities, e.g., cell/slicing

  - AI based network energy saving considering the parallel non-AI NES discussion

  - AI based CCO

For those AI for RAN use cases that may have overlap with RAN1/RAN2 use cases, RAN3 is suggested to wait for the progress in RAN1/RAN2 first with possible coordination to achieve a complete design, e.g., AI based mobility

  - Consideration of RAN3-led RAN for AI use cases

In 6G, the RAN is envisioned not just as a conduit for wireless connectivity but as an intelligent, programmable platform that can offer AI inference, training, or general-purpose computing resources to external entities—such as:

- Cloud or centralized AI orchestrators

- Core Network Entity

- Third-party applications (e.g., AR/VR, autonomous vehicles, smart factories)

This is often referred to as "RAN-as-a-Service", where the RAN exposes its computational and AI resources through standardized APIs.

An example is illustrated as below, assuming a 6G RAN is already running its RAN functionality applications and has spare AI/computing resources available, it can expose those resources to the Operator’s AI Orchestration system. Based on operational needs, the Operator AI Orchestration can then configure and deploy its own AI applications (e.g., joint training of an AIML model) onto the RAN’s unused AI/computing resources (e.g., GPU, NPU). For example, by using Docker containerization, these applications can run independently without interfering with each other.

RAN3 studies the 6GRAN-AI-as-Service use case, wherein 6G RAN can expose its spare AI/computing resources to a different node.

- Conclusion

Based on the discussion above, we observe:

Observation 1	The following RAN3-led AI for RAN use cases have been studied in 5G supported by RAN-side AIML model: AILM based load balancing, AIML based Network Energy Saving, AIML based Mobility Optimization, AIML based Network Slicing, AIML based CCO.

Observation 2	The following RAN1-led AI for Air Interface use cases have been studied in 5G supported by UE or RAN-side AIML model: AIML based beam management, AIML based CSI compression, AIML based UL positioning.

Observation 3	The following RAN2-led AI for Mobility use case have been studied in 5G supported by UE or RAN-side AIML model: AIML based RRM measurement/event prediction.

Observation 4	In 5G, RAN2 and RAN3 have separately studied and specified solutions for AI for mobility use cases without close coordination which leads to suboptimal/incomplete design.

Based on the discussion above, we propose:

Proposal 1	RAN3 is suggested to initially consider AI for RAN use cases that are not overlapped with RAN1/RAN2, e.g.,

a.	AI based load balancing including different granularities, e.g., cell/slicing

b.	AI based network energy saving considering the parallel non-AI NES discussion

c.	AI based CCO

Proposal 2	For those AI for RAN use cases that may have overlap with RAN1/RAN2 use cases, RAN3 is suggested to wait for the progress in RAN1/RAN2 first with possible coordination to achieve a complete design, e.g., AI based mobility

Proposal 3	RAN3 studies the RAN-AI-as-Service use case, wherein RAN can expose its spare AI/computing resources to a different node.