3GPP TSG-RAN WG3 Meeting #129bis	R3-256910

Prague, CZ, 13 - 17 October 2025

Agenda item:	 11.2.1

Source:		Lenovo

Title:		Discussion on MRO for inter-CU LTM

Document for:		Discussion and Decision

1	Introduction

It was agreed to support MRO in R20 [1]:

The objective of this work item is to specify data collection enhancement in NR standalone for SON/MDT purpose. The specific objectives of this work item are:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

In this paper, we would discuss MRO for inter-CU LTM.

2	Discussion

In R19, to reduce the mobility latency, inter-CU LTM is further supported on top of R18 intra-CU LTM.

LTM is a procedure in which a gNB receives L1 or L3 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by a cell switch command signalled via a MAC CE. The cell switch command indicates an LTM candidate configuration that the gNB previously prepared and provided to the UE through RRC signalling. Then the UE switches to the target configuration according to the cell switch command.

LTM supports both intra-gNB mobility, and inter-gNB mobility. LTM supports both intra-frequency and inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. LTM is supported only for licensed spectrum. The following scenarios are supported:

-	PCell change in non-CA scenario and non-DC scenario;

-	PCell and SCell(s) change in CA scenario;

-	Dual connectivity scenarios:

-	PCell change together with MCG SCells(s) change and intra-SN PSCell change.

-	PSCell change together with SCG SCell(s) change with or without MN involvement.

-	LTM for simultaneous PCell and inter-SN PSCell change is not supported. Inter-gNB LTM configuration for MN and SN at the same time is also not supported.

In R19 MRO for intra-CU LTM, due to time budget, only MRO for intra-CU MCG LTM is specified even though the previous intention is also to support intra-CU SCG LTM scenario. In R20, when we consider MRO for inter-CU LTM, MRO mechanism agreed for intra-CU LTM in R19 can be taken as baseline. Additionally, the scope of R20 SON/MDT is small and TU for each meerting is limited, we should focus on inter-CU MCG LTM scenario, and MRO for inter-CU SCG LTM would not be supported.

Proposal 1: In R20, MRO for inter-CU MCG LTM should be supported, but not MRO for inter-CU SCG LTM.

In R19, both failure and near failure cases are supported for MRO for intra-CU MCG LTM. Similarily, since failure and near failure are two normal and important cases for MRO considaration in inter-CU MCG LTM, both should be supported in R20.

Proposal 2: In R20, both failure and near failure cases should be considered for MRO for inter-CU MCG LTM.

  - 2.1 MRO for connection failure case in inter-CU LTM

Similar as intra-CU MCG LTM, a connection failure may happen in a inter-CU MCG LTM procedure as following:

- an RLF occurs before inter-CU MCG LTM cell switch is executed;

- a failure occurs during inter-CU MCG LTM cell switch execution;

- an RLF occurs shortly after a successful inter-CU MCG LTM cell switch execution.

For intra-CU MCG LTM, three connection failures including Too Late LTM, Too Early LTM and LTM to Wrong Cell are supported as following, similarily in R20, Too Late LTM, Too Early LTM and LTM to Wrong Cell should also be supported for inter-CU MCG LTM.

- For too late LTM, the following sub-cases are considered:

  - Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

  - Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

  - Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

- For too early LTM, the following sub-cases are considered:

  - Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

  - Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

  - Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

- For LTM to wrong cell, the following sub-cases are considered:

  - Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

  - Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

  - Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

Proposal 3: Too Late LTM, Too Early LTM and LTM to Wrong Cell should be supported for inter-CU MCG LTM.

Additionally in stage 2, the existing connection failure definition and failure detection mechanism of Too Late LTM, Too Early LTM and LTM to Wrong Cell captured in TS38.300 can be reused for inter-CU MCG LTM.

Proposal 4: The existing connection failure definition and failure detection mechanism of Too Late LTM, Too Early LTM and LTM to Wrong Cell captured in TS38.300 can be reused for inter-CU MCG LTM.

Similar as intra-CU MCG LTM, for Too Late LTM, Too Early LTM or LTM to Wrong Cell in inter-CU MCG LTM, the failure may occur due to wrong selection of candidate LTM cell, inappropriate cell switch triggering or wrong target cell selection for LTM cell switch.

Proposal 5: The connection failure due to wrong selection of candidate LTM cell, inappropriate cell switch triggering or wrong target cell selection for LTM cell switch should be considered for inter-CU MCG LTM.

The mechanism of failure detection and optimization for intra-CU MCG LTM as below can be taken as baseline for inter-CU MCG LTM, e.g. CU performs initial MRO analysis,

- for failures due to wrong selection of candidate LTM cell, CU performs MRO optimization;

- for failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, CU forwards the RLF report to the DU responsible for the failure (e.g. the last serving DU in case of too late LTM cell switch, or to the source DU in case of too early LTM cell switch or LTM cell switch to wrong cell), then the DU performs MRO optimization.

However, the key point for failure detection and optimization for inter-CU MCG LTM is which CU (e.g. source CU or target CU) performs MRO analysis, RLF report forwarding or MRO optimization.

For Too Late LTM, a CU receives a RLF report associated to the inter-CU MCG LTM from the UE forwards the RLF report to the source CU via the Failure Indication message over Xn. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the last serving DU, then, the last serving DU performs MRO optimization.

For HOF in Too Early LTM, the source CU receives the RLF report associated to the inter-CU MCG LTM from the UE, or the source CU receives the RLF report via the Failure Indication message over Xn from a CU which receives the RLF report from the UE. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the source DU, then, the source DU performs MRO optimization.

For RLF in Too Early LTM, the source CU receives the RLF report associated to the inter-CU MCG LTM from the UE, or the source CU receives the RLF report via the Handover Report message over Xn from the target CU. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the source DU, then, the source DU performs MRO optimization.

For HOF in LTM to Wrong Cell, the source CU receives the RLF report associated to the inter-CU MCG LTM via the Failure Indication message over Xn from a CU which receives the RLF report from the UE. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the source DU, then, the source DU performs MRO optimization.

For RLF in LTM to Wrong Cell, the source CU receives the RLF report associated to the inter-CU MCG LTM via the Handover Report message over Xn from the target CU. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the source DU, then, the source DU performs MRO optimization.

Proposal 6: For inter-CU MCG LTM, souce CU performs MRO analysis,

- in case of failures due to wrong selection of candidate LTM cell, source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, source CU forwards the RLF report to the DU responsible for the failure (e.g. the last serving DU in case of too late LTM cell switch, or to the source DU in case of too early LTM cell switch or LTM cell switch to wrong cell), then the DU performs MRO optimization.

Besides the failures due to wrong selection of candidate LTM cell, inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the following three cases are also supported and specified in R19 for intra-CU MCG LTM, these cases should also be considered in R20 for inter-CU MCG LTM:

- BFR shortly after successful LTM cell switch to wrong beam;

- LTM cell switch failure due to wrong beam;

- LTM cell switch failure due to outdated TA.

Proposal 7: The connection failure due to BFR shortly after successful LTM cell switch to wrong beam, LTM cell switch failure due to wrong beam and LTM cell switch failure due to outdated TA should be considered for inter-CU MCG LTM.

The following mechanism of identification and optimization for connection failure due to BFR shortly after successful LTM cell switch to wrong beam in intra-CU MCG LTM can be taken as baseline for inter-CU MCG LTM, e.g.

- the target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

- the target DU performs initial analysis, in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, the target DU sends the recovery beam information to the source DU via CU;

- the source DU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

The following mechanism of identification and optimization for LTM cell switch failure due to wrong beam and/or LTM cell switch failure due to outdated TA in intra-CU MCG LTM as below can be taken as baseline for inter-CU MCG LTM, e.g.

- the target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- the target DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source DU via CU;

- the source DU may take it into account for optimisation of target beam selection and/or optimisation of Timing Advance related parameters.

Compared with intra-CU MCG LTM, for inter-CU MCG LTM, both the source CU and target CU should be involved for relevant information (e.g. the BFR beam information, the re-established or recovery beam information, and/or the TA value used at successful RACH-based access) transfer.

Proposal 8: For connection failure due to BFR shortly after successful LTM cell switch to wrong beam in inter-CU MCG LTM,

- target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

- target DU performs initial analysis: in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, target DU sends the recovery beam information to source DU via target CU and source CU;

- source DU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

Proposal 9: For LTM cell switch failure due to wrong beam and/or LTM cell switch failure due to outdated TA in inter-CU MCG LTM,

- target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- target DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to source DU via target CU and source CU;

- source DU may take it into account for optimisation of target beam selection and/or optimisation of Timing Advance related parameters.

  - 2.2 MRO for near failure case in inter-CU LTM

Regarding SHR, for intra-CU MCG LTM, similar as legacy, it may be forwarded to the node(s) which configured the SHR trigger causing the SHR to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG. Upon retrieval of an SHR, the receiving node may analyse whether its mobility configuration needs adjustment.

Similarly, for inter-CU MCG LTM, the legacy SHR forwarding mechanism in Xn/NG interface can be taken as baseline.

Proposal 10: The legacy SHR forwarding mechanism in Xn/NG interface can be taken as baseline for inter-CU MCG LTM.

Similarly, for inter-CU MCG LTM, the legacy SHR forwarding mechanism in F1 interface can be taken as baseline, e.g. SHR may be forwarded by gNB-CU to gNB-DU using the ACCESS AND MOBILITY INDICATION message, then the gNB-DU may take it into account for optimisation of mobility parameters.

Proposal 11: The legacy SHR forwarding mechanism in F1 interface can be taken as baseline for inter-CU MCG LTM.

3	Conclusion

In this paper, MRO for inter-CU LTM is discussed. We have the following proposals:

Proposal 1: In R20, MRO for inter-CU MCG LTM should be supported, but not MRO for inter-CU SCG LTM.

Proposal 2: In R20, both failure and near failure cases should be considered for MRO for inter-CU MCG LTM.

Proposal 3: Too Late LTM, Too Early LTM and LTM to Wrong Cell should be supported for inter-CU MCG LTM.

Proposal 4: The existing connection failure definition and failure detection mechanism of Too Late LTM, Too Early LTM and LTM to Wrong Cell captured in TS38.300 can be reused for inter-CU MCG LTM.

Proposal 5: The connection failure due to wrong selection of candidate LTM cell, inappropriate cell switch triggering or wrong target cell selection for LTM cell switch should be considered for inter-CU MCG LTM.

Proposal 6: For inter-CU MCG LTM, souce CU performs MRO analysis,

- in case of failures due to wrong selection of candidate LTM cell, source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, source CU forwards the RLF report to the DU responsible for the failure (e.g. the last serving DU in case of too late LTM cell switch, or to the source DU in case of too early LTM cell switch or LTM cell switch to wrong cell), then the DU performs MRO optimization.

Proposal 7: The connection failure due to BFR shortly after successful LTM cell switch to wrong beam, LTM cell switch failure due to wrong beam and LTM cell switch failure due to outdated TA should be considered for inter-CU MCG LTM.

Proposal 8: For connection failure due to BFR shortly after successful LTM cell switch to wrong beam in inter-CU MCG LTM,

- target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

- target DU performs initial analysis: in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, target DU sends the recovery beam information to source DU via target CU and source CU;

- source DU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

Proposal 9: For LTM cell switch failure due to wrong beam and/or LTM cell switch failure due to outdated TA in inter-CU MCG LTM,

- target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- target DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to source DU via target CU and source CU;

- source DU may take it into account for optimisation of target beam selection and/or optimisation of Timing Advance related parameters.

Proposal 10: The legacy SHR forwarding mechanism in Xn/NG interface can be taken as baseline for inter-CU MCG LTM.

Proposal 11: The legacy SHR forwarding mechanism in F1 interface can be taken as baseline for inter-CU MCG LTM.

Reference

[1] RP-252560, Revised WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5, China Unicom