3GPP TSG-RAN WG3 Meeting #129bis	R3-256913

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	12.2.2

Source: 	Lenovo

Title: 	AI/ML for Intra-CU LTM

Document for:	Discussion

- 1. Introduction

The Rel-20 study item on AI/ML for NG-RAN phase 3 has the following objectives [1]:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

This paper discusses the potential use cases of AI/ML for intra-CU LTM.

- 2. Discussion

The main idea with LTM is to enable mobility using lower-layer signaling to reduce overhead and interruption time. As shown in the Figure 1, the basic procedure of LTM includes four phases as specified in TS 38.300.

Phase 1: LTM Preparation and configuration.

In this phase, the gNB may select multiple LTM candidate cells based on measurement reports from the UE. It then provides the RRC configurations of these candidate cells to the UE, thereby pre-configuring the UE with LTM candidate cells. With pre-configuration, the latency due to handover preparation can be reduced significantly.

Phase 2: DL/UL Early Synchronization.

In this phase, the UE may perform DL early synchronization with LTM candidate cells. When configured by the network, it can activate Transmission Configuration Indicator (TCI) states for one or more candidate cells other than the current serving cell. For example, the TCI states of LTM candidate cells can be pre-activated before they become the serving cell, enabling the UE to achieve DL synchronization with these cells in advance. This facilitates a faster cell switch when triggered.

The UE may also perform UL early synchronization with LTM candidate cells during this phase. When configured by the network, it can initiate an early UL TA acquisition procedure for one or more candidate cells. The network may request the UE to perform early TA acquisition for a candidate cell prior to triggering a cell switch, with this procedure initiated via a Physical Downlink Control Channel (PDCCH) order. The gNB/gNB-DU associated with the candidate cell calculates the TA value and transmits it via the gNB-CU to the gNB/gNB-DU of the serving cell. When triggering the LTM cell switch, the serving cell then includes this TA value in the LTM cell switch command via a MAC CE.

Phase 3: LTM Cell Switch Execution

The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the gNB. The gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

Depending on the availability of a valid TA value (as resulted by the UL early synchronization), the UE performs either a RACH-less LTM or RACH-based LTM cell switch. If the valid TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network and the UE performs RACH-less LTM cell switch upon receiving the cell switch command.

Phase 4: LTM Cell Switch Complete

The UE completes the LTM cell switch procedure by sending RRC Reconfiguration Complete message to target cell.

Figure 1. LTM procedures

Next, we discuss the potential use cases for usage of AI/ML in each phase.

Use case 1: AI/ML for LTM candidate cells selection

During the LTM preparation phase, the gNB-CU may generate a list of LTM candidate cells based on measurement reports received from UEs. For each candidate cell, the gNB-CU sends a UE CONTEXT SETUP REQUEST message to the corresponding candidate gNB-DU(s), requesting the gNB-DU to prepare resources and configurations for LTM. However, if the gNB-CU selects an inappropriate candidate cell, the LTM cell switch to this candidate cell will fail. In such cases, the UE may either recover its connection or reestablish it to a cell that is not included in the original candidate cell list.

In the meanwhile, we noticed that, it was already supported in Rel18 that gNB/gNB-CU can predict the UE trajectory which helps to improve the legacy L3 based handover. The predicted future UE trajectory can be sent from the source gNB to the target gNB over Xn interface conveyed in Handover Request message.

TS 38.423 9.1.1.1 HANDOVER REQUEST This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover. Direction: source NG-RAN node ® target NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. Source NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the source NG-RAN node; YES; reject. …; …; …; …; …; …; …. Cell Based UE Trajectory Prediction; O; 9.2.3.180; This IE contains information about cells that a UE is predicted to be connected to.; YES; ignore.

Observation 1: In Rel18, it’s supported that gNB/gNB-CU can predict UE’s future trajectory in cell granularity.

The same AI capability to predict UE trajectory can be utilized to assist the LTM candidate cell selection. In particular, to mitigate potential LTM switch failures caused by inappropriate candidate cell selection, the gNB-CU can leverage AI/ML technologies to predict candidate cells with higher accuracy. Specifically, the gNB-CU may use historical UE mobility information and past measurement results to predict the probability of a UE moving to each candidate cell. Based on this predicted probability:

- Candidate gNB-DUs can prioritize accepting candidate cells with a higher UE movement probability, ensuring resource preparation aligns with actual mobility needs.

- The source gNB-DU can reference this probability information when selecting the target cell for the LTM Cell Switch Command MAC CE, further reducing the risk of switch failures.

Proposal 1: RAN3 to discuss the AI/ML for LTM candidate cell selection, e.g., gNB-CU predicts the probability of a UE moving to each candidate cell.

Use case 2: AI/ML for DL/UL Early Synchronization

For DL Early Synchronization, the TCI states of LTM candidate cells can be pre-activated before they become the serving cell. It is more related to AI/ML for beam management which relies on RAN1-led WI.

A key aspect of UL early synchronization is obtaining the TA employed for the candidate cell. For UL early synchronization in LTM, UE performs early TA acquisition procedure before LTM switch execution.

Use case 2-1: AI/ML based Early TA value

To obtain the TA value, the UE must switch to the candidate cell and perform a RACH procedure by transmitting a preamble to it. Even though this occurs prior to the LTM cell switch, service interruption persists because the UE has to redirect its transmitter (Tx) to the candidate cell. This problem is exacerbated when multiple candidate cells are configured, as the UE may need to perform UL early synchronization for each. It also involves network signaling overhead.

It is beneficial to use AI/ML enabling network to predict the TA value for a candidate cell directly which does not need UE perform early TA acquisition procedure anymore. For example, gNB-CU can predict the TA value for a candidate prior to the LTM cell switch and transmit the predicted TA information to the UE via the LTM Cell Switch Command MAC CE. The inputs for TA value prediction can include historical UE location or measurement information associated with a TA value.

With the predicted early TA, the candidate gNB-DU does not need to prepare PRACH and preamble resource anymore and the signalling exchanged among gNB-CU, source gNB-DU and candidate gNB-DU can be significantly reduced.

Proposal 2: RAN3 to discuss AI/ML for Early TA prediction, e.g., gNB-CU predicts the TA value for the candidate cell.

Use case 2-2: AI/ML based UL synchronization occasion prediction

The timing of triggering or performing early TA acquisition is critical. If network can predict the optimal timing to trigger or perform early TA acquisition for a candidate cell e.g., sending PDCCH order, it would facilitate executing early TA acquisition at the best moment. This would also afford the network more time to prepare PRACH and preamble resources, eliminating the need for the candidate RAN node to continuously reserve these resources for early TA acquisition. Thus, it is beneficial to use AI/ML to predict the UL synchronization occasion. The gNB-CU may use the historical measurement information to predict the early TA acquisition occasion towards a candidate cell, e.g., when to send PDCCH order to trigger early TA acquisition procedure.

Proposal 3: RAN3 to discuss AI/ML based UL synchronization occasion prediction, e.g., gNB-CU predicts when to send PDCCH order to trigger early TA acquisition procedure.

Use case 3: AI/ML for LTM Cell Switch

Among the candidate cells, the gNB may predict the most suitable target cell and include it in the LTM Cell Switch Command MAC CE. In the intra-CU scenario, the gNB-DU currently makes cell switch decisions based on L1 measurement reports. However, according to the established principle, AI/ML training and inference functions are exclusively located in the gNB-CU. Consequently, for the gNB-CU to predict the target cell in LTM, it would require access to L1 measurement information

Proposal 4: RAN3 to discuss AI/ML based target cell prediction, e.g., gNB-CU predicts the target cell among the candidate cells.

- 3. Conclusion

In this contribution, we discuss the potential use cases of AI/ML for intra-CU LTM:

Proposal 1: RAN3 to discuss the AI/ML for LTM candidate cell selection, e.g., gNB-CU predicts the probability of a UE moving to each candidate cell.

Proposal 2: RAN3 to discuss AI/ML for Early TA prediction, e.g., gNB-CU predicts the TA value for the candidate cell.

Proposal 3: RAN3 to discuss AI/ML based UL synchronization occasion prediction, e.g., gNB-CU predicts when to sends PDCCH order to trigger early TA acquisition procedure.

Proposal 4: RAN3 to discuss AI/ML based target cell prediction, e.g., gNB-CU predicts the target cell among the candidate cells.

- Reference

[1] RP-252867. Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3. ZTE Corporation, NEC.

[2] TS 38.300