3GPP TSG-RAN WG3 Meeting #129bis	R3-256914

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	13.2

Source: 	Lenovo

Title: 	Discussion on ISAC network architecture

Document for:	Discussion and decision

- Introduction

A new SID for Integrated Sensing and Communication [1] has been approved to be discussed in Rel-20 and following detail objectives are identified for RAN3.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this contribution, following aspects are discussed for ISAC: connection between SF and Sensing gNB, protocol stacks for sensing, gNB-based mono-static sensing and CU-DU split architecture of sensing gNB.

- Discussion

Connection between SF and Sensing gNB

In SA2, there is another SI to discuss sensing, it had been triggered before RAN3 and currently had some progress as specified in the TR 23.700-14 [2]. As agreed in SA2, a new Sensing Function (SF) is introduced to enable the 5GS system architecture to support sensing service, and both Direct Connectivity and Indirect Connectivity can be considered between the Sensing gNB and the SF.

-	Direct Connectivity: the SF communicates directly with the Sensing gNB

## 6.1 Solution #1: Support of Sensing operations with direct interfaces ### 6.1.0 High-level solution Principles The solution is based on the following general principles to support sensing service: - The Sensing Function performs a selection of the Sensing Entity(ies) based on the sensing service request. - The Sensing Function triggers each of the selected Sensing Entities to establish a direct connection with the Sensing Function, if the connection was not yet available. - The Sensing Function uses the direct connection to request the sensing service operation at the Sensing Entity(ies) and obtain the Sensing Data and Associated information from the Sensing Entity(ies).

-	Indirect Connectivity: the SF communicates indirectly with the Sensing gNB via an AMF

## 6.5 Solution #5: AMF-Inclusive Framework for 5GC Sensing ### 6.5.0 High-level solution Principles The high-level solution principles are as follows: - An AMF-inclusive framework where the AMF is responsible for: - Discovery and selection of sensing entities - Forwarding the sensing request to the sensing entities. - Introduction of a new network function for Sensing called Sensing Function (SF) which: - Authorizes the usage of sensing service and selects the relevant AMF(s) based on the sensing service request parameters. - Processes the sensing data to derive the sensing result. - The N1/N2 interface may need to be enhanced to support sensing data transmission.

To align with SA2’s conclusions, RAN3 agrees to support both the direct and indirect connectivity for the Sensing gNB to communicate with the SF.

Proposal 1: To align with SA2’s conclusions, RAN3 agrees to support both the direct and indirect connectivity for the Sensing gNB to communicate with the SF.

Protocol Stacks for sensing

As for the protocol stacks between the Sensing gNB and the SF, for control plane, option #1 is to reference the protocol design for positioning, where a new protocol layer may be introduced for the transport of the sensing information, e.g., sensing related configuration. For example, it is illustrated as XXAP as shown in the Figure 1 both for direct connectivity and indirect connectivity..

Direct Connectivity									 Indirect Connectivity

Figure 1: Option #1 for control plane protocol between Sensing gNB and SF

Another option #2 for control plane is to take the examples defined for A-IoT. In case of Direct Connectivity, the SF needs to transport the sensing information in a NGAP like protocol layer (illustrated as NXAP in Figure 2) established between the sensing gNB and the SF, or maybe it can even reuse the NGAP. And In case of Indirectly Connectivity, the SF needs to transport sensing information to AMF via SBI interface and then the AMF sends the sensing information to the sensing gNB in NGAP.

Direct Connectivity									 Indirect Connectivity

Figure 2: Option #2 for control plane protocol between Sensing gNB and SF

Anyway, both Option #1 and Option #2 may have impacts on the NGAP. However, Option #1, with additional architecture impacts, needs to introduce a new protocol specification. Considering R20 is the later phase the 5G, Option #2 is preferred to be used for control plane sensing information transport between the sensing gNB and the SF with less architecture impacts.

Proposal 2: Reuse NGAP or introduce a NGAP like protocol to transport control plane sensing information between sensing gNB and SF.

Proposal 3: Agree the protocol stacks for control plane in Figure 2 for both Direct Connectivity and Indirectly Connectivity between sensing gNB and SF.

As for the user plane, in case of direct connectivity, it is assumed that the Sensing gNB triggers a NG like interface (NX interface) establishment towards the SF. And the SF triggers data connection establishment towards the Sensing gNB. While for indirect connectivity, it is assumed that AMF notifies the Sensing Function about the existence of the Sensing gNB. Then the SF triggers data connection establishment towards the Sensing gNB. The protocol stack for user plane between the Sensing gNB and the SF is shown in Figure 3. Where a GTP-U/UDP/IP based protocol is used to align with other user plane protocol like NGAP/XnAP/F1AP for 5G. The sensing data at least includes the sensing report generated by the sensing gNB. While the format of sensing data, it requires further inputs from RAN1.

Figure 3: User plane protocol between Sensing gNB and SF

Proposal 4: Agree the protocol stack for user plane in Figure 3 between sensing gNB and SF, where the sensing DATA is encapsulated by GTPU/UDP/IP layers.

gNB monostatic sensing

As specified in the SID, R20 ISAC only considers gNB-based mono-static sensing for UAV sensing target use cases and no inter-gNB coordination will be studied for gNB bistatic sensing. For gNB-based mono-static sensing, the gNB which is selected as the sensing entity functions as both a Sensing Transmitter and a Sensing Receiver. In some cases, same TRP or same DU of the sensing gNB is selected as both the Sensing Transmitter and the Sensing Receiver. In some other cases, to alleviate the self-interference and avoid duplex issue on the sensing entity, different TRPs or different gNB-DUs of the sensing gNB may be selected as the Sensing Transmitter or the Sensing Receiver.

Proposal 5: For gNB-based mono-static sensing, same TRP or same DU of the sensing gNB may be selected as both the Sensing Transmitter and the Sensing Receiver.

Proposal 6: For gNB-based mono-static sensing, different TRPs or different gNB-DUs of the sensing gNB may be selected as a Sensing Transmitter or a Sensing Receiver.

CU-DU split architecture

A gNB may consist of a gNB-CU and one or more gNB-DU(s) in the current architecture. If the sensing procedure cannot be applied in the CU-DU split architecture, there are some limitations on the selection of sensing gNB as the sensing entity. In addition, CU-DU split architecture can achieve combination of sensing result from different DUs to improve sensing accuracy. And the case that different DUs as the Sensing Transmitter or the Sensing Receiver can also effectively reduce the interference of sensing signals.

Proposal 7: For gNB-based mono-static sensing, the sensing gNB supports CU-DU split architecture.

Sensing capability of gNB

As a sensing entity, the sensing gNB at least supports the functionalities of sensing signal transmission and reception (or measurement). While for the sensing data processing, it might be completely processed by the SF. Alternatively, the sensing gNB needs to support the functionality to pre-process the sensing data and then sends the pre-processed data to the SF rather than to send the raw sensing measurement result to the SF. In this way, the volume of transmitted sensing data between the sensing gNB and the SF can be decreased.

Proposal 8: RAN3 to study whether a sensing gNB supports the functionality of pre-processing the sensing data.

# Conclusion

This contribution aims to discuss some architecture issues for 5G ISAC. And following observations and proposals are concluded.

Proposal 1: To align with SA2’s conclusions, RAN3 agrees to support both the direct and indirect connectivity for the Sensing gNB to communicate with the SF.

Proposal 2: Reuse NGAP or introduce a NGAP like protocol to transport control plane sensing information between sensing gNB and SF.

Proposal 3: Agree the protocol stacks for control plane in Figure 2 for both Direct Connectivity and Indirectly Connectivity between sensing gNB and SF.

Proposal 4: Agree the protocol stack for user plane in Figure 3 between sensing gNB and SF, where the sensing DATA is encapsulated by GTPU/UDP/IP layers.

Proposal 5: For gNB-based mono-static sensing, same TRP or same DU of the sensing gNB may be selected as both the Sensing Transmitter and the Sensing Receiver.

Proposal 6: For gNB-based mono-static sensing, different TRPs or different gNB-DUs of the sensing gNB may be selected as a Sensing Transmitter or a Sensing Receiver.

Proposal 7: For gNB-based mono-static sensing, the sensing gNB supports CU-DU split architecture.

Proposal 8: RAN3 to study whether a sensing gNB supports the functionality of pre-processing the sensing data.

# Reference

- RP-252819. Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR.

- TR 23.700-14. Study on Integrated Sensing and Communication; Stage 2 (Release 20).