3GPP TSG-RAN WG3 Meeting #129bis	R3-256916

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	14.2

Source: 	Lenovo

Title: 	On A-IOT Topology 2

Document for:	Approval

- 1. Introduction

The Rel-20 work item on solutions for A-IOT in NR phase 2 has the following objectives [1]:

- Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility.

This paper discusses the open issues on topology 2 of A-IOT.

- 2. Discussion

As described in TR 23.700-30, the architectural assumptions for support topology 2 are

- No Rel-19 AIoT Device impact is expected.

- The following topology as defined in TR 38.848 is assumed:

  - BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control

- Only Indirect Connectivity (i.e. NG-RAN <-> AMF <-> AIOTF) is applied.

- The support of Topology 2 in this study is based on the RRC-based option and the interim conclusions of Rel-19 in TR 23.700-13is the baseline.

Based on the assumption, the protocol stack for topology 2 is given in the Fig. 1.

Figure 1. Protocol Stack for RRC based option.

For RRC based protocol stack, the messages between the UE readers and the AIOTF are delivered using RRC between UE and RRC, NGAP between gNB and AMF, and using an SBI interface between AMF and AIOTF.

In Rel-19, the A-IOT information transfer container are defined between NG-RAN node and AIOTF. It is assumed that AMF transparently relays the A-IOT information transfer container between NG-RAN node and AIOTF. The Rel-19 defined signalling can be reused for topology 2 with necessary enhancements.

Proposal 1: The A-IOT related messages and IEs defined in Rel-19 NGAP, specifically for the Topology 1 indirect scenario, can be reused for Topology 2.

Besides the protocol stack, the main issues to support RRC based protocol stack for topology 2 includes:

- UE reader authorization and revocation;

- UE reader selection;

- UE reader mobility.

UE reader authorization and revocation

From our perspective, it is essential for the UE to first obtain authorization as an AIoT reader before interacting with AIoT Devices to perform AIoT service operations. Additionally, this authorization should be fully controlled by the operator with which the UE reader is currently connected and registered.

What’s more, the authorization is not a one time for all thing and should be associated with the validity information, which can be stored as part of the UE subscription data in the UDM/UDR. The validity information (e.g., time based, or location based) can not only be used by the AMF and the AIOTF to determine whether to authorize the UE as an AIoT reader but also can be used by the AMF and the AIOTF as one of the triggers/factors to revoke the UE reader authorization, i.e., UE will stop acting as the AIoT reader. This kind of revocation can ensure the operator has the control and flexibility over the UE reader and make full use of the resources.

The basic procedure to support UE reader authorization and revocation can include:

- The UE subscription data stored in the UDM is enhanced with UE reader allowed indication, and the associated validity information per UE that contains the allowed time/duration, allowed area/location (e.g., TAI, cell ID, AIoT Area) for the UE as an AIoT reader. This information is available to both AMF and AIOTF.

- The AMF performs the UE reader authorization by checking the UE subscription data stored in the UDM.

- AMF can determine to revoke the UE reader authorization, based on local policies and configuration, UE reader associated validity information, and subscription data update from UDM, and sends the UE reader revocation message to both the NG-RAN (that serves the UE reader) and the UE. NG-RAN will release the radio resources configured for that UE to act as an AIoT reader.

- AIOTF can also determine to revoke the UE reader authorization based on local configuration that is related to the AIoT service.

Although the discussion is still pending to SA2, the RAN3 impact can be foreseen to add AIOT UE reader authorization and revocation indications in NG-AP e.g., in UE associated signalling.

Proposal 2: Introduce AIOT UE reader authorization and revocation indications in NG-AP e.g., in UE associated signalling.

UE reader selection

For UE reader selection, there are three scenarios needs to be considered

- Scenario 1: AF requests a specific UE reader for AIoT service operation.

- Scenario 2: Any authorized UEs can be selected as UE reader.

- Scenario 3: Fixed deployed UE reader.

In Scenario 1, a third-party AF intends to use specific UE reader(s) for AIoT service operations and includes the UE reader information (i.e., UE ID) in the service operation request sent to the core network. A key potential issue in this scenario is how to convey the requested UE reader information from the AIOTF to the NG-RAN node.

As illustrated in the protocol stack Fig.1, the AIoT information exchanged between the AIOTF and NG-RAN is transparent to the AMF. This transparency gives rise to a critical question: which UE ID format should the AIOTF use to communicate with the NG-RAN? For instance, if the AF requests a specific UE reader using a GPSI, the AIOTF or NEF will first resolve this GPSI to a SUPI via the UDM. However, the AIOTF cannot directly send the SUPI to the NG-RAN as the UE reader ID - since the intermediate AMF cannot read or translate the SUPI (e.g., into an ID recognizable by the NG-RAN), and the NG-RAN itself cannot obtain the SUPI directly due to security concerns. On the other hand, if the UE ID is outside the AIOT information transfer container and readable to the AMF, the AMF between the AIOTF and NG-RAN may not always be the AMF that currently serves the UE (i.e., serving AMF).

Proposal 3: For the scenario that AF requests a specific UE reader for AIOT service operation, how to convey the requested UE reader information from the AIOTF to the NG-RAN node needs to be addressed.

In Scenario 2, any authorized UE can be selected to act as a UE reader. Since the UE reader may be a regular UE, radio conditions must be taken into account in such cases. In other words, the final selection of the UE reader should reside with the NG-RAN node, as it is aware of each UE's radio conditions.

As specified in Proposal 1, the NG-RAN node is already aware of UE reader authorization status - for example, through the Initial UE Context Setup Request or UE Context Modification Request messages. For example, in the inventory request, the AIOTF needs to provide the inventory area information to the NG-RAN node. With the inventory area, the NG-RAN node selects a appropriate UE reader among the authorized UEs.

Proposal 4: For the scenario that any authorized UEs can be selected as UE reader, the NG-RAN node selects the UE readers among the authorized UEs within the inventory area.

In Scenario 3, the UE reader is assumed to be deployed as a fixed device within a specific area. In this case, both the UE reader's location and its ID can be preconfigured in the NG-RAN node and the AIOTF. The AIOTF may then select a UE reader based on this preconfigured information.

Proposal 5: For the scenario that UE is deployed as fixed UE reader, the AIOTF can select the UE reader according to the pre-configuration information of the UE reader.

UE reader mobility

Since the UE reader can be a regular UE, the UE reader may handover to another cell from serving cell. It is also possible for the UE to experience a RLF and subsequently reconnect to another cell. In these cases, how to handle the UE reader’s A-IOT context should be addressed.

One potential solution is that when a handover or RLF occurs, the NG-RAN node terminates the UE's role as a UE reader and selects another authorized UE to take over. With this approach, the source NG-RAN node does not need to forward the UE's AIOT-related information to the target NG-RAN node. However, a challenge arises: when the UE receives a handover command, an LTM Cell Switch Command, or when an RLF is detected, the UE may have collected inventory data that has not yet been successfully transmitted to the network. The handling of such data requires further discussion.

Another consideration is that if a new UE is selected as the UE reader, the inventory procedure would have already been initiated by the previous UE reader. It is therefore necessary to address how service continuity can be maintained during the UE reader switch for the same inventory session.

Another solution is the source NG-RAN node indicates the UE is acting as a UE reader to the target gNB. The target NG-RAN node decides whether the UE can continue to be UE reader.

Proposal 6: RAN3 to discuss how service continuity can be maintained during the switch of UE readers for the same inventory session.

Proposal 7: RAN3 to discuss whether source the NG-RAN node should provide UE reader information to the target NG-RAN node during handover preparation.

- 3. Conclusion

In this contribution, we discuss the potential issues to support A-IOT topology 2.

Proposal 1: The A-IOT related messages and IEs defined in Rel-19 NGAP, specifically for the Topology 1 indirect scenario, can be reused for Topology 2.

Proposal 2: Introduce AIOT UE reader authorization and revocation indications in NG-AP e.g., in UE associated signalling.

Proposal 3: For the scenario that AF requests a specific UE reader for AIOT service operation, how to convey the requested UE reader information from the AIOTF to the NG-RAN node needs to be addressed.

Proposal 4: For the scenario that any authorized UEs can be selected as UE reader, the NG-RAN node selects the UE readers among the authorized UEs within the inventory area.

Proposal 5: For the scenario that UE is deployed as fixed UE reader, the AIOTF can select the UE reader according to the pre-configuration information of the UE reader.

Proposal 6: RAN3 to discuss how service continuity can be maintained during the switch of UE readers for the same inventory session.

Proposal 7: RAN3 to discuss whether source the NG-RAN node should provide UE reader information to the target NG-RAN node during handover preparation.

- Reference

[1] RP-252867. Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3. ZTE Corporation, NEC.

[2] TS 38.300