3GPP TSG-RAN WG3 #129bis	R3-256922

Prague, Czech Republic, 13-17 October 2025

Agenda Item:	13.2

Source:	Ericsson, Jio Platforms

Title:	Discussion on Network Architecture for ISAC

Document for:	Discussion

# Introduction

In [1], a study item on Integrated Sensing and Communication (ISAC) for NR was approved, listing the following RAN3 related objectives:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3]

Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3]

- Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts.

- No inter-gNB coordination will be studied.

- Coordination with SA2 as necessary.

RAN3#129bis is the first meeting dedicated to ISAC SI. This RAN3 study on architecture and signaling procedures is expected to run for five meetings and complete at RAN3#132 (May 2026).

This paper discusses ISAC network architecture for NG-RAN.

# Discussion

ISAC allows the system to detect and localize passive targets, as a service in NG-RAN. ISAC is based on sensing functionality in the NG-RAN (defined as “sensing entity” by SA2 [2]) and on a Sensing Function (SF) in the 5GC.

As part of the SID [1], RAN3 discussion should:

- Discuss logical functions for ISAC to be included in the gNB,

- Take into account logical functions for ISAC to be included in the SF,

- Include at least a quantitative discussion of the amount of information to be transferred between them.

From the above points, the definition for the logical interface between NG-RAN and SF, and its functionality, will descend.

ISAC procedures and signaling are discussed in a separate paper. [3]

## Initial Observations

Observation 1: As no inter-gNB coordination will be studied, ISAC does not impact Xn.

Observation 2: Targets are passive and UEs are currently not involved in ISAC in any way, so there is currently no UE-associated signaling nor UP in ISAC.

The SF requests the gNB to perform sensing measurements, then the gNB reports back the results of the sensing processing to the SF.

Observation 3: ISAC signaling between gNB and SF consists of sensing information requests from SF to gNB and result reports from gNB to SF.

Due to the specific nature of sensing, the amount of information to be transferred from gNB to SF as sensing results is considerable. RAN3 may expect quantitative evaluations from RAN1 later in the course of the study, but at least for now RAN3 should consider that it may be much larger than for existing functionalities (e.g. positioning).

Observation 4: The amount of information to be transferred from gNB to SF as sensing results is considerable; it may be much larger than for other existing functionalities.

The above sets an important requirement not only for transport network capacity, but also (and more importantly) for logical architecture design. Any architecture which relies on message routing from gNB to SF via another node (e.g. the AMF) is clearly not scalable, as it adds an additional burden on already deployed AMFs.

Observation 5: A sensing architecture which relies on message routing from gNB to SF via e.g. the AMF is clearly not scalable, as it adds an additional burden on already deployed AMFs.

Proposal 1: Direct connection of a logical interface between gNB (sensing data source) and SF should be supported, with no impact on AMF.

A simple logical architecture for ISAC (this corresponds to e.g. Sol. 21 in [2]), is shown in Figure 1.

Figure 1: Logical architecture for ISAC (corresponding to Sol. 21 in [2]).

Due to the above observations, the Ns interface carries non-UE-associated CP signaling only.

Proposal 2: Ns, the logical interface between gNB and SF, only carries non-UE-associated CP signaling.

Sensing reports might be quite large. Not all deployed transport networks may be capable of transporting such information as CP messages. For this reason, it seems beneficial to also (optionally) support sensing result reporting via other means, i.e., a direct data connection to be set up based on information exchanged over CP signaling between the NG-RAN and SF (similarly to existing MDT reporting). This allows for maximum flexibility and scalability in the future, to support larger periodic sensing reports, and to minimize the load on transport networks.

Proposal 3: Direct CP connection between NG-RAN node to the Sensing Function  (and optionally a direct data connection) should be supported to send the sensing reports; an optional direct data connection for result reporting seems also beneficial.

## gNB Functionality for ISAC

Ongoing discussions in RAN1 refer to the TRP (Transmission and Reception Point), which is responsible for transmitting and/or receiving sensing signals. From the point of view of logical architecture:

- A TRP is part of the gNB, hence it is not visible outside the gNB.

- A gNB can have one or more TRPs.

Proposal 4: The TRP, which is responsible for transmitting and/or receiving sensing signals, is part of the gNB and is not visible outside the gNB; a gNB can have one or more TRPs.

Pending RAN1 discussion, it is likely that sensing results to be reported may include e.g. (x,y,z) object location, 3D velocity, range, and possibly others. For tracking purposes, sensing results produced by the gNB may need to be reported periodically (details about periodic reporting are found in [3]).

Sensing results will be generated by appropriately processing one or more sensing measurements, as collected by TRP(s) which are part of the gNB.

Following the considerations made in Sec. 2.1, including e.g. those on the amount of data to be transferred from RAN to CN vs. backhaul requirements, it seems critical that processing is done in the gNB, to generate the sensing results to be reported.

The alternative, i.e. sending pre-processed results (e.g. doppler, AoA, delay measurements) to SF, would increase processing load on CN, would be much less “backhaul-friendly”, and might require additional radio awareness in the CN and/or additional information from the RAN for final processing.

Proposal 5: Processing should be done in the gNB to generate the sensing results to be reported to the SF.

## Considerations on split gNB

Some initial considerations on ISAC with respect to split gNB can be made.

First of all, the TRP is part of the gNB-DU. A gNB-DU may have more than one TRP.

Proposal 6: For a split gNB, the TRP is part of the gNB-DU; a gNB-DU may have more than one TRP.

We should now consider other impacts with respect to split gNB. Rel-20 is fairly late in the NG-RAN lifecycle, and ISAC is a significantly new technology. For this reason, at this point it seems beneficial to leave to split gNB implementations to decide where sensing measurement processing takes place (i.e. gNB-CU or gNB-DU). This seems to have the following benefits:

- It allows maximum implementation flexibility,

- It avoids F1AP impacts due to ISAC;

- It avoids the need to discuss e.g. potential upgradeability of already existing gNB-DUs for ISAC.

In addition, considering that ISAC is expected to be adopted also as a 6G feature, and that 6G RAN architecture discussions are independent, this also avoids the risk of pre-empting 6G ISAC in any way due to architecture considerations which are only pertinent to NG-RAN.

Proposal 7: To limit impacts on NG-RAN and any possible additional impacts on 6G ISAC, split gNB architecture should not be considered in NG-RAN ISAC.

Proposal 8: Agree the TP in Annex.

# Conclusions and Proposals

Our observations and proposals are summarized below.

Observation 1: As no inter-gNB coordination will be studied, ISAC does not impact Xn.

Observation 2: Targets are passive and UEs are currently not involved in ISAC in any way, so there is currently no UE-associated signaling nor UP in ISAC.

Observation 3: ISAC signaling between gNB and SF consists of sensing information requests from SF to gNB and result reports from gNB to SF.

Observation 4: The amount of information to be transferred from gNB to SF as sensing results is considerable; it may be much larger than for other existing functionalities.

Observation 5: A sensing architecture which relies on message routing from gNB to SF via e.g. the AMF is clearly not scalable, as it adds an additional burden on already deployed AMFs.

Proposal 1: Direct connection of a logical interface between gNB (sensing data source) and SF should be supported, with no impact on AMF.

Proposal 2: Ns, the logical interface between gNB and SF, only carries non-UE-associated CP signaling.

Proposal 3: Direct CP connection between NG-RAN node to the Sensing Function  (and optionally a direct data connection) should be supported to send the sensing reports; an optional direct data connection for result reporting seems also beneficial.

Proposal 4: The TRP, which is responsible for transmitting and/or receiving sensing signals, is part of the gNB and is not visible outside the gNB; a gNB can have one or more TRPs.

Proposal 5: Processing should be done in the gNB to generate the sensing results to be reported to the SF.

Proposal 6: For a split gNB, the TRP is part of the gNB-DU; a gNB-DU may have more than one TRP.

Proposal 7: To limit impacts on NG-RAN and any possible additional impacts on 6G ISAC, split gNB architecture should not be considered in NG-RAN ISAC.

Proposal 8: Agree the TP in Annex.

# References

RP-252819	Study on Integrated Sensing and Communication (ISAC) for NR.

TR 23.700-14 v. 1.0.0.

R3-257025	Discussion on RAN-CN procedures and signaling to support ISAC, Ericsson, Jio Platforms.

W. Shakespeare, Romeo and Juliet, II:ii,46-47.

TS 38.300 Rel-18.

# Text Proposal for TR 38.xxx

START OF CHANGES

## x.1 Architecture

As no inter-gNB coordination will be studied, ISAC does not need to impact Xn.

Targets are passive and UEs are currently not involved in ISAC in any way, so there is currently no need for UE-associated signaling or UP in ISAC.

The SF requests the gNB to perform sensing measurements, and the gNB reports the results back to the SF.

ISAC signaling between gNB and SF consists of sensing information requests from SF to gNB and result reports from gNB to SF.

Due to the specific nature of sensing, the amount of information to be transferred from gNB to SF as sensing results is considerable: it may be much larger than for existing functionality (e.g. SON, MDT, positioning).

The above sets an important requirement not only for transport network capacity, but also (and more importantly) for logical architecture design. Any architecture which relies on message routing from gNB to SF via another node (e.g. the AMF) is clearly not scalable, as it adds an additional burden on already deployed AMFs. Direct connection of a logical interface between gNB (sensing data source) and SF should be supported, with no impact on AMF.

A possible logical architecture for ISAC is shown in Figure x.1-1.

Figure x.1-1 Logical architecture for ISAC.

Due to the above observations, the Ns interface terminated at the gNB and at the SF carries non-UE-associated CP signaling only.

It seems beneficial to also (optionally) support sensing result reporting via other means, i.e., a direct data connection to be set up based on information exchanged over CP signaling between the NG-RAN and SF (similarly to existing MDT reporting). This allows for maximum flexibility and scalability in the future, to support larger periodic sensing reports, and to minimize the load on transport networks.

NEXT CHANGE

## x.2 gNB Functionality

The TRP (Transmission and Reception Point) is responsible for transmitting and/or receiving sensing signals. From the point of view of logical architecture:

- A TRP is part of the gNB, hence it is not visible outside the gNB.

- A gNB may have one or more TRPs.

- For a split gNB, the TRP is part of the gNB-DU; a gNB-DU may have more than one TRP.

Sensing results will be generated by appropriately processing one or more sensing measurements, as collected by TRP(s) which are part of the gNB.

Sensing results to be reported may include e.g. (x,y,z) object location, 3D velocity, range, and possibly others. For tracking purposes, the sensing results may be expected to be reported periodically.

Following the considerations made in Sec. x.1, including e.g. those on backhaul requirements and amount of data to be transferred from RAN to CN, it seems critical that processing is done in the gNB, to generate the sensing results to be reported. The alternative, i.e. to send pre-processed results (e.g. doppler, AoA, delay measurements) to SF, will increase processing load on CN, will be much less “backhaul-friendly” and might require additional assistance data to be sent to CN for final processing.

Split gNB architecture is not considered for NG-RAN ISAC.

END OF CHANGES

The term “passive target” here means that the target object is not connected to or served by the 3GPP network, e.g. it is not a UE. For this reason, it does not seem appropriate to refer to e.g. “UAV detection” in subsequent discussions, as a UAV points to an aerial UE. [5]

We chose a plausible name for this logical interface, but any other name will do. [4]