3GPP TSG-RAN WG3 #129-bis																	R3-256923

Prague, Czech Republic, 13-17 October 2025

Title: 	Abnormal conditions for gNB-CU Configuration Update

Source: 	Ericsson, InterDigital, Jio Platforms, FiberCop

Agenda item:	9.2.2

Document Type:	Discussion and Decision

# 1. Introduction

In this contribution we discuss some abnormal conditions for the gNB-CU Configuration Update procedure related to the Rel19 work on AI/ML supported CCO.

# 2. Cancelling a predicted CCO issue

According to the solution agreed in Rel-19 for cancelling a predicted CCO issue, the cells and beams predicted to be affected by the CCO issue should be the same cells and beams for which the predicted CCO issue is being cancelled. In the following we discuss some implications of this choice and related abnormal conditions which should be considered.

When a predicted CCO issue is cancelled, and the gNB-DU receiving the GNB-CU CONFIGURATION UPDATE  serve the predicted affected cells and beams, it shall receive a Predicted CCO issue IE set to “cancel” and a Predicted Affected Cells and Beams IE where the list of cells and beams is exactly the same as the list of cells and beams received in the Predicted Affected Cells and Beams IE when the CCO issue was predicted.

Observation 1: When a predicted CCO issue is cancelled, the DU serving the predicted affected cells and beams for the predicted CCO issue needs to receive a Predicted CCO Assistance Information IE where the Predicted CCO issue is set to “cancel” and the list of cells and beams contained in the Predicted Affected Cells and Beams IE coincides with the list of cells and beams contained in the Predicted Affected Cells and Beams IE received when the CCO issue was predicted.

When a predicted CCO issue is cancelled, and the gNB-DU receiving the GNB-CU CONFIGURATION UPDATE does not serve the predicted affected cells and beams, in addition to receiving a Predicted CCO Assistance Information IE with a Predicted CCO issue IE set to “cancel”, the DU shall also receive the Neighbour Future Coverage Modification Notification IE. Similarly to the previous case, the list of cells and beams in the Predicted Affected Cells and Beams IE shall be the same as the list of cells and beams in the Neighbour Future Coverage Modification Notification IE.

Observation 2: When a predicted CCO issue is cancelled, a DU not serving the predicted affected cells and beams for the predicted CCO issue needs to receive the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE. The Predicted CCO issue shall be set to “cancel” and the list of cells and beams in the Predicted Affected Cells and Beams IE shall be the same as the list of cells and beams in the Neighbour Future Coverage Modification Notification IE.

In the following, we discuss abnormal conditions that can arise when a predicted CCO issue is being cancelled.

## Non-matching information

A first abnormal condition arises when a predicted CCO issue is being cancelled, and the DU serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message including the Predicted CCO Assistance Information IE where the Predicted CCO issue IE is set to “cancel” but the list of cells and beams in the Predicted Affected Cells and Beams IE does not match the list of cells and beams received in a previous Predicted CCO Assistance Information IE. In this case, the DU cannot understand that the predicted CCO issue being cancelled refers to the same cells and beams for which the future coverage was produced. The DU should discard the Predicted Affected Cells and Beams IE, and the cancelling is not executed.

Proposal 1-A: When the DU serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message including the Predicted CCO Assistance Information IE with the Predicted CCO issue IE set to “cancel” but the list of cells and beams is not the same as the list of cells and beams included in the previously received Predicted CCO Assistance Information IE, the DU discards the Predicted CCO Assistance Information IE, and the cancelling is not executed.

Another abnormal condition exists when a predicted CCO issue is cancelled, and a DU not serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message including the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE where the Predicted CCO issue IE is set to “cancel”, but the list of cells and beams in the Predicted Affected Cells and Beams IE does not match the list of cells and beams contained in the Neighbour Future Coverage Modification Notification IE. Since the lists of cells and beams are not the same, the predicted CCO issue cannot be cancelled, and the DU should discard both the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE.

Proposal 1-B: When a DU not serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message including the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE, where the Predicted CCO issue IE is set to “cancel” but the list of cells and beams in the Predicted Affected Cells and Beams IE is not the same as the list of cells and beams contained in the Neighbour Future Coverage Modification Notification IE, the DU discards both the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE, and the cancelling is not executed.

## Missing information

Another abnormal condition arises when a predicted CCO issue is being cancelled, a DU not serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message including a Predicted CCO Assistance Information IE where the Predicted CCO issue IE is set to “cancel”, but the Neighbour Future Coverage Modification Notification IE is not present.

The DU cannot understand that the predicted CCO issue being cancelled refers to the same cells and beams for which the future coverage was produced. The DU shall discard the Predicted CCO Assistance Information IE, and the cancelling is not executed.

Proposal 2-A: When a DU not serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE where the Predicted CCO issue IE is set to “cancel”, but the Neighbour Future Coverage Modification Notification IE is not present, the DU discards the Predicted CCO Assistance Information IE, and the cancelling is not executed.

A further abnormal condition exists when a predicted CCO issue is being cancelled, and a DU not serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message including the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE, and within the Predicted CCO Assistance Information IE the Predicted CCO issue IE is present and set to “cancel”, but the Predicted Affected Cells and Beams IE is not present.

As for the case of Proposal 2-A, the DU cannot understand that the predicted CCO issue being cancelled refers to the same cells and beams for which the future coverage was produced. The DU shall discard both the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE, and the cancelling is not executed.

Proposal 2-B: When a DU not serving the cells and beams initially predicted to be affected by the predicted CCO issue receives a GNB-CU CONFIGURATION UPDATE message which includes the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE, if the Predicted CCO issue IE is present and set to “cancel” but the Predicted Affected Cells and Beams IE is not present, the DU discards the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE, and the cancelling is not executed.

Proposal 3: Agree to the CR for TS 38.473 in R3-256924.

# 3. Conclusion

In this contribution, the following observations and proposals are made:

Observation 1: When a predicted CCO issue is cancelled, the DU serving the predicted affected cells and beams for the predicted CCO issue needs to receive a Predicted CCO Assistance Information IE where the Predicted CCO issue is set to “cancel” and the list of cells and beams contained in the Predicted Affected Cells and Beams IE coincides with the list of cells and beams contained in the Predicted Affected Cells and Beams IE received when the CCO issue was predicted.

Observation 2: When a predicted CCO issue is cancelled, a DU not serving the predicted affected cells and beams for the predicted CCO issue needs to receive the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE. The Predicted CCO issue shall be set to “cancel” and the list of cells and beams in the Predicted Affected Cells and Beams IE shall be the same as the list of cells and beams in the Neighbour Future Coverage Modification Notification IE.

Proposal 1-A: When the DU serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message including the Predicted CCO Assistance Information IE with the Predicted CCO issue IE set to “cancel” but the list of cells and beams is not the same as the list of cells and beams included in the previously received Predicted CCO Assistance Information IE, the DU discards the Predicted CCO Assistance Information IE, and the cancelling is not executed.

Proposal 1-B: When a DU not serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message including the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE, where the Predicted CCO issue IE is set to “cancel” but the list of cells and beams in the Predicted Affected Cells and Beams IE is not the same as the list of cells and beams contained in the Neighbour Future Coverage Modification Notification IE, the DU discards both the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE, and the cancelling is not executed.

Proposal 2-A: When a DU not serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE where the Predicted CCO issue IE is set to “cancel”, but the Neighbour Future Coverage Modification Notification IE is not present, the DU discards the Predicted CCO Assistance Information IE, and the cancelling is not executed.

Proposal 2-B: When a DU not serving the cells and beams initially predicted to be affected by the predicted CCO issue receives a GNB-CU CONFIGURATION UPDATE message which includes the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE, if the Predicted CCO issue IE is present and set to “cancel” but the Predicted Affected Cells and Beams IE is not present, the DU discards the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE, and the cancelling is not executed.

Proposal 3: Agree to the CR for TS 38.473 in R3-256924.