3GPP TSG-RAN WG3 #129bis								R3-256949

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	10.3.1

Source:	China Telecom

Title: 	General Principles and Requirements for RAN-CN interface

Document for:	Discussion

Introduction

As quoted in [1], 6G study represents an opportunity to revisit the foundations of radio system design, free from the constraints of backward compatibility, and thus focusing on a transformative leap in capability, efficiency, and simplicity. With regard to the design of RAN-CN interface, the following objectives need to be studied in 6G SI phase:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In RAN#109 meeting, the requirements for architecture/migration and service for 6G were discussed and captured into TR38.914[2][3]. Based on the initial requirements for 6G network and new service, this contribution aims to discuss the general principles and requirements for RAN-CN interface.

Discussion

In the 5G era, the NG-RAN comprises a set of gNBs and ng-eNBs connected to the 5GC via the NG interface. According to the architectural and migration requirements agreed upon in the RAN plenary meeting, the 6G RAN architecture is expected to support a standalone RAN configuration. Therefore, we propose assuming an NG-like XX direct interface between the 6GR node and the CN during the SI phase. To support ambient IoT in 6G, AIoTF may connect directly or indirectly to the 6G node. Regarding sensing and AI/ML services, 5G-Advanced is studying the corresponding architecture in Release 20. The finalized interface and architectural solutions in Release 20 can be adopted as a baseline to evaluate whether new solutions are required based on 6G service requirements. Similar to the 5G architecture, the 6GR node may comprise a set of 6G nodes with new radio access technologies and gNB nodes with evolved RATs, all connected to the 6G CN via the new NG-like interface. In addition, according to TS 38.401, NG-RAN and 5GC functions are fully decoupled from transport functions. The same principle is expected to apply to the 6G network.

Proposal 1: a NG-like XX direct interface between 6GR node and CN shall be supported. 6GR and 6G CN functions are fully separated from transport functions. Addressing scheme used in 6GR node and 6G CN shall not be tied to the addressing schemes of transport functions. The fact that some 6GR node or 6G CN functions reside in the same equipment as some transport functions does not make the transport functions part of the 6GR node or the 6G CN.

Proposal 2: to take the final solution on interface and architecture on sensing and AI/ML in Rel-20 as baseline and evaluate whether to use a new solution on interface and architecture base on 6G service requirement.

RAN-CN Interface Architecture

The new direct interface shall also be a logical interface, consisting of CP and UP interfaces similar to the NG interface. RAN sharing among multiple operators has been supported and widely deployed in commercial 4G and 5G networks as an effective means to reduce CAPEX and OPEX, and support for RAN sharing has been established as a mandatory requirement for 6G architecture and migration in RAN#109 [2]. In this context, the 6GR node shall support connections to the CN of different operators. A single 6GR node may also establish multiple XX-C logical interfaces toward the 6G CN, with the selection of the 6G-C interface determined by the NAS node selection function.

Proposal 3: this new XX direct interface shall be also a logical interface which consist of CP and UP interface like NG interface. 6GR node shall support to connect to different operator’s CN. There may be multiple XX-C logical interfaces towards the 6G CN from any one 6GR node. The selection of the 6G-C interface is then determined by the NAS Node Selection function.

RAN-CN interface general principles

Based on the above analysis, we think the similar principle for 5G can be also applied. Therefore, the general principles for the specification of the XX interface are as follows:

-	the XX interface is open;

-	the XX interface supports the exchange of signalling information between the 6GR node and 6G CN;

-	from a logical standpoint, the XX is a point-to-point interface between an 6GR node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the 6G node and 6G CN;

-	the XX interface supports control plane and user plane separation;

-	the XX interface separates Radio Network Layer and Transport Network Layer;

-	the XX interface is future proof to fulfil different new requirements and support of new services and new functions;

-	the XX interface is decoupled with the possible NG-RAN deployment variants;

-	the XX Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency.

Proposal 4: the general principles for the specification of the XX interface are as follows:

-	the XX interface is open;

-	the XX interface supports the exchange of signalling information between the 6GR node and 6G CN;

-	from a logical standpoint, the XX is a point-to-point interface between an 6GR node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the 6G node and 6G CN;

-	the XX interface supports control plane and user plane separation;

-	the XX interface separates Radio Network Layer and Transport Network Layer;

-	the XX interface is future proof to fulfil different new requirements and support of new services and new functions;

-	the XX interface is decoupled with the possible NG-RAN deployment variants;

-	the XX Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: a NG-like XX direct interface between 6GR node and CN shall be supported. 6GR and 6G CN functions are fully separated from transport functions. Addressing scheme used in 6GR node and 6G CN shall not be tied to the addressing schemes of transport functions. The fact that some 6GR node or 6G CN functions reside in the same equipment as some transport functions does not make the transport functions part of the 6GR node or the 6G CN.

Proposal 2: to take the final solution on interface and architecture on sensing and AI/ML in Rel-20 as baseline and evaluate whether to use a new solution on interface and architecture base on 6G service requirement.

Proposal 3: this new XX direct interface shall be also a logical interface which consist of CP and UP interface like NG interface. 6GR node shall support to connect to different operator’s CN. There may be multiple XX-C logical interfaces towards the 6G CN from any one 6GR node. The selection of the 6G-C interface is then determined by the NAS Node Selection function.

Proposal 4: the general principles for the specification of the XX interface are as follows:

-	the XX interface is open;

-	the XX interface supports the exchange of signalling information between the 6GR node and 6G CN;

-	from a logical standpoint, the XX is a point-to-point interface between an 6GR node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the 6G node and 6G CN;

-	the XX interface supports control plane and user plane separation;

-	the XX interface separates Radio Network Layer and Transport Network Layer;

-	the XX interface is future proof to fulfil different new requirements and support of new services and new functions;

-	the XX interface is decoupled with the possible NG-RAN deployment variants;

-	the XX Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency.

Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- RP-252870,pCR for 38.914 on requirements for architecture and migration for 6G, NTT DOCOMO, INC. (Moderator)

- RP-252948,pCR for 38.914 on Requirements of new and existing services for 6G,CMCC (moderator)