3GPP TSG-RAN WG3 Meeting #129bis	R3-256952

# Prague, Czech Republic, 13 – 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Femto |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The clause 4.10.2.1 captures the functions for NR Femto node, it is not appropriate to describe the selection of an AMF performed by NR Femto GW from NR Femto GW’s perspective. It is better to describe like Xn handover case, i.e. to allow the NR Femto GW to select an AMF the Femto node should include the necessary information. - In clause 4.10.2.2, the last bullet “In case of NG handover towards an NR Femto node connected to an NR Femto GW” is put in the wrong place, i.e. it should not be under the bullet “In case of an NR Femto node directly connected to an AMF”. - In addition, the first bullet on routing of handover message may be a little misunderstanding that the selected TAI is contained in the HO request message which is to be routed, however, routing of the HO request message is based on the selected TAI contained in the HO required message which received from the source Femto node. - These two bullets can be merged under the same up level bullet. - Besides routing of Downlink RAN Configuration Transfer message, the routing of DOWNLINK RIM INFORMATION TRANSFER message may also need to be considered. - In clause 4.10.2.3, change the “source AMF” to “serving AMF” to keep align with the description in clause 4.10.2.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 4.10.2.1, reword the sentence to allow the NR Femto GW to select an AMF. - In clause 4.10.2.2, rephase the routing of handover message and including the target NR Femto node ID. - In clause 4.10.2.2, add the routing of DOWNLINK RIM INFORMATION TRANSFER message. - In clause 4.10.2.3, change the “source AMF” to “serving AMF”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains mistakes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.2.1, 4.10.2.2, 4.10.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------Start of the First Change---------------

4.10.2	Functional Split

4.10.2.1	NR Femto node

An NR Femto node hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

NOTE 1:	The NR Femto node may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto node shall only connect to a single NR Femto GW at one time:

-	The NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF;

NOTE 2:	NR Femto nodes may be deployed without network planning. An NR Femto node may be moved from one geographical area to another and therefore it may need to connect to a different NR Femto GW depending on its location.

-	The TAC and PLMN ID(s) used by the NR Femto node shall also be supported by the NR Femto GW;

-	At Initial UE Message:

-	Allowing the NR Femto GW to select an AMF at UE attachment:

-	Including the GUAMI, GUAMI Type and requested NSSAI, if received from the UE, to the NR Femto GW in the INITIAL UE MESSAGE message.

-	At Xn handover:

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the serving AMF in the PATH SWITCH REQUEST message;

-	Allowing the AMF to identify the termination of the source UE associated signalling connection:

-	Including the Source AMF UE NGAP ID to the NR Femto GW in the PATH SWITCH REQUEST message.

4.10.2.2	AMF

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	In case of NG handover towards an NR Femto node connected to an NR Femto GW:

-	Routing of HANDOVER REQUEST messages towards the NR Femto GW based on the Selected TAI contained in the HANDOVER REQUIRED message received from the source NR Femto node;

-	Including the Global RAN Node ID of the target NR Femto node in the HANDOVER REQUEST message.

-	Routing of Downlink RAN Configuration Transfer message and DOWNLINK RIM INFORMATION TRANSFER message towards NR Femto GWs based on the Selected TAI contained in these messages.

-	In case of an NR Femto node directly connected to an AMF:

-	At NG Setup, verifying, as defined in TS 33.545 [70], that the identity used by the NR Femto node is valid;

-	At Initial UE Message and NG Handover procedures, verifying, as defined in TS 33.545 [70], that the reported CAG ID list is valid for the indicated cell of the NR Femto node;

-	At NG PWS Restart Indication and PWS Failure Indication, verifying, as defined in TS 33.545 [70], that the indicated cell identity is valid.

4.10.2.3	NR Femto GW

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NGAP messages between the AMF and the NR Femto node serving the UE, applying the following additional functions:

-	Terminating the UE Context Release Request procedure if an explicit GW Context Release Indication is included. In this case, the NR Femto GW releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by another NR Femto node attached to it.

-	At UE context establishment (Initial Context Setup or NG Handover) the NR Femto GW sends to the NR Femto node the serving AMF’s GUAMI as well as the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	At Path Switch, the NR Femto GW sends to the NR Femto node the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	At Initial UE Message and NG Handover procedures, the NR Femto GW verifies, as defined in TS 33.545 [70], that the reported CAG ID list is valid for the indicated cell of the NR Femto node.

-	Terminating non-UE associated NGAP procedures towards the NR Femto node and towards the AMF, applying the following additional functions:

-	At NG Setup, the NR Femto GW verifies, as defined in TS 33.545 [70], that the identity used by the NR Femto node is valid.

-	In case of NG PWS Restart Indication and PWS Failure Indication, the NR Femto GW verifies, as defined in TS 33.545 [70], that the indicated cell identity is valid and replaces the Global gNB ID of the NR Femto node with the NR Femto GW ID before sending the respective message to the AMF.

-	At Overload Start/Stop, the NR Femto GW should provide the NR Femto node with the identities of the affected AMF node(s). The NR Femto node uses the received information to identify the traffic and the AMF to which the overload indication applies. The NR Femto node shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto node receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

-	Supporting TAC and PLMN ID used by the NR Femto node.

-	Relaying the PATH SWITCH REQUEST message towards the AMF indicated by the GUAMI of the serving AMF received from the NR Femto node.

A TAI used in a NR Femto GW shall not be reused in another NR Femto GW.

-----------------End of the First Change---------------