3GPP TSG RAN3 Meeting #129bis	R3-256971

Prague, Czech Republic, 13 – 17 October 2025

Agenda item: 	    10.4

Source: 	China Unicom

Title: 	Considerations on 6G RAN internal functional split and interfaces

Document for:	Discussion and Decision

- Introduction

At RAN#108 meeting, the Study on 6G Radio has been approved [1]. Following the schedule of the study item, this contribution provides our views on 6G RAN internal functional split and interfaces.

- Discussion on the 6G RAN split architecture

  - Background

The discussion on the 6G RAN split architecture within RAN network is mainly focusing on the whether to support the legacy 5G split architecture as defined since Rel-15. Another issue is a long discussion on the split work with fronthaul interface. According to the conclusions of RAN#109 meeting, there is an endorsement of the way forward in [2]. The text description of support the logical node is left for normative work. Therefore, we should focus on the other issue related to 6G RAN split architecture.

- Whether to support interface between 6G gNodeB and 5G gNB, e.g. similar as Xn interface.

- Whether to support CU DU split architecture as 5G, e.g. interface similar as F1 interface.

- Whether to support CP UP split architecture as 5G, e.g. interface similar as E1 interface.

- Whether to support other interface, e.g. direct interface between the logical Radio Unit (RU).

Therefore, the general considerations on RAN split architecture design are analysis and summarized for discussion. Firstly, RAN3 needs to focus on the critical requirements of 6G network architecture as well as considering 6G typical deployment scenarios.

  - Whether to support direct interface between 6G gNodeB and 5G gNB

The requirements of supporting direct interface between 6G gNodeB and 5G gNB in figure 1, and direct interface between 6G gNodeBs are considering the maturity of 5G commercial deployment and 5G commercial features. In 5G commercial networks, the mobility mechanism and important features including energy saving and inter-cell coordination are relying on the direct interface between gNodeBs. The benefits of supporting this direct interface aims to reduce the latency and improve the system performance with the assistance of the direct interface. Therefore, the direct interface similar as the Xn interface in 5G is mandatory for 6G. While the functionality of this direct interface needs further discussion and avoid the over engineering designs as 5G. For the 4G and 6G interworking scenario, it is proposed to deprioritize this case to minimize the impact of the legacy 4G NodeB shown in figure 2.

Observation 1: The direct interfaces between 6G gNodeB and 5G gNB are mandatory requirements. The functionality design of this interface should learn the lessons from 5G and avoid the over engineering design as 5G Xn interface.

Proposal 1: RAN3 confirms to support the direct interfaces between 6G gNodeB and 5G gNB similar as Xn interface in 5G.

Figure 1: direct interface between 6G NodeB and 5G gNB

Figure 2: NO direct interface between 6G NodeB and 4G NodeB

  - Whether to support CU DU split architecture

For 6G, one of the principles of 6G system design is to guarantee the openness of RAN network architecture and 6G network architecture shall inherit the design as 5G RAN network. Although the commercial progress of F1 and E1 interface in 5G is impacting a very small scale of market, 3GPP RAN working groups define and support the open interface and move forward further than 3G and 4G. It leads to more specification work as the increment of workload if vendors supporting 5G new features via the F1 interface bring with more complexity of the specifications. For 6G new features, the split architecture between CU and DU introduces the extra latency and the new messages are needed to exchange between CU and DU shown in figure 3. Although the drawbacks of CU and DU split architecture are more than the benefits according to 5G commercial experience, the integrated 5G base station in commercial networks are not mandatory for implementation of F1 interface. Considering the new service with AI and sensing, 6G base stations are designed to be more powerful than 5G base stations. CU is the one of the candidate entities to support AI and sensing functionalities sharing with multiple DUs. The specifications need to support CU and DU split architecture while the implementation of this new interface is relying on commercial requirements from operators. In addition, the simplicity of the interface than 5G F1 interface shall be considered if RAN3 confirms to support this split architecture.

Figure 3 direct interface between CU and DU

Proposal 2: RAN3 confirms to support CU and DU split architecture as 5G high layer split architecture, and the simplicity of this interface should be considered during the discussion of functionalities.

  - Whether to support CP UP split architecture

The split of CP and UP in 5G is the new design than 4G, and it provides a flexible solution for commercial deployment especially for vertical deployment scenarios. The design of 5G UP functionalities is very simple than F1 interface. Considering the new service of 6G, the legacy functionalities such as trace, load balancing are necessary to support in the future. The example of split architecture of CP and UP is shown in figure 4.

Figure 4: direct interface between CP and UP

Proposal 3: RAN3 confirms to support CP and UP split architecture similar as 5G E1 interface.

  - Whether to support some other new interface in RAN

In RAN#109 meeting, it is endorsed a way forward of 6G fronthaul interface in [2].

- 6G NodeB architecture includes a distinct logical Radio Unit (RU).

- The functions hosted by the RU as well as the interface between RU and other parts of 6G NodeB are not specified in 3GPP.

RAN3 follow RAN plenary conclusion and capture the endorsed text in normative phase. There is no need to further discuss on fronthaul interface in RAN3. Furthermore, as the lower-lay split is part of fronthaul topic, there is no need to discuss whether to support other direct interface between logical radio unit in RAN3.

Proposal 4: RAN3 confirms the other interface outside CU and DU is not supported in 6G study.

- Conclusion

The observations and proposals of this contribution are as follows.

Observation 1: The direct interfaces between 6G gNodeB and 5G gNB are mandatory requirements. The functionality design of this interface should learn the lessons from 5G and avoid the over engineering design as 5G Xn interface.

Proposal 1: RAN3 confirms to support the direct interfaces between 6G gNodeB and 5G gNB similar as Xn interface in 5G.

Proposal 2: RAN3 confirms to support CU and DU split architecture as 5G high layer split architecture, and the simplicity of this interface should be considered during the discussion of functionalities.

Proposal 3: RAN3 confirms to support CP and UP split architecture similar as 5G E1 interface.

Proposal 4: RAN3 confirms the other interface outside CU and DU is not supported in 6G study.

- References

- RP‑251881 New SID: Study on 6G Radio 	NTT DOCOMO

- RP-252881 Way Forward on 6G Fronthaul Nokia