3GPP TSG-RAN WG3 Meeting #129bis	R3-256981

# Prague, Czech Republic, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0180 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE Performance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, FiberCop, BT, Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the context of the Rel-19 “Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” RAN3 introduced in TS 37.483 a set of metrics representing the UE performance that could be requested by the gNB-CU-CP to be reported by the gNB-CU-UP via the Data Collection Reporting (Initiation) procedures after an AI/ML-driven mobility decision. However, there is no clear understanding on how a gNB-CU-UP could derive UE performance metrics, i.e., Average Packet Loss DL/UL metrics. Following the LS sent by the RAN3 in R3-253816, SA5 provided the reply LS in R3-256518 (S5-253854) and confirmed that it is feasible to specify in TS 28.558 the packet loss/drop measurements with a per-DRB per-UE granularity, performed at PDCP layer. This CR clarifies in the semantics description of Average Packet Loss DL and Average Packet Loss UL IEs within the UE Performance IE that the corresponding measurements refer to TS 28.558 clauses 6.3.1.6.2.1 and 6.3.1.3.1, respectively. The CR also corrects the name of the measurement in TS 28.558 used as a reference for the semantics description of the Average Packet Delay UL and Average Packet Delay DL IEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The semantics description of Average Packet Loss DL and Average Packet Loss UL IEs within the UE Performance IE is updated so that the corresponding measurements refer to TS 28.558 clauses 6.3.1.6.2.1 and 6.3.1.3.1, respectively. The semantics descriptions of the Average Packet Delay UL and Average Packet Delay DL IEs are improved to include the correct name of the measurement in TS 28.558 used as a reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear how Average Packet Loss DL/UL and Average Packet Delay DL/UL are derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.149, 9.3.1.150 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.149	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.3.1.150 |  |
| Average Packet Loss DL | O |  | INTEGER(0..1000000, …) | Corresponds to DL PDCP SDU Drop Rate in gNB-CU-UP per-DRB per-UE as specified in TS 28.558 [37], clause 6.3.1.6.2.1. |
| Average Packet Loss UL | O |  | INTEGER(0..1000000, …) | Corresponds to UL PDCP SDU Loss Rate per-DRB per-UE as specified in TS 28.558 [37], clause 6.3.1.3.1. |

#### 9.3.1.150	Average Packet Delay

This IE indicates the RAN part of the average packet delay in the UL and DL directions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay UL | M |  | INTEGER (0.. 10000) | Corresponds to Packet delay per-DRB per-UE as specified in TS 28.558 [37] clause 6.3.1.1. Unit: 0.1 millisecond |
| Average Packet Delay DL | M |  | INTEGER (0.. 10000) | Corresponds to Packet delay per-DRB per-UE as specified in TS 28.558 [37] clause 6.3.1.1. Unit: 0.1 millisecond |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>