3GPP TSG-RAN WG3 Meeting #129bis	R3-256986

# Prague, Czech Republic, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1570 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on AI/ML-based CCO use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the context of the Rel-19 “Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” RAN3 discussed the AI/ML-based Coverage and Capacity Optimization (CCO) use case, and enhanced the NG-RAN Node Configuration Update procedure to support such use case which, in the context of NG-RAN, is applicable to the gNB (i.e., NR RAT) only. This CR corrects the ID type of the cells listed in the Future Coverage Modification List IE included in NG-RAN NODE CONFIGURATION UPDATE message, which considers only NR cells; corresponding ASN.1 changes are also provided. Moreover, the codepoint ‘cancel’ was also introduced in the Predicted Coverage Modification Cause IE of the Future Coverage Modification List IE included in the NG-RAN NODE CONFIGURATION UPDATE message. This codepoint, signalled together with cells and beams listed in the Future Coverage Modification List IE, is interpreted at the neighbor NG-RAN node side as a notification that the sending NG-RAN node has cancelled all the future coverage modifications associated to those cells and beams. However, it remains unclear what is the neighbor NG-RAN node behaviour when receiving also the Future Cell Coverage State IE and the Future SSB Coverage State IE (which are mandatorily sent) but the cancel codepoint is also received. This CR proposes to allow the neighbor NG-RAN node to ignore the Future Cell Coverage State IE and the Future SSB Coverage State IE when received in the cancellation notification provided by the sending NG-RAN node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the following corrections pertaining the AI/ML-based CCO use case: - ID type of the cells listed in the Future Coverage Modification List IE included in NG-RAN NODE CONFIGURATION UPDATE message needs to be related to NR cells only; - ASN.1 changes corresponding to the above. - the sematics descriptions of the Future Cell Coverage State IE and the Future SSB Coverage State IE clarify that they will be ignored by the neighbor NG-RAN node after they are received within the cancellation notification of neighbor future coverage modifications provided by the sending NG-RAN node. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification support for the AI/ML-based CCO use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2, 9.1.3.4, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

*** skip unmodified parts ***

Update of Future Cell Coverage:

If the Future Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to identify the future cell deployment configuration to be enabled by the NG-RAN node1 at a time indicated by Time for Future Coverage State IE and for configuring mobility towards the cell(s) indicated by the NR CGI IE, as described in TS 38.300 [9].

If the Future SSB Coverage Modification List IE is present in the Future Coverage Modification List IE, the NG-RAN node2 may use the information to identify the future SSB beam deployment configuration to be enabled by the NG-RAN node1 at a time indicated by Time for Future Coverage State IE and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

*** skip unmodified parts ***

<<<<<<<<<< Next Change >>>>>>>>>>

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| *** skip unmodified parts *** |  |  |  |  |  |  |
| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified. | YES | ignore |
| >Future Coverage Modification Item |  | 1..<maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Identifier of the NR cell whose coverage will be modified. | – |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicates the future coverage configuration of the concerned cell. The IE is ignored if the Predicted Coverage Modification Cause is set to “cancel” | – |  |
| >>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams whose coverage will be modified. | – |  |
| >>>Future SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB index | M |  | INTEGER (0..63) | Identifier of the SSB beam whose coverage will be modified. | – |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also Indicates the future coverage configuration of the concerned SSB beams. The IE is ignored if the Predicted Coverage Modification Cause is set to “cancel” | – |  |
| >>Predicted Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, cancel, ...) | Indicates the reason for the predicted coverage modification in NG-RAN node1, or that a previously sent Future Coverage Modification List is cancelled. | – |  |
| >>Time for Future Coverage State | O |  | INTEGER (1..60, …) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information, in seconds. This IE is ignored if the Predicted Coverage Modification Cause is set to “cancel”. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

<<<<<<<<<< Next Change >>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

*** skip unmodified parts ***

#### -- F

*** skip unmodified parts ***

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

globalNG-RANCell-ID						NR-CGI,

future-cellCoverageState				INTEGER (0..63, ...),

future-SSB-Coverage-Modification-List	Future-SSB-Coverage-Modification-List		OPTIONAL,

predicted-Coverage-Modification-Cause	Predicted-CoverageModificationCause			OPTIONAL,

timeForFutureCoverageState				INTEGER (1..60, ...)						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<< End of Changes >>>>>>>>>>