3GPP TSG-RAN WG3 Meeting #129-bis	R3-256990

Prague, Czech Republic, 13th – 17th October, 2025

Title:	(TP for 6G TR) Considerations on AI/ML use cases for 6G RAN

Agenda Item:	10.5.1

Source:	Huawei

Document for:	Discussion and decision

# 1	Introduction

- In the 6G SID [1], there is the following objective pertaining AI/ML for 6GR and RAN:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

- This paper provides initial considerations on AI/ML use cases of interest in 6G.

# 2	Discussion

- In this paper, we will start with the status quo in 5G-A about what has been specified for AI/ML use cases in RAN3, based on which we will try to further discuss whether any new requirements for new potential use cases are worth to consider in 6G.

## 2.1	Current status

TS 38.300 [2], clause 16.20.2, lists the general principles which drove the specification of AI/ML for NG-RAN in both Rel-18 and Rel-19 5G-A:

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

AI/ML algorithms and models are out of 3GPP scope. Model-specific performance information, e.g. model performance indicators specified in clause 6 of TS 28.105 [64], is not exchanged over NG-RAN interfaces in TS 38.401 [4].

Support of AI/ML for NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-		AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-		AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [64]. An AI/ML Model needs to be trained, validated and tested before deployment for AI/ML Model Inference.

AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [64].

The following NG-RAN AI/ML use cases were introduced in 5G-A in Rel-18 and Rel-19, as listed in the corresponding WIDs in [3] and [4], respectively:

Rel-18: Network Energy Saving, Load Balancing, Mobility Optimization

Rel-19: Coverage and Capacity Optimization, Network Slicing

Moreover, as part of the Rel-19 WI on AI/ML for NR air interface [5], support for the AI/ML-based positioning Case 3a (AI/ML model located in the gNB) was specified by RAN3, and such use case could be seen as another RAN AI/ML use case since it leverages an AI/ML model which is hosted in NG-RAN.

Based on the above, and considering the statement in the 6G SID in that AI/ML for 6GR and Radio Access Network should leverage the 5G AI/ML framework (as appropriate), it is our understanding that RAN3 should start discussing the benefits deriving from new AI/ML use cases not previously addressed, which may be considered for optimizing further the network performance, as well as enabling new applications and tools. Of course, as clearly indicated in the SID, the 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML, and this shall be kept as guidance during the whole study.

Proposal 1: RAN3 to study the benefits of new AI/ML use cases not addressed in previous 3GPP releases.

## 2.2	Consideration on new use cases

In this section we investigate a potential new use case which we think is worth to be specified in 6G, that is, RAN Digital Twin (DT).

### 2.2.1 RAN Digital Twin (DT)

According to Recommendation ITU-R M.2160, IMT-2030 technology is expected to be used to replicate the physical world into a digital virtual world as precise real-time representations or Digital Twin (DT). In RAN network, DT is a virtual replica of the physical RAN. It can reflect the real time physical world and provide the environmental information of the physical RAN, such as radio frequency information (e.g., multi-path information) and environmental surroundings (e.g., buildings and vehicles, etc.). These information output by the RAN DT may not only serve as prior information to improve communication performance, but also have the potential to assist other services which require environmental information. For example, once the DT of a physical environment is available, the wireless propagation characteristics in the physical environment can be easily predicted. This can be used to assist in improving performance, e.g., improving energy consumption, handover successful rate, MIMO operation with reduced overhead, etc.

The RAN DT can be constructed through AI/ML methods. The required data for AI/ML model, i.e., environmental information, could be acquired through different data collection entities, e.g., base stations (BSs), UEs, then distributed learning and inference are adopted to improve the precision and completeness of the environmental information. Since raw data between UE and BS or among BSs may be of large size and could be subject to privacy restrictions, transmission of AI/ML assisted information for distributed learning and inference is sometimes more desirable in terms of overhead reduction and privacy protection.

In distributed learning, models are trained based on local data on each network entity and information gathered from other entities, e.g., other BSs and/or other UEs. Continuous learning may be needed to overcome data distribution drift over time, which may also require information exchange among different entities. Hence, procedures for information exchange among multiple entities should be studied. For example, the information exchange among different BSs should be considered to exploit BS’s AI capability or data and construct a large-scale RAN DT, achieving better performance than single BS’s DT.

In addition, collaboration with other 3GPP WGs may also be needed to understand which type of data the AI/ML model may need as input and which is the expected output, as well as the type of data that would need to be made available as feedback information.

Proposal 2: For new use cases, RAN3 to study AI/ML-enabled RAN Digital Twin for leveraging environmental data, e.g., to aggregate local environmental data reported from neighboring BSs.

Corresponding TP for the 6G TR could be seen in the Annex.

# 3	Conclusion

This document proposes:

Proposal 1: RAN3 to study the benefits of new AI/ML use cases not addressed in previous 3GPP releases.

Proposal 2: For new use cases, RAN3 to study AI/ML-enabled RAN Digital Twin for leveraging environmental data, e.g., to aggregate local environmental data reported from neighboring BSs.

# 4	Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator), RAN#108

- 3GPP TS 38.300, NR; NR and NG-RAN Overall description; Stage-2, Rel-18, V18.7.0 (2025-09)

- RP-233441, Revised WID on Artificial Intelligence (AI)Machine Learning (ML) for NG-RAN, CMCC, Ericsson, RAN#102

- RP-251245, Revised WID Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC, RAN#108

- RP-251186, Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Qualcomm, RAN#102

# Annex	- Text Proposal for 6G TR (based on R3-255517)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 7	AI/ML for RAN

## 7.1 	AI/ML use cases

AI/ML in RAN was initially introduced in Rel-18 5G-Advanced by focusing on Network Energy Saving, Load Balancing, Mobility Optimization use cases. Then, in Rel-19 5G-Advanced, two new use cases have been considered, that is, Coverage and Capacity Optimization and Network Slicing.

In 6G new AI/ML use cases have been studied and described in the following subsections.

### 7.1.1 RAN Digital Twin

The IMT-2030 technology is expected to be used to replicate the physical world into a digital virtual world as precise real-time representations or Digital Twin (DT). In RAN network, DT is a virtual replica of the physical RAN which can reflect the real time physical world and provide the environmental information of the physical RAN, such as radio frequency information (e.g., multi-path information) and environmental surroundings (e.g., buildings and vehicles, etc.). These information output by the RAN DT may not only serve as prior information to improve communication performance, but also have the potential to assist other services which require environmental information. For example, once the DT of a physical environment is available, the wireless propagation characteristics in the physical environment can be easily predicted. This can be used to assist in improving performance, e.g., improving energy consumption, handover successful rate, MIMO operation with reduced overhead, etc.

The RAN DT can be constructed through AI/ML methods. The required data for AI/ML model, i.e., environmental information, could be acquired through different data collection entities, e.g., base stations (BSs), UEs, then distributed learning and inference are adopted to improve the precision and completeness of the environmental information. Since raw data between UE and BS or among BSs may be of large size and could be subject to privacy restrictions, transmission of AI/ML assisted information for distributed learning and inference is sometimes more desirable in temrs of overhead reduction and privacy protection.

In distributed learning, models are trained based on local data available at each network entity and information gathered from other entities (e.g., other BSs and/or other UEs). Continuous learning may be needed to overcome data distribution drift over time, which may also require information exchange among different entities.

Procedures for information exchange among multiple entities may be needed, and the information exchange among different BSs should be considered to exploit BS’s AI capability or data and construct a large-scale RAN DT, achieving better performance than single BS’s DT.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>