3GPP TSG-RAN WG3 Meeting #129bis	R3-256991

Prague, Czech Republic, 13th – 17th October, 2025

Title: 	(TP for TR 38.745) Considerations on multi-hop UE trajectory in AI/ML-based mobility

Source: 	Huawei

Agenda item:	12.2.1

Document Type:	Discussion and Decision

# 1. Introduction

A new SI “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” was approved for Rel-20 5G-Advanced (5G-A) at RAN #108 meeting [1], then revised in RAN#109 and eventually approved in [2], aiming at investigating mobility scenarios that could benefit from using AI/ML techniques.

The objective of the new SI is excerpted from [2] and reported below for convenience:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

This contribution focuses in particular on the “Multi-hop UE trajectory across gNBs” scenario, providing our considerations and proposals for discussion.

# 2. Discussion

## 2.1 Multi-hop UE trajectory across gNBs

The inference-based UE mobility enhancement specified in Rel-18 focused on optimizing UE handovers across cells within the 1st hop target gNB. For example, the AI/ML model in the source gNB predicts the cell-based UE trajectory (i.e., the list of target cells where the UE will likely be handed over, with optional time of stay in each of these cells) and provides this prediction to the next-hop gNB, hence optimizing the handover resources in advance as well as reducing UE handover delay and failure probability.

Back then in Rel-18, RAN3 did not have time to address in depth the multi-hop UE trajectory prediction scenario, i.e., a prediction of the UE trajectory spanning multiple gNBs. The topic was also re-discussed at the time of the Rel-19 SI “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [3] but no conclusion was reached and, in RAN3#124 meeting, it was decided that the multi-hop UE trajectory is not supported in Rel-19 – the following was captured in the Chair’s meeting minutes [4]:

No consensus on the solution the multiple-hop UE trajectory. Multiple-hop UE trajectory is not supported in R19.

However, it is well acknowledged in RAN3 that the multi-hop UE trajectory prediction may bring some benefits for the 1st hop target gNB as well as for the gNB in each hop to, e.g., predict the incoming/outgoing cell traffic in a future period of time, understand the UE multi-hop destination, etc.

Observation 1: Multi-hop UE trajectory may be useful for mobility-related decisions of the 1st hop target gNB as well as intermediate gNBs.

In our view, the multi-hop UE trajectory use case can be seen as an extension of the Mobility Optimization use case specified in Rel-18. As a consequence, we see no reasons to deviate from the framework already established back then in terms of AI/ML Model Training and AI/ML Model Inference functions’ location. Specifically, we think that the deployment scenarios to be taken into account when discussing the multi-hop UE trajectory use case should be the same as specified in TS 38.300 (for the non-split architecture) [5] and TS 38.401 (for the split architecture) [6], namely:

-	AI/ML Model Training located in OAM, while AI/ML Model Inference located in the gNB(-CU);

-	AI/ML Model Training and AI/ML Model Inference located in the gNB(-CU).

Proposal 1: RAN3 to confirm that, for the multi-hop UE trajectory use case, the deployment scenarios specified in Rel-18 in terms of location of AI/ML Model Training and AI/ML Model Inference functions are kept valid.

Based on the above, we provide an analysis of possible solutions to enable the transmission of the multi-hop UE trajectory prediction across multiple gNBs as well as multi-hop UE trajectory feedback collection and reporting.

As agreed in Rel-18, the source gNB can send the cell-based UE trajectory prediction in the HANDOVER REQUEST message. Moreover, the source gNB can configure the collection and one-time reporting of the measured UE trajectory by the target gNB in the DATA COLLECTION REQUEST message, and the collection configuration comprises the collection time duration and number of visited cells. At successful handover, the target gNB starts collecting information related to the measured UE trajectory (i.e., list of visited cells and corresponding time UE stayed in each cell) and it shall terminate the collection of the UE trajectory information either at expiration of the collection time duration previously configured by the source gNB, or the UE moves to inactive or idle state, or the UE is handed over to a cell belonging to a different gNB. Then, the target gNB can send the measured UE trajectory in the DATA COLLECTION UPDATE message back to the source gNB.

In order to enable the multi-hop UE trajectory prediction to span across multiple gNBs, we think that the multi-hop UE trajectory prediction could also be transferred in the HANDOVER REQUEST message from the source gNB to target gNB through multiple intermediate gNBs. One direct enhancement is that the XnAP’s Cell Based UE Trajectory Prediction IE should be enhanced to contain the identifiers of the multiple gNBs whose cells are predicted to be visited by the UE. In addition, the intermediate gNB may not need to transfer the list of cells previously predicted to be visited (or include the whole UE trajectory prediction from the source gNB) to the next-hop gNB, since we do not see any benefits for the next-hop gNB to have this information.

Proposal 2: RAN3 to agree to transfer the multi-hop UE trajectory prediction predicted by the source gNB(-CU) to the 1st hop target gNB following the design of the single-hop UE trajectory prediction transmission in Rel-18 (i.e., in the HANDOVER REQUEST message).

Proposal 3: RAN3 to discuss whether either the whole UE trajectory prediction as received from the source gNB(-CU) is transferred from the 1st hop target gNB to the subsequent (intermediate) gNBs, or that each intermediate gNB provides just a subset of the cells predicted to be visited (i.e., it removes its own cells from the list or predicted cells).

Since the multi-hop UE trajectory prediction could be transferred across multiple gNBs, we need to specify how to transfer the measured multi-hop UE trajectory. Assume for instance that the multi-hop UE trajectory prediction spans three gNBs: Node1 (initial source node), Node2 (intermediate node) and Node3 (final node). Since the initial source gNB (Node1) uses AI/ML tools to predict the multi-hop UE trajectory, it may be beneficial for the initial source gNB to obtain the measured multi-hop UE trajectory as a feedback information to further monitor the model’s performance and decide whether to re-train the AI/ML model.

Figure 1 - Multi-hop UE trajectory prediction spanning three gNBs.

Observation 2: It may be beneficial for the initial source gNB that predicted the multi-hop UE trajectory to obtain the measured multi-hop UE trajectory as a feedback information to further monitor the model’s performance and decide whether to re-train the AI/ML model.

However, collecting the measured UE trajectory across multiple gNBs to eventually be provided to the source gNB is challenging from a complexity point of view due to a number of reasons:

- the need to keep the UE context in the source gNB after the UE has been handed over to subsequent hop target gNBs, so that the source gNB is able to then associate the received UE trajectory as feedback information;

- the potential lack of the Xn interface between the source gNB and “far away” target gNB(s). To be specific, if a trajectory prediction covers the N future cell hops, it is very likely that the gNB serving the nth cell will not be Xn-connected to the source gNB that produced the prediction. Hence, even if the source gNB kept the UE context stored, there would be likely no means for the nth target gNB to provide the actual UE trajectory feedback back;

- cell deployment changes (e.g. cells’ activation and deactivation) during the time interval between the UE trajectory prediction is provided by the source gNB and the measured UE trajectory is received at the source gNB.

Observation 3: There are technical challenges preventing the source gNB to be provided with the measured multi-hop UE trajectory feedback.

From our point of view, there are four possible solutions for the collection and reporting of the measured multi-hop UE trajectory:

Option1: Parallel transmission from each hop node to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1). Node1 initiates a Data Collection Reporting Initiation procedure towards Node2 which includes collection time duration and number of visited cells covering multiple gNBs based on the UE trajectory prediction. Each next-hop target gNB transfers the locally collected measured UE trajectory directly back to the initial source gNB, hence Node2 sends feedback information to Node1 and Node3 sends feedback information to Node1. In this way, how does the concerned next-hop target gNB (Node3) know which is the initial source gNB (Node1), and the UE trajectory collection and reporting configuration to use? One possible way is that intermediate gNB (Node2) sends the initial source gNB ID (Node1 ID) and an update of the measured UE trajectory collection and reporting configuration previously sent to Node2 to the next-hop target gNB (Node3), e.g., in the HANDOVER REQUEST message. The updated measured UE trajectory collection and reporting configuration sent by Node2 to Node3 mentioned above means that the Node2 needs to revise the measured UE trajectory collection and reporting configuration initially provided by Node1 to Node2 based on the actual UE path and behaviour within Node2. Once each gNB provided its own portion of the measured UE trajectory to Node1 it will be up to Node1 to merge all the received portions to derive the complete measured multi-hop UE trajectory and use it for, e.g., model performance monitoring. The main drawbacks of this solution are 1) for how long the Node1 should keep the UE context stored after UE handover to Node 2, and 2) the potential unavailability of the Xn interface between Node1 and a “far away” final target gNB (e.g., Node3).

Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1). Node1 initiates a Data Collection Reporting Initiation procedure for Node2, and Node2 initiates another Data Collection Reporting Initiation procedure for Node3, and in each procedure the measured UE trajectory collection (and reporting) configuration is provided. Each next-hop target gNB sends the locally collected measured UE trajectory up to the initial source gNB via the intermediate gNBs, which may require enhancements of the measured single-hop UE trajectory feedback mechanism of Rel-18 because it is now expected that each intermediate node (Node2) is able to keep the UE context for a sufficient amount of time to ensure that, once it has received the portion of measured UE trajectory from the subsequent node (Node3) and has merged such contribution with its own portion of measured UE trajectory, it will be possible to propagate the measured UE trajectory back to Node1. Or, alternatively, each intermediate node (e.g., Node3) sends the locally collected measured UE trajectory to the previous node (Node2) immediately after the UE is handed over to another subsequent gNB (Node4, not shown in Figure 1), and then Node2 transparently forwards the measured UE trajectory collected by and received from Node3, hence leaving to Node1 the responsibility to merge all the received portions of measured UE trajectory to derive the complete measured multi-hop UE trajectory. Anyway, we think that the main drawback of this solution is mainly for how long each node should keep the UE context stored in order to enable hop-by-hop propagation of the measured UE trajectory portions along the chain.

Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1). Node1 initiates a Data Collection Reporting Initiation procedure for Node2, and Node2 initiates another Data Collection Reporting Initiation procedure for Node3, and in each procedure the measured UE trajectory collection (and reporting) configuration is provided. Each intermediate gNB (Node2) transfers its own (locally collected) portion of the measured UE trajectory to the next-hop target gNB (Node3) via, e.g., the HANDOVER REQUEST message. Then the final gNB (Node3) collects its own (locally collected) portion of the measured UE trajectory, it merges its own portion with the measured UE trajectory previously collected by and received from Node2 and sends the complete measured multi-hop UE trajectory to the initial source gNB (Node1). Even though the legacy measured UE trajectory collection configuration may be enough to support this solution, each subsequent target gNB needs to acquire the initial source gNB ID (Node1 ID). In addition, we believe that the necessity of this solution is worth discussing, as we think that the main drawback for the transmission of locally collected measured UE trajectory information over the Xn interface is the significant Xn signalling overhead, in addition to the ones already observed for Option1.

Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1). Node1 initiates a Data Collection Reporting Initiation procedure for Node2 to provide the measured UE trajectory collection configuration, and Node2 initiates another Data Collection Reporting Initiation procedure for Node3 to provide the measured UE trajectory collection configuration as well as the initial source gNB ID (Node1 ID). At least except the first target gNB, i.e., Node2, which can send its own portion (locally collected) of the measured UE trajectory back to Node1 directly, each next-hop target gNB sends its own portion (locally collected) of the measured UE trajectory to the AMF, then either the AMF merges each portion of the measured UE trajectory provided by each next-hop target gNB and forwards the merged measured UE trajectory to Node1 which, in turn, merges what it received from the AMF with the portion of the measured UE trajectory collected locally and previously received by Node2 (the first next-hop target gNB), or the AMF separately forwards each and every portion of the measured UE trajectory received from each next-hop target gNB to Node1 sequentially, and it is up to Node1 to merge all the received portions of measured UE trajectory to derive the complete measured multi-hop UE trajectory. In this manner, we need to propagate the initial source gNB ID (Node1 ID) over the Xn interface towards each next-hop target gNB, and ensure that the initial source gNB and each next-hop target gNB can maintain the connection to the same AMF. In our view, involving the AMF could be applicable also to Option1 and Option3 to at least solve the issue of potential unavailability of the Xn interface between Node1 and a “far away” final target NG-RAN gNB (e.g., Node3), at a cost of impacting the NG interface which was ruled out in previous releases.

Proposal 4: RAN3 to down-select one option for the transfer of the measured multi-hop UE trajectory feedback among the following:

- Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

- Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

- Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

- Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

Proposal 5: RAN3 to adopt Option2 (hop-by-hop transmission) as the solution to enable the propagation of the multi-hop UE trajectory prediction as well as the transfer of the measured multi-hop UE trajectory back to the source gNB, which ensures minimum standard impact.

# 3. Conclusion

Based on the above discussion, we have the following observations and proposals:

Observation 1: Multi-hop UE trajectory may be useful for mobility-related decisions of the 1st hop target gNB as well as intermediate gNBs.

Proposal 1: RAN3 to confirm that, for the multi-hop UE trajectory use case, the deployment scenarios specified in Rel-18 in terms of location of AI/ML Model Training and AI/ML Model Inference functions are kept valid.

Proposal 2: RAN3 to agree to transfer the multi-hop UE trajectory prediction predicted by the source gNB(-CU) to the 1st hop target gNB following the design of the single-hop UE trajectory prediction transmission in Rel-18 (i.e., in the HANDOVER REQUEST message).

Proposal 3: RAN3 to discuss whether either the whole UE trajectory prediction as received from the source gNB(-CU) is transferred from the 1st hop target gNB to the subsequent (intermediate) gNBs, or that each intermediate gNB provides just a subset of the cells predicted to be visited (i.e., it removes its own cells from the list or predicted cells).

Observation 2: It may be beneficial for the initial source gNB that predicted the multi-hop UE trajectory to obtain the measured multi-hop UE trajectory as a feedback information to further monitor the model’s performance and decide whether to re-train the AI/ML model.

Observation 3: There are technical challenges preventing the source gNB to be provided with the measured multi-hop UE trajectory feedback.

Proposal 4: RAN3 to down-select one option for the transfer of the measured multi-hop UE trajectory feedback among the following:

- Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

- Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

- Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

- Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

Proposal 5: RAN3 to adopt Option2 (hop-by-hop transmission) as the solution to enable the propagation of the multi-hop UE trajectory prediction as well as the transfer of the measured multi-hop UE trajectory back to the source gNB, which ensures minimum standard impact.

# 4. Reference

- RP-251868, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#108

- RP-252867, “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#109

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom.doc)

- 3GPP TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

- 3GPP TS 38.401, NG-RAN; Architecture description

# Annex – TP to TR 38.745

<<<<<<<<<< Start of Changes >>>>>>>>>>

# 1	Scope

The present document provides the description and investigation of new AI/ML based use cases, i.e., multi-hop UE trajectory, intra-CU LTM and other handover enhancements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X1]	3GPP TS 38.300: “NR; NR and NG-RAN Overall description; Stage-2”.

<<<<<<<<<< Next Change >>>>>>>>>>

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

AI/ML based Mobility Optimization is specified in TS 38.300 [X1] and, in Rel-18, it aimed at optimizing UE handovers across cells within the first hop target gNB. Specifically, the source gNB predicts the cell-based UE trajectory and provides this prediction to the next-hop gNB.

The multi-hop UE trajectory prediction – i.e., a UE trajectory encompassing cells of different gNBs – may be beneficial not only for the first hop target gNB but also for the gNB in each hop to, e.g., predict the incoming/outgoing cell traffic in a future period of time, understand the UE multi-hop destination, etc.

Provisioning of the multi-hop UE trajectory could be done by enhancing the existing mechanism introduced for transferring the cell-based UE trajectory prediction, i.e., via the Handover Preparation procedure. However, collecting the measured UE trajectory across multiple gNBs to eventually be provided to the source gNB is challenging from a complexity point of view due to a number of reasons:

- the need to keep the UE context in the source gNB after the UE has been handed over to the subsequent hop target gNBs, so that the source gNB is able to then associate the received measured UE trajectory as feedback information;

- the potential lack of the Xn interface between the source gNB and “far away” target gNB(s): if a trajectory prediction covers the N future cell hops, it is very likely that the gNB serving the nth cell will not be Xn-connected to the source gNB that produced the prediction;

- cell deployment changes (e.g. cells’ activation and deactivation) during the time interval between the UE trajectory prediction is provided by the source gNB and the measured UE trajectory is received at the source gNB.

4.1.2	Solutions and standard impacts

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Input data of multi-hop UE trajectory

The following data is required as input data.

From the local node:

-	Multi-hop measured/predicted UE trajectory

-	Measured/predicted radio resource status

-	Measured/predicted UE traffic

From the neighbouring gNBs:

-	UE’s history information from neighbour

-	QoS parameters and the performance information of historical HO-ed UE (e.g., loss rate, delay, etc.)

-	Measured/predicted radio resource status

-	UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism.

From the UE:

-	UE location information (e.g., serving cell ID, moving velocity) interpreted by gNB implementation when available.

-	Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR.

-	UE Mobility History Information.

#### 4.1.2.3	Output of multi-hop UE trajectory

The following information can be generated as output:

-	Multi-hop UE trajectory prediction (list of cells predicted to be visited, optionally including the predicted time of stay of the UE in each cell)

-	UE traffic prediction in each hop (output used internally by the RAN)

#### 4.1.2.4	Feedback of multi-hop UE trajectory

The following data is required as feedback data:

-	Measured multi-hop UE trajectory

-	per-UE UE performance (Average UE Throughput DL/UL, Average Packet Loss DL/UL, Average Packet Delay DL/UL)

-	Measured radio resource status from target gNBs

#### 4.1.2.5	Standard impact

The following standard impacts are listed for subsequent Rel-20 normative work compared to what was specified in Rel-18.

Xn interface:

-	Enhance the existing Handover Preparation and Data Collection Reporting (Initiation) procedures to transfer the multi-hop UE trajectory prediction and to collect the measured multi-hop UE trajectory, respectively.

<<<<<<<<<< End of Changes >>>>>>>>>>