3GPP TSG-RAN WG3 Meeting #129bis	R3-256992

Prague, Czech Republic, 13th – 17th October, 2025

Title: 	(TP to TR 38.745) Discussion for AI/ML assisted intra-CU LTM

Source: 	Huawei

Agenda item:	12.2.2

Document Type:	Discussion and Decision

# 1. Introduction

A new SI “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” was approved for Rel-20 5G-Advanced (5G-A) at RAN #108 meeting [1], then revised in RAN#109 and eventually approved in [2], aiming at investigating mobility scenarios that could benefit from using AI/ML techniques.

The objective of the new SI is excerpted from [2] and reported below for convenience:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

This contribution focuses in particular on the “intra-CU LTM” scenario, providing our considerations and proposals for discussion.

# 2. Discussion

## 2.1 AI/ML for intra-CU LTM

Intra-CU LTM specified in Rel-18 is designed to enable low-latency cell switching by pre-configuring candidate cells for a UE and allowing the UE to perform early uplink/downlink synchronization with each candidate cell before the actual cell switch is executed. Once the CU decides to configure LTM for a UE, all candidate cells must reserve dedicated resources (e.g., configured grant resources, PRACH resources, CSI-RS) for that UE as soon as the CU instructs the candidate DU(s) to prepare for LTM. These reserved resources remain unavailable for other UEs until the candidate cell is explicitly released. If the UE does not switch to certain candidates, the pre-allocated resources in those cells remain unused, resulting in low resource utilization. In addition, a UE can be configured with up to eight LTM candidate cells, each reserving dedicated resources for that UE. As the UE eventually connects to only one target cell, resource reserved in multiple candidates may remain unused, leading to avoidable resource locking and reduced overall efficiency.

- Observation 1: Pre-configured LTM candidate cells must reserve dedicated resources for a UE until released, and if unused, these locked resources lead to poor overall utilization.

- Observation 2: A UE may be configured with up to eight LTM candidate cells, each reserving dedicated resources, but only one becomes the target, leaving several candidate cells’ resources unused and hence wasting capacity.

- An AI/ML model can be leveraged to determine the appropriate timing for each candidate DU to reserve specific resources for a UE ahead of the corresponding event (e.g., PRACH resources for early uplink synchronization, CG resources after the UE accesses the target cell). By leveraging time predictions for key LTM events such as cell switch or early synchronization, candidate DUs can reserve the necessary resources only when needed, according to implementation policy. Until the reservation is triggered, those resources remain available for other UEs or services, improving overall resource utilization and reducing unnecessary resource locking. In our understanding, and by leveraging the AI/ML for NG-RAN framework established since Rel-18 as specified in TS 38.401 [3], the predicted time information for LTM-specific events can be generated by an AI/ML model hosted in the gNB-CU, trained either in the gNB-CU or in OAM.

- Observation 3: Indicating predicted time information for key LTM events (e.g., early synchronization, cell switch) to candidate DUs enables DUs themselves to reserve LTM resources (e.g., PRACH for early synchronization, CG after target cell access) only when needed, hence minimizing resource waste.

- Proposal 1: RAN3 to agree that CU hosts the AI/ML model which predicts the time information for key LTM events (e.g., early synchronization, cell switch).

- Proposal 1bis: RAN3 to agree that, for intra-CU LTM, the AI/ML Model Inference is located in the gNB-CU and AI/ML Model Training is located either in the gNB-CU or in the OAM.

- Proposal 2: RAN3 to agree to transfer the time information for key LTM events (e.g., early synchronization, cell switch) predicted by the AI/ML model located in the CU to candidate DUs for enabling optimized LTM resources reservation.

- In terms of the predicted time for cell switch, the predicted UE trajectory spanning cells belonging to different DUs but still controlled by the same CU – hence the same type of prediction introduced in Rel-18 as specified in TS 38.300 [4] – could provide information about the subsequent cells where the UE is predicted to connect. In particular, the expected time of stay in each cell predicted to be visited may be used to determine the time information for future subsequent cell switches.

- Proposal 3: RAN3 to agree that the expected time of stay in each cell of the predicted UE trajectory may be used to determine the time information for future subsequent LTM cell switches.

Based on Observation 2, to identify the LTM candidate cells which are more likely to become the actual serving cell in the future, the Rel-18 predicted UE trajectory can indicate the future UE mobility information which can be beneficial for the CU to determine the candidate cells for LTM or subsequent LTM events.

Proposal 4: RAN3 to agree that the list of candidate cells for LTM or subsequent LTM can be determined based on the Rel-18 predicted UE trajectory.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

- Observation 1: Pre-configured LTM candidate cells must reserve dedicated resources for a UE until released, and if unused, these locked resources lead to poor overall utilization.

- Observation 2: A UE may be configured with up to eight LTM candidate cells, each reserving dedicated resources, but only one becomes the target, leaving several candidate cells’ resources unused and hence wasting capacity.

- Observation 3: Indicating predicted time information for key LTM events (e.g., early synchronization, cell switch) to candidate DUs enables DUs themselves to reserve LTM resources (e.g., PRACH for early synchronization, CG after target cell access) only when needed, hence minimizing resource waste.

- Proposal 1: RAN3 to agree that CU hosts the AI/ML model which predicts the time information for key LTM events (e.g., early synchronization, cell switch).

- Proposal 1bis: RAN3 to agree that, for intra-CU LTM, the AI/ML Model Inference is located in the gNB-CU and AI/ML Model Training is located either in the gNB-CU or in the OAM.

- Proposal 2: RAN3 to agree to transfer the time information for key LTM events (e.g., early synchronization, cell switch) predicted by the AI/ML model located in the CU to candidate DUs for enabling optimized LTM resources reservation.

- Proposal 3: RAN3 to agree that the expected time of stay in each cell of the predicted UE trajectory may be used to determine the time information for future subsequent LTM cell switches.

Proposal 4: RAN3 to agree that the list of candidate cells for LTM or subsequent LTM can be determined based on the Rel-18 predicted UE trajectory.

# 4. Reference

- RP-251868, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#108

- RP-252867, “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#109

- 3GPP TS 38.401, NG-RAN; Architecture description

- 3GPP TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

# Annex – TP for TR 38.745

<<<<<<<<<< Start of Changes >>>>>>>>>>

# 1	Scope

The present document provides the description and investigation of new AI/ML based use cases, i.e., multi-hop UE trajectory, intra-CU LTM and other handover enhancements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X1]	3GPP TS 38.300: “NR; NR and NG-RAN Overall description; Stage-2”.

[X2]	3GPP TS 38.401 “NG-RAN; Architecture description”.

<<<<<<<<<< Next Change >>>>>>>>>>

## 4.2	AI/ML assisted Intra-CU LTM

### 4.2.1	Use case description

Support for intra-CU LTM is specified in TS 38.300 [X1] and TS 38.401 [X2].

The objectives of AI/ML assisted intra-CU LTM is to optimize the intra-CU LTM functionality further. Specifically, in intra-CU LTM a redundant and often suboptimal resource reservation can be observed, caused by the need to prepare a set of candidate LTM cells before the actual cell switch is performed.

An AI/ML function can be adopted not only to generate a recommended set of candidate LTM cells, but also to determine the appropriate time for each candidate LTM cell to reserve resources for a UE ahead of key LTM events, e.g., early synchronization, cell switch.

### 4.2.2	Solutions and standard impacts

#### 4.2.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

In case of CU-DU split architecture, the following solutions can be considered for supporting AI/ML assisted intra-CU LTM:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.2.2.2	Input data of AI/ML assisted Intra-CU LTM

To optimize LTM performance, a gNB-CU may need the following information as input data for AI/ML assisted intra-CU LTM:

From local node:

-	Measured/Predicted radio resource status per cell/SSB area

-	Measured/Predicted UE trajectory

-	UE history information

-	Historical UE’s candidate LTM cell list(s)

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.2.2.3	Output data of AI/ML assisted Intra-CU LTM

An AI/ML model in the gNB-CU for AI/ML assisted intra-CU LTM can generate the following information as output:

-	Predicted candidate LTM cells list for subsequent intra-CU mobility events (both intra-DU and inter-DU)

-     Predicted time of stay of the UE in each predicted candidate LTM cell

-	Predicted time for key LTM events (e.g., cell switch, early synchronization)

#### 4.2.2.4	Feedback of AI/ML assisted Intra-CU LTM

To optimize the performance of the AI/ML model used for AI/ML assisted intra-CU LTM, the following feedback can be considered to be collected from gNB-DU(s):

-	Actual LTM cells for each cell switch event

-    Actual time of stay of the UE in each LTM cell of each cell switch event

-    Actual time of occurrence of key LTM events (e.g., cell switch, early synchronization)

#### 4.2.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-20 normative work compared with what was specified during Rel-19.

F1 interface:

- Enable transfer of the following predicted information from CU to DU:

- Predicted candidate LTM cell(s) of the corresponding DU

- Predicted time of stay of the UE in each predicted candidate LTM cell of the corresponding DU

- Predicted time for key LTM events (e.g., cell switch, early synchronization) in the corresponding DU

- Enable transfer of the following measured/actual information from DU to CU:

- Actual LTM cell(s) of the corresponding DU for each cell switch event

- Actual time of stay of the UE in each LTM cell of the corresponding DU for each cell switch event

- Actual time of occurrence of key LTM events (e.g., cell switch, early synchronization) in the corresponding DU for each cell switch event

<<<<<<<<<< End of Changes >>>>>>>>>>