3GPP TSG-RAN WG3 Meeting #129bis	R3-256993

Prague, Czech Republic, 13th – 17th October, 2025

Title: 	(TP for TR 38.745) Discussion for Handover Enhancements

Source: 	Huawei

Agenda item:	12.2.3

Document Type:	Discussion and Decision

# 1. Introduction

A new SI “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” was approved for Rel-20 5G-Advanced (5G-A) at RAN #108 meeting [1], then revised in RAN#109 and eventually approved in [2], aiming at investigating mobility scenarios that could benefit from using AI/ML techniques.

The objective of the new SI is excerpted from [2] and reported below for convenience:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

This contribution focuses in particular on the “handover enhancements, e.g. inter-CU LTM” scenario, providing our considerations and proposals for discussion.

# 2. Discussion

## 2.1 AI/ML for handover enhancements

Back in Rel-17 RAN3 started discussing how to optimize the UE mobility by means of AI/ML and eventually in Rel-18 the cell-based UE trajectory prediction was specified to be signalled in the XnAP’s HANDOVER REQUEST message (refer to TS 38.300 [3]), allowing the target gNB to be made aware of the list of its own cells predicted to be visited by the handed over UE, optionally together with the predicted time of stay of the UE in those cells. Delivering the predicted trajectory information to the target gNB enables more efficient handover preparation, allowing proactive resource reservation and enhancing mobility robustness. As part of the Rel-19 SI, instead, the predicted multi-hop UE trajectory – spanning multiple gNBs beyond the first target gNB – was also discussed but no conclusion was reached back then, and RAN3 will make an attempt to re-discuss such topic in Rel-20.

LTM is under discussion since Rel-18, focusing on LTM cell switching between cells either belonging to the same gNB(-CU) or, since Rel-19, to different gNB(-CU)s – in the first case we refer to intra-CU LTM, while the latter refers to inter-CU LTM. Unlike the legacy L3 handover, where the chosen SSBs are configured at the time of handover through RRC reconfiguration (i.e., the UE learns which SSB/beam to use when executing the switch), LTM introduces an earlier preparation mechanism. In LTM, in fact, candidate TCI (Transmission Configuration Indicator) states are pre-configured and activated before the UE accesses the target cell, and the specific TCI state can be associated with the cell-switch decision itself. As a result, when the LTM switch is triggered, the UE has already completed the early synchronization and knows which beam and candidate configuration to use for immediate access, reducing post-handover beam training and supporting the low-latency objective of LTM.

Since LTM configures the cells (and beams) likely to be used by the UE prior to the cell-switch decision, the UE trajectory prediction becomes particularly valuable in LTM; for the intra-CU case, the legacy Rel-18 cell-based UE trajectory prediction can be used directly since such trajectory includes cells of a single gNB only (i.e., of the target gNB). In the inter-CU case, instead, since cells from multiple gNBs are part of the LTM candidate cells list, the multi-hop UE trajectory may be leveraged. Having knowledge of the predicted cell-based UE trajectory allows the network to optimize both candidate cell selection and resource allocation. During the LTM preparation phase, the gNB is not only responsible for determining the candidate cells but also for configuring the corresponding beam resources. Therefore, providing the gNB with information on when a UE is expected to access a specific beam enables more precise scheduling. This foresight helps to avoid conflicts in beam resource allocation across time, frequency, and spatial domains, while also improving spectrum efficiency and ensuring seamless low-latency mobility. However, the current UE trajectory prediction is available only with a cell level granularity, and to enable LTM-specific beam-level early preparation, a finer granularity (i.e., per SSB) of the predicted UE trajectory should be considered.

Proposal 1: RAN3 to agree to introduce a finer granularity (i.e., per SSB) of the predicted UE trajectory.

- Based on the discussion on the AI/ML-based intra-CU LTM, RAN3 may evaluate to extend the outcome of such discussion to the inter-CU LTM.

- Proposal 2: RAN3 to put on hold the discussion on AI/ML-based inter-CU LTM and wait for some progress on the AI/ML-based intra-CU LTM before starting to discuss the inter-CU case.

- In Rel-16, the Mobility Robustness Optimization (MRO) was specified for NR and enhanced further in subsequent 3GPP releases. Several events defined for MRO and corresponding feedback were proposed, as listed in TS 38.300 clause 15.5.2 [3]. Building on this, it can be evaluated to study the use of AI/ML techniques for predicting MRO events proactively, rather than relying solely on self-optimization after failure events occur. The existing MRO feedback information (e.g., success/failure indication, failure type, beam failure indicator) can be reused as feedback to evaluate and improve the prediction accuracy of AI/ML models. This closed-loop approach may enable adaptive triggering, reduce unnecessary signalling, and enhance mobility robustness in dynamic deployment scenarios.

- Proposal 3: RAN3 to agree to study the use of AI/ML techniques for predicting MRO events proactively, rather than relying solely on self-optimization after failure events occur.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: RAN3 to agree to introduce a finer granularity (i.e., per SSB) of the predicted UE trajectory.

- Proposal 2: RAN3 to put on hold the discussion on AI/ML-based inter-CU LTM and wait for some progress on the AI/ML-based intra-CU LTM before starting to discuss the inter-CU case.

- Proposal 3: RAN3 to agree to study the use of AI/ML techniques for predicting MRO events proactively, rather than relying solely on self-optimization after failure events occur.

# 4. Reference

- RP-251868, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#108

- RP-252867, “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#109

- TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

# Annex – TP for TR 38.745

<<<<<<<<<< Start of Changes >>>>>>>>>>

# 1	Scope

The present document provides the description and investigation of new AI/ML based use cases, i.e., multi-hop UE trajectory, intra-CU LTM and other handover enhancements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X1]	3GPP TS 38.300: “NR; NR and NG-RAN Overall description; Stage-2”.

<<<<<<<<<< Next Change >>>>>>>>>>

## 4.3	Other handover enhancements

The AI/ML function may be beneficial for:

-	enhancing mobility-related features (e.g., LTM) by means of beam-level UE trajectory predictions, and

-	predicting Mobility Robustness Optimization (MRO) events proactively.

Cell-based UE trajectory prediction was specified in Rel-18; a finer granularity of the UE trajectory prediction may be used in LTM during the preparation phase, where the gNB is not only responsible for determining the candidate LTM cells but also for configuring the corresponding beam resources. In fact, providing the gNB with information on when a UE is expected to access a specific beam enables more precise scheduling, avoids conflicts in beam resource allocation across time, frequency, and spatial domains, and also improves spectrum efficiency while ensuring seamless low latency mobility.

- MRO was specified for NR in Rel-16 and enhanced further in subsequent 3GPP releases. Several events defined for MRO and corresponding feedback were proposed, as listed in TS 38.300 [X1]. AI/ML techniques may be useful for predicting MRO events proactively, rather than relying solely on self-optimization after failure events occur, hence enhancing mobility robustness further, especially in dynamic deployment scenarios.

Editor’s Note: Assessment of solutions and standard impact of the above is FFS

Editor’s Note: To discuss whether additional enhancements are considered in this study (e.g., inter-CU LTM)

<<<<<<<<<< End of Changes >>>>>>>>>>