3GPP TSG-RAN WG3 Meeting # 129bis	R3-257002

# Prague, Czech Republic, 13 – 17 October, 2025

Agenda Item:	8.1

Source:	Ericsson

Title:	Discussion on Average Window for Alternative QoS

Document for:	Discussion

- Introduction

In RAN3#127bis (Wuhan April 2025) meeting, RAN3 discussed the addition of Averaging Window (AW) into in Alternative QoS Profile Parameters (AQP) in NGAP specification based on noticing the addition to SA2 TS 23.501 [1]:

#### 5.7.1.2a Alternative QoS Profile The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, a prioritized list of Alternative QoS Profile(s) to the NG-RAN. If the SMF provides a new prioritized list of Alternative QoS Profile(s) to the NG-RAN (if the corresponding PCC rule information changes), the NG-RAN shall replace any previously stored list with it. An Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt. NOTE 1: There is no requirement that the GFBR monotonically decreases, nor that the PDB or PER monotonically increase as the Alternative QoS Profiles become less preferred. For a GBR QoS Flow using the Delay-critical resource type, an Alternative QoS Profile may also include the QoS parameter MDBV.

However, decision could not be concluded in RAN3 because of different understandings on stage 2 behaviour and how should NG-RAN use its monitoring resources with the addition of this new AW parameter. Hence a LS to SA2 was sent [2].

In this meeting, RAN3 received a reply LS [3] that no consensus was reached in SA2 on stage 2 understanding. Hence a bottle neck situation. In this paper we propose a way-forward.

# 2	Discussion

According to RAN3 discussion in RAN3#127bis meeting, there are two understandings from companies’ viewpoint:

- Understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

- Understanding 2: The Alternative QoS Parameter sets may contain different averaging window configurations. It means that the NG-RAN will use different durations to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. This represents a new requirement to NG-RAN.

Despite a respectful request to SA2 to clarify the introduction of AW into the AQP, SA2 was unable to reach a consensus on this issue and left it to RAN3 to decide on the usage of AW in the AQP. It should also be noted that although SA2 acknowledged the lack of consensus, they did not remove AW from TS 23.501 and agreed to keep the specification as is.

Observation 1: SA2 did not reach consensus on stage 2 impact

Observation 2: SA2 did not have clear understanding on usage of AW to AQP that they have introduced to TS 23.501

Observation 3: Despite this, SA2 will not revert their CR to TS 23.501.

…

In order to facilitate the discussion and avoid further LS back and forth with SA2, we propose the way-forward in RAN3:

- RAN3 will agree that NG-RAN node will not use different AW values across the AQPs and the QoS profile. NG-RAN node re-uses the default AW value defined for the primary QoS profile for the AQP when it is signalled over NGAP.

- The above principle also applies for the other interfaces (Xn/F1/E1).

- Capture the above understanding in TS 38.300 that the NG-RAN node performs monitoring using only one default value of AW.

Proposal 1: NG-RAN node performs QoS monitoring when AQP are present including AW using only one default value of AW that is the default AW present in the QoS profile. Extend this principle to other interfaces: Xn, F1, E1

CRs to NGAP, XnAP, F1AP, E1AP and TS 38.300 from Rel-18 are available for agreement.

# 3	Conclusion and Proposals

Observation 1: SA2 did not reach consensus on stage 2 impact

Observation 2: SA2 did not have clear understanding on usage of AW to AQP that they have introduced to TS 23.501

Observation 3: Despite this, SA2 will not revert their CR.

Proposal 1: NG-RAN node performs QoS monitoring when AQP are present including AW using only one default value of AW that is the default AW present in the QoS profile. Extend this principle to other interfaces: Xn, F1, E1

# 4	References

- 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

- R3-252397, LS on Average Window for Alternative QoS, RAN3(CATT).

- S2-2508118, LS on Average Window for Alternative QoS, SA2(CATT).