3GPP TSG-RAN WG3 Meeting # 129bis	R3-257003

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1355 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Alternative QoS parameter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.501 section 5.7.1.2a mentions that Averaging window is part of Alternative QoS Parameters. When adding Avering window to AQP, NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which AQP can be used as reference if the QoS profile is not fulfilled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Averaging window in AQP that has the same default value as the one in the QoS Profile. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only impacts Alternative QoS parameters in the specs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA2 spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.151, 9.4.5, 9.4.7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1571 TS 38.473 CR 1618 TS 37.483 CR 0181 TS 38.300 draftCR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.3.1.151	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  |  | Semantics description |  |  | Criticality |  |  | Assigned Criticality |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  |  |  | 1..<maxnoofQoSparaSets> |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >Alternative QoS Parameters Set Index |  | M |  |  |  | 9.3.1.152 |  |  |  |  |  | - |  |  |  |  |  |
| >Guaranteed Flow Bit Rate Downlink |  | O |  |  |  | Bit Rate 9.3.1.4 |  |  |  |  |  | - |  |  |  |  |  |
| >Guaranteed Flow Bit Rate Uplink |  | O |  |  |  | Bit Rate 9.3.1.4 |  |  |  |  |  | - |  |  |  |  |  |
| >Packet Delay Budget |  | O |  |  |  | 9.3.1.80 |  |  |  |  |  | - |  |  |  |  |  |
| >Packet Error Rate |  | O |  |  |  | 9.3.1.81 |  |  |  |  |  | - |  |  |  |  |  |
| >Maximum Data Burst Volume |  | O |  |  |  | 9.3.1.83 |  |  | Maximum Data Burst Volume is specified in TS 23.501 [9]. This IE may be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. |  |  | YES |  |  | ignore |  |  |
| >Averaging Window | O |  | 9.3.1.82 |  | Averaging Window is specified in TS 23.501 [9]. This IE has the same value as the Averaging Window IE present in QoS Flow Level QoS Parameters IE in 9.3.1.12. |  | YES |  | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSparaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS profile. Value is 8. |

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

SKIPPED TEXT UNCHANGED

id-MaximumDataBurstVolume,

id-MBS-NGUFailureIndication,

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-ExtendedBackupAMFName,

id-AveragingWindow,

maxnoofAllowedAreas,

SKIPPED TEXT UNCHANGED

#### -- A

SKIPPED TEXT UNCHANGED

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQosParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			AlternativeQoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate												OPTIONAL,

guaranteedFlowBitRateUL				BitRate												OPTIONAL,

packetDelayBudget					PacketDelayBudget									OPTIONAL,

packetErrorRate						PacketErrorRate										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaximumDataBurstVolume	CRITICALITY ignore	EXTENSION MaximumDataBurstVolume		PRESENCE optional		}|

{ ID id-AveragingWindow			CRITICALITY ignore	EXTENSION AveragingWindow				PRESENCE optional	},

...

}

NEXT CHANGE

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

SKIPPED TEXT UNCHANGED

id-PDUSessionListMTCommHReq								ProtocolIE-ID ::= 431

id-MaximumDataBurstVolume 								ProtocolIE-ID ::= 432

id-MN-only-MDT-collection								ProtocolIE-ID ::= 433

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-TAIMBSSupportList										ProtocolIE-ID ::= 441

id-ExtendedBackupAMFName    								ProtocolIE-ID ::= 442

id-ExtendedOldAMF			    							ProtocolIE-ID ::= 443

id-AveragingWindow										ProtocolIE-ID ::= xxx

END

-- ASN1STOP

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////