3GPP TSG-RAN WG3 Meeting # 129bis	R3-257014

Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1621 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 2Rx XR UE Capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Inc., Nokia, Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 2Rx XR UEs are intended only for use in XR devices that are worn on human head and whose constrained form factors have limited volume available for Rx chains. In Release 18, barring information on which neighboring nodes allow 2Rx XR UEs to allow access was added over the Xn and F1 interface, as part of the system information broadcast provided in the served cell infomation. When it comes to paging aspects, information that 2 Rx XR UE is supported is aiialable in the RRC UERadioPagingInformation message: - UERadioPagingInformation This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs. Direction: gNB to/ from 5GC and gNB to/from gNB UERadioPagingInformation message -- ASN1START -- TAG-UE-RADIO-PAGING-INFORMATION-START UERadioPagingInformation ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ ueRadioPagingInformation UERadioPagingInformation-IEs, spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } UERadioPagingInformation-IEs ::= SEQUENCE { supportedBandListNRForPaging SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, nonCriticalExtension UERadioPagingInformation-v15e0-IEs OPTIONAL } UERadioPagingInformation-v15e0-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1700-IEs OPTIONAL } UERadioPagingInformation-v1700-IEs ::= SEQUENCE { ue-RadioPagingInfo-r17 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17) OPTIONAL, inactiveStatePO-Determination-r17 ENUMERATED {supported} OPTIONAL, numberOfRxRedCap-r17 ENUMERATED {one, two} OPTIONAL, halfDuplexFDD-TypeA-RedCap-r17 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1800-IEs OPTIONAL } UERadioPagingInformation-v1800-IEs ::= SEQUENCE { numberOfRxERedCap-r18 ENUMERATED {one, two} OPTIONAL, supportOf2RxXR-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1840-IEs OPTIONAL } UERadioPagingInformation-v1840-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } -- TAG-UE-RADIO-PAGING-INFORMATION-STOP -- ASN1STOP Over F1AP PAGING message, it is possible to indicate the RedCap UE, eRedCap UE, INACTIVE State PO-Determination support indication in the UE Paging Capability IE towards DU, so that DU can apply appropriate paging scheme, e.g., paging boosting, etc.. But information on 2 Rx XR UE support indication is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new IE 2 Rx XR UE indication IE in the UE Paging Capability IE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The gNB-DU cannot page the UE according to its UE paging capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.270, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------- CHANGES START HERE --------------------------------------

#### 9.3.1.270	UE Paging Capability

This IE provides the UE Paging Capability information needed for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| INACTIVE State PO-Determination | O |  | ENUMERATED(supported,…) | Corresponds to the inactiveStatePO-Determination contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. | - |  |
| RedCap Indication | O |  | ENUMERATED(true,…) | Indicates that the paged UE is a Redcap UE or an eRedCap UE. | YES | ignore |
| Paging Adaptation Indication | O |  | ENUMERATED(supported,…) | Corresponds to the pagingAdaptation contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. | YES | ignore |
| 2 Rx XR UE indication | O |  | ENUMERATED(true,…) | Indicates that the paged UE is a 2 Rx XR UE. | YES | ignore |

-------------------------------------- NEXT CHANGE --------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

//skip the unaffected text//

id-MMSID,

id-Indication-of-Bitrate-Adaptation,

id-DLPDUSetInformationMarkingSupportIndication,

id-FiveGProSeLayer3MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay,

id-FiveGProSeLayer2MHRemote,

id-LPWUSSubgroupingSupportIndication,

id-SBFD-Frequency-Configuration,

id-SSB-resource-config,

id-NZP-CSI-RS-Resources-Config,

id-SRS-Resource-Configuration,

id-rLFReportFailureType,

id-C-RNTI,

id-OnDemandSIB1,

id-PagingAdaptationIndication,

id-PEISubgroupingSupportIndication-PagingAdaptation,

id-ServingCellMO-Ondemand,

id-LTMgNB-ID,

id-L1ExecutionConditionList,

id-LTMSecurityInformation,

id-RequestforCSI-RSResourceConfigforL1Measure,

id-RequestforCSI-RSResourceConfigforCSIAcquisition,

id-CSI-RSResourceConfigforL1Measure,

id-CSI-RSResourceConfigforCSIAcquisition,

id-CSIReportConfgforCSIAcquisition,

id-RequestforL1ExecutionCondition,

id-CSI-RSMeasurementsList,

id-LTMResidualTAInfoList,

id-ChannelResponseInformation,

id-UL-SRS-TDCT,

id-PerformanceDelayMonitoring,

id-Two-Rx-XR-UE-Indication,

//skip the unaffected text//

TwoPHRModeMCG ::= ENUMERATED {enabled, ...}

TwoPHRModeSCG ::= ENUMERATED {enabled, ...}

TxHoppingConfiguration ::= SEQUENCE {

overlapValue						ENUMERATED {rb0, rb1, rb2, rb4},

numberOfHops						INTEGER (2..6),

slotOffsetForRemainingHopsList		SlotOffsetForRemainingHopsList,

iE-extensions		ProtocolExtensionContainer { { TxHoppingConfiguration-ExtIEs } }	OPTIONAL,

...

}

TxHoppingConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Two-Rx-XR-UE-Indication :: = ENUMERATED {supported, ...}

TAInformation-List	::= SEQUENCE (SIZE(1.. maxnoofTAList)) OF TAInformation-Item

TAInformation-Item	::= SEQUENCE {

nRCGI					NRCGI,

tAValue					TAValue,

iE-Extensions			ProtocolExtensionContainer { { TAInformation-Item-ExtIEs} }	OPTIONAL,

...

}

TAInformation-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID	id-TagIDPointer	CRITICALITY ignore	EXTENSION	TagIDPointer	PRESENCE optional},

...

}

//skip the unaffected text//

UEPagingCapability ::= SEQUENCE {

iNACTIVEStatePODetermination						ENUMERATED {supported, ...} 	OPTIONAL,

iE-Extension				ProtocolExtensionContainer { { UEPagingCapability-ExtIEs} } 		OPTIONAL,

...

}

UEPagingCapability-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RedCapIndication		CRITICALITY ignore 	EXTENSION RedCapIndication		PRESENCE optional }|

{ 	ID id-PagingAdaptationIndication	CRITICALITY ignore	EXTENSION PagingAdaptationIndication	PRESENCE optional	}|

{	ID id-Two-Rx-XR-UE-Indication		CRITICALITY ignore 	EXTENSION Two-Rx-XR-UE-Indication		PRESENCE optional },

...

}

-------------------------------------- NEXT CHANGE --------------------------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

//skip the unaffected text//

id-RequestforCSI-RSResourceConfigforCSIAcquisition	ProtocolIE-ID ::= 900

id-CSI-RSResourceConfigforL1Measure					ProtocolIE-ID ::= 901

id-CSI-RSResourceConfigforCSIAcquisition			ProtocolIE-ID ::= 902

id-CSIReportConfgforCSIAcquisition					ProtocolIE-ID ::= 903

id-RequestforL1ExecutionCondition					ProtocolIE-ID ::= 904

id-LTMInformationSCGAdd								ProtocolIE-ID ::= 905

id-LTMInformationSCGMod								ProtocolIE-ID ::= 906

id-TARemainingInfoList								ProtocolIE-ID ::= 907

id-CSI-RSCoordinationRequestList					ProtocolIE-ID ::= 908

id-CSI-RSCoordinationResultList						ProtocolIE-ID ::= 909

id-CSI-RSMeasurementsList							ProtocolIE-ID ::= 910

id-LTMResidualTAInfoList							ProtocolIE-ID ::= 911

id-ChannelResponseInformation						ProtocolIE-ID ::= 912

id-UL-SRS-TDCT										ProtocolIE-ID ::= 913

id-Future-Coverage-Modification-Notification		ProtocolIE-ID ::= 914

id-Predicted-CCO-Assistance-Information				ProtocolIE-ID ::= 915

id-PerformanceDelayMonitoring						ProtocolIE-ID ::= 916

id-NodeAssociatedInfoResult							ProtocolIE-ID ::= 917

id-NeighbourFutureCoverageModNotification			ProtocolIE-ID ::= 918

id-Two-Rx-XR-UE-Indication							ProtocolIE-ID ::= XXX

-------------------------------------- END OF CHANGE --------------------------------------