3GPP TSG-RAN WG3 #129-bis																	R3-257044

Prague, Czech Republic, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1625 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PRACH resources for RACH-less LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A source gNB-DU can use PRACH resources allocated for LTM by a candidate DU for multiple UEs. However, the source gNB-DU is not informed when PRACH resources are released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce two new class 2 procedures, one from candidate gNB-DU to gNB-CU, and one from gNB-CU to source gNB-DU, to notify that PRACH resources previously allocated are released. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The source gNB-DU has no clear understanding of the amount PRACH resources available for LTM, which may lead to use RACH-based LTM or failures in LTM execution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.3.x (new), 8.3.y (new), 9.2.2.x1 (new), 9.2.2.y1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1574 TS 38.401 CR 0499 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

(skip unchanged)

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| DU-CU Cell Switch Notification | DU-CU CELL SWITCH NOTIFICATION |
| CU-DU Cell Switch Notification | CU-DU CELL SWITCH NOTIFICATION |
| DU-CU TA Information Transfer | DU-CU TA INFORMATION TRANSFER |
| CU-DU TA Information Transfer | CU-DU TA INFORMATION TRANSFER |
| QoE Information Transfer Control | QOE INFORMATION TRANSFER CONTROL |
| RACH Indication | RACH INDICATION |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| Mobile IAB F1 Setup Triggering | MIAB F1 SETUP TRIGGERING |
| Mobile IAB F1 Setup Outcome Notification | MIAB F1 SETUP OUTCOME NOTIFICATION |
| Broadcast Transport Resource Request | BROADCAST TRANSPORT RESOURCE REQUEST |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| DU-CU Access And Mobility Indication | DU-CU ACCESS AND MOBILITY INDICATION |
| CU-DU Mobility Initiation | CU-DU MOBILITY INITIATION REQUEST |
| CLI Indication | CLI INDICATION |
| DU-CU RACH Resources Release Indication | DU-CU RACH RESOURCES RELEASE INDICATION |
| CU-DU RACH Resources Release Indication | CU-DU RACH RESOURCES RELEASE INDICATION |

<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>

### 8.3.x	DU-CU RACH Resources Release Indication

#### 8.3.x.1	General

The purpose of the RACH Resource Indication procedure is to inform the gNB-CU about the release of RACH Resources for LTM.

#### 8.3.x.2	Successful Operation

Figure 8.3.x.2-1: DU-CU RACH Resources Release Indication procedure, successful operation

Upon receipt of the RACH RESOURCES RELEASE INDICATION, the gNB-CU shall take the information into account.

### 8.3.y	CU-DU RACH Resources Release Indication

#### 8.3.y.1	General

The purpose of the RACH Resource Indication procedure is to inform the gNB-DU about the release of RACH Resources for LTM.

#### 8.x.y.2	Successful Operation

Figure 8.3.y.2-1: CU-DU RACH Resources Release Indication procedure, successful operation

Upon receipt of the RACH RESOURCES RELEASE INDICATION, the gNB-DU shall take the information into account.

<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>

#### 9.2.2.x1	DU-CU RACH RESOURCES RELEASE INDICATION

This message is sent by the gNB-DU to inform the gNB-CU about the release of RACH Resources for LTM.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| RACH Resources Release gNB-DUs List |  | 1 |  | This IE contains the IDs of the candidate gNB-DU(s). | YES | reject |
| >RACH Resources Release gNB-DUs Item |  | 1..< maxnoofLTMgNB-DUs> |  |  | - |  |
| >>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>Preamble Index List | O |  | 9.3.1.329 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

#### 9.2.2.y1	CU-DU RACH RESOURCES RELEASE INDICATION

This message is sent by the gNB-CU to inform the gNB-DU about the release of RACH Resources for LTM.

Direction: gNB-CU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| RACH Resources Release gNB-DUs List |  | 1 |  | This IE contains the IDs of the candidate gNB-DU(s). | YES | reject |
| >RACH Resources Release gNB-DUs Item |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>Preamble Index List | O |  | 9.3.1.329 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Start of ASN.1 changes >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM F1AP-CommonDataTypes

Reset,

ResetAcknowledge,

F1SetupRequest,

(skip)

MulticastContextNotificationConfirm,

MulticastContextNotificationRefuse,

MulticastCommonConfigurationRequest,

MulticastCommonConfigurationResponse,

MulticastCommonConfigurationRefuse,

BroadcastTransportResourceRequest,

DUCUAccessAndMobilityIndication,

DUCUAccessAndMobilityIndication,

SRSInformationReservationNotification,

CUDUMobilityInitiationRequest,

CLI-Indication,

DUCUCSIRSCoordinationRequest,

DUCUCSIRSCoordinationResponse,

CUDUCSIRSCoordinationRequest,

CUDUCSIRSCoordinationResponse,

DUCURACHResourcesReleaseIndication,

CUDURACHResourcesReleaseIndication

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

id-gNBDUConfigurationUpdate,

id-gNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-DUCUAccessAndMobilityIndication,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-GNBDUResourceCoordination,

id-privateMessage,

id-UEInactivityNotification,

id-InitialULRRCMessageTransfer,

id-SystemInformationDeliveryCommand,

id-Paging,

id-Notify,

id-WriteReplaceWarning,

id-PWSCancel,

id-PWSRestartIndication,

id-PWSFailureIndication,

id-GNBDUStatusIndication,

id-RRCDeliveryReport,

id-F1Removal,

id-NetworkAccessRateReduction,

id-TraceStart,

id-DeactivateTrace,

id-DUCURadioInformationTransfer,

id-CUDURadioInformationTransfer,

id-BAPMappingConfiguration,

id-GNBDUResourceConfiguration,

id-IABTNLAddressAllocation,

id-IABUPConfigurationUpdate,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-accessAndMobilityIndication,

id-ReferenceTimeInformationReportingControl,

id-ReferenceTimeInformationReport,

id-accessSuccess,

id-cellTrafficTrace,

id-PositioningMeasurementExchange,

id-PositioningAssistanceInformationControl,

id-PositioningAssistanceInformationFeedback,

id-PositioningMeasurementReport,

id-PositioningMeasurementAbort,

id-PositioningMeasurementFailureIndication,

id-PositioningMeasurementUpdate,

id-TRPInformationExchange,

id-PositioningInformationExchange,

id-PositioningActivation,

id-PositioningDeactivation,

id-PositioningInformationUpdate,

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-BroadcastContextSetup,

id-BroadcastContextRelease,

id-BroadcastContextReleaseRequest,

id-BroadcastContextModification,

id-MulticastGroupPaging,

id-MulticastContextSetup,

id-MulticastContextRelease,

id-MulticastContextReleaseRequest,

id-MulticastContextModification,

id-MulticastDistributionSetup,

id-MulticastDistributionRelease,

id-PDCMeasurementInitiation,

id-PDCMeasurementTerminationCommand,

id-PDCMeasurementFailureIndication,

id-PDCMeasurementReport,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-PosSystemInformationDeliveryCommand,

id-DUCUCellSwitchNotification,

id-CUDUCellSwitchNotification,

id-DUCUTAInformationTransfer,

id-CUDUTAInformationTransfer,

id-QoEInformationTransferControl,

id-RachIndication,

id-TimingSynchronisationStatus,

id-TimingSynchronisationStatusReport,

id-MIABF1SetupTriggering,

id-MIABF1SetupOutcomeNotification,

id-MulticastContextNotification,

id-MulticastCommonConfiguration,

id-BroadcastTransportResourceRequest,

id-SRSInformationReservationNotification,

id-CUDUMobilityInitiationRequest,

id-CLI-Indication,

id-DUCUCSIRSCoordination,

id-CUDUCSIRSCoordination,

id-DUCURACHResourcesReleaseIndication,

id-CUDURACHResourcesReleaseIndication

FROM F1AP-Constants

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

##### skip unchanged #####

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

##### skip unchanged #####

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

multicastGroupPaging						|

broadcastContextReleaseRequest				|

multicastContextReleaseRequest				|

pDCMeasurementReport						|

pDCMeasurementTerminationCommand			|

pDCMeasurementFailureIndication				|

measurementActivation						|

qoEInformationTransfer						|

posSystemInformationDelivery					|

dUCUCellSwitchNotification					|

cUDUCellSwitchNotification					|

dUCUTAInformationTransfer					|

cUDUTAInformationTransfer					|

qoEInformationTransferControl					|

rachIndication								|

timingSynchronisationStatusReport				|

mIABF1SetupTriggering						|

mIABF1SetupOutcomeNotification				|

broadcastTransportResourceRequest			|

dUCUAccessAndMobilityIndication			|

sRSInformationReservationNotification		|

cUDUMobilityInitiation						|

cLI-Indication								|

dUCURACHResourcesReleaseIndication			|

cUDURACHResourcesReleaseIndication			,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

##### skip unchanged #####

dUCUCSIRSCoordination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUCSIRSCoordinationRequest

SUCCESSFUL OUTCOME		DUCUCSIRSCoordinationResponse

PROCEDURE CODE			id-DUCUCSIRSCoordination

CRITICALITY				reject

}

cUDUCSIRSCoordination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDUCSIRSCoordinationRequest

SUCCESSFUL OUTCOME		CUDUCSIRSCoordinationResponse

PROCEDURE CODE			id-CUDUCSIRSCoordination

CRITICALITY				reject

}

dUCURACHResourcesReleaseIndication 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCURACHResourcesReleaseIndication

PROCEDURE CODE			id-DUCURACHResourcesReleaseIndication

CRITICALITY				ignore

}

cUDURACHResourcesReleaseIndication 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDURACHResourcesReleaseIndication

PROCEDURE CODE			id-CUDURACHResourcesReleaseIndication

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

##### skip unchanged #####

LastVisitedLTMCells,

OnDemand-SIB1-Cell,

LTMSecurityInformation,

LTMInformationSCGAdd,

LTMInformationSCGMod,

TARemainingInfoList,

CSI-RSCoordinationRequestList,

CSI-RSCoordinationResultList,

Future-Coverage-Modification-Notification,

Predicted-CCO-Assistance-Information,

Neighbour-Future-Coverage-Modification-Notification,

RACHResourcesRelease-gNB-DUs-List

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

##### skip unchanged #####

id-TARemainingInfoList,

id-CSI-RSCoordinationRequestList,

id-CSI-RSCoordinationResultList,

id-Future-Coverage-Modification-Notification,

id-Predicted-CCO-Assistance-Information,

id-NeighbourFutureCoverageModNotification,

id-RACHResourcesRelease-gNB-DUs-List,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofMRBsforUE,

maxnoofServingCellMOs

FROM F1AP-Constants

##### skip unchanged #####

-- **************************************************************

--

#### -- DU-CU RACH RESOURCES RELEASE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU RACH RESOURCES RELEASE INDICATION

--

-- **************************************************************

DUCURACHResourcesReleaseIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {DUCURACHResourcesReleaseIndicationIEs} },

...

}

DUCURACHResourcesReleaseIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-NRCGI                   				CRITICALITY reject  TYPE NRCGI                          				PRESENCE mandatory  }|

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-RACHResourcesRelease-gNB-DUs-List		CRITICALITY reject	TYPE RACHResourcesRelease-gNB-DUs-List			PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- CU-DU RACH RESOURCES RELEASE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU RACH RESOURCES RELEASE INDICATION

--

-- **************************************************************

CUDURACHResourcesReleaseIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {CUDURACHResourcesReleaseIndicationIEs } },

...

}

CUDURACHResourcesReleaseIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-NRCGI                   				CRITICALITY reject  TYPE NRCGI                          				PRESENCE mandatory  }|

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-RACHResourcesRelease-gNB-DUs-List		CRITICALITY reject	TYPE RACHResourcesRelease-gNB-DUs-List			PRESENCE mandatory	},

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

##### skip unchanged #####

#### -- R

RACHResourcesRelease-gNB-DUs-List ::= SEQUENCE (SIZE(1..maxnoofLTMgNB-DUs)) OF RACHResourcesRelease-gNB-DUs-Item

RACHResourcesRelease-gNB-DUs-Item ::= SEQUENCE {
    lTMgNB-DU-ID								GNB-DU-ID,
    preambleIndexList						PreambleIndexList OPTIONAL,
    iE-Extensions                   	ProtocolExtensionContainer { { RACHResourcesRelease-Item-ExtIEs} } OPTIONAL,
    ...
}

RACHResourcesRelease-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {
    ...
}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

##### skip unchanged #####

id-DUCUTAInformationTransfer				ProcedureCode ::= 84

id-CUDUTAInformationTransfer				ProcedureCode ::= 85

id-QoEInformationTransferControl			ProcedureCode ::= 86

id-RachIndication							ProcedureCode ::= 87

id-TimingSynchronisationStatus				ProcedureCode ::= 88

id-TimingSynchronisationStatusReport		ProcedureCode ::= 89

id-MIABF1SetupTriggering					ProcedureCode ::= 90

id-MIABF1SetupOutcomeNotification			ProcedureCode ::= 91

id-MulticastContextNotification 			ProcedureCode ::= 92

id-MulticastCommonConfiguration				ProcedureCode ::= 93

id-BroadcastTransportResourceRequest		ProcedureCode ::= 94

id-DUCUAccessAndMobilityIndication			ProcedureCode ::= 95

id-SRSInformationReservationNotification	ProcedureCode ::= 96

id-CUDUMobilityInitiationRequest			ProcedureCode ::= 97

id-CLI-Indication							ProcedureCode ::= 98

id-DUCUCSIRSCoordination					ProcedureCode ::= 99

id-CUDUCSIRSCoordination					ProcedureCode ::= 100

id-DUCURACHResourcesReleaseIndication		ProcedureCode ::= 101				-- to be reassigned by MCC

id-CUDURACHResourcesReleaseIndication		ProcedureCode ::= 102

##### skip unchanged #####

-- **************************************************************

--

-- IEs

--

-- **************************************************************

##### skip unchanged #####

id-L1ExecutionConditionList							ProtocolIE-ID ::= 897

id-LTMSecurityInformation							ProtocolIE-ID ::= 898

id-RequestforCSI-RSResourceConfigforL1Measure		ProtocolIE-ID ::= 899

id-RequestforCSI-RSResourceConfigforCSIAcquisition	ProtocolIE-ID ::= 900

id-CSI-RSResourceConfigforL1Measure					ProtocolIE-ID ::= 901

id-CSI-RSResourceConfigforCSIAcquisition			ProtocolIE-ID ::= 902

id-CSIReportConfgforCSIAcquisition					ProtocolIE-ID ::= 903

id-RequestforL1ExecutionCondition					ProtocolIE-ID ::= 904

id-LTMInformationSCGAdd								ProtocolIE-ID ::= 905

id-LTMInformationSCGMod								ProtocolIE-ID ::= 906

id-TARemainingInfoList								ProtocolIE-ID ::= 907

id-CSI-RSCoordinationRequestList					ProtocolIE-ID ::= 908

id-CSI-RSCoordinationResultList						ProtocolIE-ID ::= 909

id-CSI-RSMeasurementsList							ProtocolIE-ID ::= 910

id-LTMResidualTAInfoList							ProtocolIE-ID ::= 911

id-ChannelResponseInformation						ProtocolIE-ID ::= 912

id-UL-SRS-TDCT										ProtocolIE-ID ::= 913

id-Future-Coverage-Modification-Notification		ProtocolIE-ID ::= 914

id-Predicted-CCO-Assistance-Information				ProtocolIE-ID ::= 915

id-PerformanceDelayMonitoring						ProtocolIE-ID ::= 916

id-NodeAssociatedInfoResult							ProtocolIE-ID ::= 917

id-NeighbourFutureCoverageModNotification			ProtocolIE-ID ::= 918

id-RACHResourcesRelease-gNB-DUs-List				ProtocolIE-ID ::= 919		-- to be reassigned by MCC

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of ASN.1 changes >>>>>>>>>>>>>>>>>>>>