3GPP TSG-RAN WG3 Meeting #129bis	R3-257061

# Prague, Czech Republic, 13th - 17th October 2025

Agenda Item:	9.2.7

Source:	Ericsson

Title:	On further open issue for Rel-19 AIoT

Document for:	Discussion and agreement

# 1	Introduction

Proposals for (non-security related) open topics are discussed in this paper

# 2	Discussion

## 2.1	Type constraint for the AIOTF Identifier

The AIOTF Identifier is defined in 38.413 as follows:

| AIOTF Identifier | M |  | OCTET STRING | Coded as the NfInstanceId IE defined in TS 29.571 [35]. |
| --- | --- | --- | --- | --- |

The NfInstanceId is defined in 29.571 as follows

| NfInstanceId | string | String uniquely representing a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 9562 [15]. The hexadecimal letters should be formatted as lower-case characters by the sender, and they shall be handled as case-insensitive by the receiver. Example: "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" (NOTE 3) |
| --- | --- | --- |

While NOTE 3 states " NOTE 3:	NFs shall be able to receive a NF Instance Id in any UUID format."

IETF RFC 9562 defines all versions of UUID as being of constant length of 128bits. Its representation as a string of hexadecimal digits as lower-case character would require a string of at least 32 characters, not sure about the "hyphens" in the example. At least, it shows that (1) the definition as an OCTET STRING is probably wrong and (2) that the type definition can be constrained.

Proposal 1:	Discuss the type definition of the AIOTF Identifier and consider its definition as e.g. a PrintableString of at least 32 characters with a to be defined upper length limit.

## 2.2	Type constraint for the A-IoT Correlation Identifier

The A-IoT Correlation Identifier is defined in 38.413 as follows:

| A-IoT Correlation Identifier | M |  | OCTET STRING |  |
| --- | --- | --- | --- | --- |

It should be possible to uniquely denote an AIoT operation initiated by the AIOTF within a certain time with e.g. 4 octets. As discussed at the last meeting, even if the available number space of 4 octets will be never utilised, having constrained the length of the OCTET STRING to a fixed lengths avoids complex receiving routines at the receiver necessary for an unconstrained type definition.

Proposal 2:	Discuss whether the type definition of the A-IoT Correlation Identifier is agreeable to be changed to OCTET STRING (SIZE(4)).

## 2.3	Reference/semantics for the Time Interval

The Time Interval is part of the Inventory Assistance Information and defined in 38.413 as follows:

| Time Interval | O |  | INTEGER (1..256, ...) | Unit: 0.1 second. |
| --- | --- | --- | --- | --- |

Procedural text in 38.413 is as follows:

Upon receiving the Inventory Assistance Information IE contained in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

38.300 states

The Inventory Request message also includes the Inventory Assistance Information. The Inventory Assistance Information includes the expected D2R message size and may also include e.g., approximate number of Target A-IoT devices, time interval.

So, one can see that there is some lack of specification in RAN TSs what the "time interval" is actually about.

23.369 defines the following:

The AIOTF determines the aggregation assistance information based on the request from the AF or local configuration, which includes:

-	Time interval: the fixed time interval for which NG-RAN collects multiple AIoT Devices’ operation responses before reporting the aggregated AIoT response to the AIOTF. The reporting based on time interval may potentially happen multiple times until the NG-RAN completes the request operation.

If the AF has provided a time interval, then the AIOTF should signal a time interval to the NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

We conclude that one of the affected RAN TSs should reference to TS 23.369, either stage 2 or stage 3.

Proposal 3:	Discuss in which way stage 2 specification in TS 23.369 on the usage of the Time Interval IE should be referenced, either in 38.413 or in 38.300.

# 3	Conclusion and Proposals

Some of the open issues have been discussed in this paper and we propose

Proposal 1:	Discuss the type definition of the AIOTF Identifier and consider its definition as e.g. a PrintableString of at least 32 characters with a to be defined upper length limit.

Proposal 2:	Discuss whether the type definition of the A-IoT Correlation Identifier is agreeable to be changed to OCTET STRING (SIZE(4)).

Proposal 3:	Discuss in which way stage 2 specification in TS 23.369 on the usage of the Time Interval IE should be referenced, either in 38.413 or in 38.300.

# 4	References

[1]	"reference"