3GPP TSG RAN WG3 Meeting #129-bis	R3-257070

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on Trace Suspension feasibility

Source: 	Ericsson

Agenda item:	8.1

Document Type:	Discussion and agreement

# 1. Introduction

In [1] SA5 discusses current operational inefficiencies stemming from the luck of the presence of the administrativeState and operationalState attributes in the TraceJob IOC and asks RAN3 to evaluate the feasibility of adding such attributes in the TraceJob IOC. In particular the following is stated in the referenced LS:

“The TraceJob IOC currently lacks administrativeState and operationalState attributes that exist in the perfMetricJob IOC. This creates operational inefficiencies when users need to temporarily halt trace jobs.

The problems are:

- Operators currently must delete and recreate trace jobs to pause/resume them

- This causes unnecessary overhead and potential configuration loss

- Operators needs indicate strong demand for pause/resume functionality

SA5 has discussed the possible solution: Introduce state management for TraceJob IOC by adding administrativeState and operationalState attributes, consistent with perfMetricJob IOC.

This will provide the following benefits:

- Preserve job configurations during temporary stoppages

- Reduce operational overhead

- Align with perfMetricJob functionality”

In RAN traces are controlled by the use of the Trace Activation message received from the management system in management based Trace activation or the Trace Activation IE in signalling based Trace activation.

# 2. Discussion

## 2.1 Management Based Trace Activation

In NG-RAN the Management Based Trace Activation can be fulfilled with the NG-RAN Cell Traffic trace functionality. In this case the Trace Session Activation is done to one or a list of NG-RAN cell(s) within one NG-RAN node, where Trace Session is activated.

The following trace control and configuration parameters of the Trace Session are received by NG-RAN node in the Trace Session activation message from the management system:

-	Trace Reference.

-	Trace Depth.

-	Trace Target: list of NG-RAN cells.

-	List of Interfaces for NG-RAN node.

-	Trace Collection Entity IP Address for the file-based trace reporting and Trace Reporting

-	Consumer URI for the streaming trace reporting (if streaming based report is supported).

It is expected that if SA5 is to introduce a pause/resume functionality, a related control parameter reflecting the appropriate management state would need to be included in the existing set of the above parameters. Support of such parameters by the RAN is deemed to be feasible and within the current operational framework for Traces in the RAN.

Observation 1: Enabling a pause/resume functionality is feasible in the current management based Trace operational framework in RAN.

## 2.2 Signalling Based Trace Activation

Trace Session is activated in an NG-RAN node when the NG-RAN node receives the TRACE START, INITIAL CONTEXT SETUP REQUEST or HANDOVER REQUEST message with the IE Trace Activation from the AMF and if some activities have been started on the interfaces that have been requested to be traced.

If the subscriber or equipment which is traced makes a handover to a target NG-RAN node using the Xn interface, the source NG-RAN node should propagate the trace control and configuration parameters further to the target NG-RAN node by using the HANDOVER REQUEST message. When the target NG-RAN node receives the HANDOVER REQUEST message it should immediately start a Trace Session according to the trace control and configuration parameters received in the HANDOVER REQUEST message.

If the subscriber or equipment being traced at the old NG-RAN node has been sent to RRC_INACTIVE and then establishes RRC Connection to a new NG-RAN node, the new NG-RAN node initiates the Retrieve UE Context procedure using Xn interface. The old NG-RAN node should propagate the trace control and configuration parameters further to the new NG-RAN node by using the RETRIEVE UE CONTEXT RESPONSE message. When the new NG-RAN node receives the RETRIEVE UE CONTEXT RESPONSE message it should immediately start a Trace Session according to the trace control and configuration parameters received in the RETRIEVE UE CONTEXT RESPONSE message.

If the subscriber or equipment which is traced makes a handover to a target NG-RAN node using the NG interface, it is the AMF's responsibility to propagate the trace control and configuration parameters to the target NG-RAN node.

If the tracing shall continue also after the relocation has been performed, the 5GC Trace Start procedure shall be re-initiated from the 5GC towards the future NG-RAN node after the Relocation Resource Allocation procedure has been executed successfully.

In all those cases Signalling Based Trace Activation is controlled by the Trace Activation IE that contains the following information:

-	Trace Reference and Trace Recording Session Reference.

-	List of Interfaces for NG-RAN node.

-	Trace Depth.

-	Trace Collection Entity IP Address for the file-based trace reporting and Trace Reporting

-	Consumer URI for the streaming trace reporting (if streaming based report is supported).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NG-RAN Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [11] (leftmost 6 octets, with PLMN information encoded as in 9.3.3.5), and Trace Recording Session Reference defined in TS 32.422 [11] (last 2 octets). | - |  |
| Interfaces to Trace | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents an NG-RAN node interface: first bit = NG-C, second bit = Xn-C, third bit = Uu, fourth bit = F1-C, fifth bit = E1: other bits reserved for future use. Value '1' indicates 'should be traced'. Value '0' indicates 'should not be traced'. | - |  |
| Trace Depth | M |  | ENUMERATED (minimum, medium, maximum, minimumWithoutVendorSpecificExtension, mediumWithoutVendorSpecificExtension, maximumWithoutVendorSpecificExtension, …) | Defined in TS 32.422 [11]. | - |  |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.3.2.4 | For File based Reporting. Defined in TS 32.422 [11]. This IE is ignored if the Trace Collection Entity URI IE is present. | - |  |
| MDT Configuration | O |  | 9.3.1.167 |  | YES | ignore |
| Trace Collection Entity URI | O |  | URI 9.3.2.14 | For Streaming based Reporting. Defined in TS 32.422 [11]. | YES | ignore |

It is expected that if SA5 is to introduce a pause/resume functionality, a related control parameter reflecting the appropriate management state would need to be included in the existing set of the above parameters. Support of such parameters by the RAN is feasibleand  within the current operational framework for Traces in the RAN.

Observation 2: Enabling pause/resume functionality is feasible in the current Singalling Based Trace operational framework in RAN.

Proposal 1: Agree that enabling suspension\resumption of trace functionality in RAN, through the use of the administrative and operation state attributes in the TraceJob IOC is feasible

Proposal 2: Agree in responding to SA5 as in [2].

# 2. Conclusion

In this paper we discuss the SA5 LS on LS on temporary suspension of trace production and agree on the Reply LS.

The following proposals and observations are derived:

Observation 1: Enabling a pause/resume functionality is feasible in the current management based Trace operational framework in RAN.

Observation 2: Enabling pause/resume functionality is feasible in the current Singalling Based Trace operational framework in RAN.

Proposal 1: Agree that enabling suspension\resumption of trace functionality in RAN, through the use of the administrative and operation state attributes in the TraceJob IOC is feasible

Proposal 2: Agree in responding to SA5 as in [2].

2. References

- S5-253909, LS on temporary suspension of trace production, SA5

- R3-257071, Reply LS on temporary suspension of trace production, RAN3