3GPP TSG-RAN3 Meeting #129bis                                                    R3-257111

# Prague, Czech Republic, Oct 13 – 17, 2025

Agenda Item:	13.2

Source: 	CEWiT

Title: 	Discussion paper on ISAC network architecture

Document for:	Discussion and Decision

# Introduction

RAN#108 agreed study on ISAC which is updated in the RAN#109 [1]. The study is divided between the evaluation of the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case and study the procedures, signalling between RAN and CN to support ISAC along with network architecture for gNB-based mono-static sensing for UAV sensing target use cases. The latter objective is relevant from RAN 3 perspectives based on the RAN 1 input for the prior objective. This relevant objective regarding for ISAC from RAN 3 perspective is as below,

Study the procedures, signalling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

Further a stage 1 work on ISAC has been completed by SA1 including the use cases and requirement aspects for ISAC. TR 22.837 identifies a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137, in particular, detection and/or tracking of UAV (Uncrewed Aerial Vehicles). SA2 also has an ongoing Rel-20 study on Architecture Enhancement to support Integrated Sensing and Communication (FS_Sensing_ARC, SP-250401). In Release 19, a channel modelling framework for ISAC to enable evaluation of sensing techniques for such use cases was added to the existing communication models in TR38.901.

Considering these aspects and the defined objective, this contribution highlights necessary aspects of the network architecture for ISAC considering mono static sensing as baseline but can be extended to bistatic sensing too.

# ISAC Network architecture

SA2 is working on the different key issues to define the sensing architecture, sensing function, sensing entity, discovery of sensing functions & sensing entity, sensing data collection, sensing parameters configuration provisioning and sensing result exposure. Whereas RAN 1 is working on the physical layer parameter evaluation for sensing purpose. Further the scope of the study limits to mono static sensing for detection and/or tracking of Uncrewed Aerial Vehicles (UAV). It is agreed to extend the same framework to bistatic sensing too without specific enhancement related to it. From RAN 3 perspectives, the interim agreement can be considered for initial discussion purpose and details can be updated once the inputs are available from SA2.

For the initial discussion ISAC perspective following network architecture can be studied.

Figure 1: Network architecture for ISAC

The network architecture for ISAC includes the following components:

- External Client: External Client is external application which requests sensing data. External clients can be authorized or unauthorized.

- NEF: Network exposure function (NEF) performs authorization for application function (AF) (i.e. in case of 3rd party AF) before accepting any sensing service request from AF. This is based on the existing procedure as per TS 23.502 [5].

- SeMF: The Sensing Management Function (SeMF) receives the sensing service request from NEF/AF and performs the authorization of the sensing service request. SeMF also responsible for managing the sensing entity (NG-RAN). SeMF provides the sensing configuration to sensing entities.

- AMF: AMF may maintain the capabilities for the NG-RAN from sensing perspective. Further it may help SeMF to authorize the service requested by AF.

- NG-RAN: It is the sensing entity which performs the sensing measurement estimation/generation task. NG-RAN may have sensing management component (SMC) to execute the sensing task in the NG-RAN.

- Sensing storage entity: It is the entity which stores the sensing result for future tasks including the sensing related analytics estimation, sensing result storage for future usage etc.

Proposal 1: The network architecture for ISAC includes the following components

- External Client

- NEF

- SeMF

- AMF

- NG-RAN

- Sensing storage entity

Proposal 2: For ISAC network architecture, consider figure 1 as baseline.

# Impact on RAN-CN interfaces from RAN perspective:

NG-RAN can be configured to transfer the sensing data using control plane and/or data plane. SA 2 is discussion this in the TR 23.700-14 KI#4. From RAN perspective the impacted interfaces are NG-C (N2), NG-U (N3 or new interface), Xn to coordinate between RANs and sensing measurement component within NR-RAN.

Proposal 3:  From RAN perspective the support of sensing functionality in NG-RAN impacts:

-	NG-C interface: signaling between NG-RAN and AMF

-	NG-U interface: to support the sensing protocol between NG-RAN and SeMF if decided to use direct NG-U equivalent interface. This may be N3 or a new interface.

-	Protocol/signaling between NG-RANs;

-	Architecture of sensing management components within NG-RAN;

# Conclusion

This contribution provides the network architecture based on SA2 architecture related discussions. The contribution provides following proposals for the discussion.

Proposal 1: The network architecture for ISAC includes the following components

- External Client

- NEF

- SeMF

- AMF

- NG-RAN

- Sensing storage entity

Proposal 2: For ISAC network architecture, consider figure 1 as baseline.

Figure 1: Network architecture for ISAC

Proposal 3:  From RAN perspective the support of sensing functionality in NG-RAN impacts:

-	NG-C interface: signaling between NG-RAN and AMF

-	NG-U interface: to support the sensing protocol between NG-RAN and SeMF if decided to use direct NG-U equivalent interface. This may be N3 or a new interface.

-	Protocol/signaling between NG-RANs;

-	Architecture of sensing management components within NG-RAN;

# References

[1] RP-252819: Revised SID: Study on Integrated Sensing and Communication (ISAC) for NR

[2] TR 22.837 : Feasibility Study on Integrated Sensing and Communication (Stage 1)

[3] TS 22.137 : Service requirements for Integrated Sensing and Communication.

Stage 1 (Release 19)

[4] TR 38.901 : Study on channel model for frequencies from 0.5 to 100 GHz  (Release 19)

[5] TS 23.502 : Procedures for the 5G System (5GS)