3GPP TSG-RAN WG3 Meeting #129-bis	R3-257116

# Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	11.2.2

Source:	Ericsson

Title:	MRO for Conditional LTM

Document for:	Discussion and decision

# 1	Introduction

One of the objectives of the SON/MDT Rel-20 WID [RP-252560] is to specify MRO enhancements to the intra-CU conditional LTM cell switch procedure.

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] If needed, co-operate with RAN1, SA5.

This contribution discusses considerations regarding MRO enhancements to this feature. The main differences between LTM and conditional LTM (CLTM) are discussed, along with the various use cases of MRO enhancements for CLTM.

# 2	Discussion

## 2.1. Differences between LTM and CLTM from a network point-of-view

As opposed to the network triggered LTM, conditional LTM is UE triggered. The network configures the UE with the triggering conditions that the UE monitors and triggers CLTM by itself when the conditions are met. This is expected to make the CLTM procedure more robust while still retaining the main attraction of LTM, i.e. short interruption time. However, this also means that the network is not aware of certain aspects associated with the procedure. The detailed signalling steps of CLTM are shown in figure 1.

- In case of CLTM cell switch, the responsibility for triggering its execution is with the   UE rather than the network.

In CLTM, the network configures the UE with a set of candidate cells, beams and TCI states and is sometimes unware of which target cell, beam and TCI state was actually selected from these candidates by the UE for the CLTM cell switch procedure. As a result, some differences may be expected in MRO targeting CLTM, which are discussed ahead. For other aspects that are similar to rel-19 intra-CU LTM, the already agreed RAN3 solutions from rel-19 are proposed for CLTM.

- The network is sometimes unaware of the target cell, beam and TCI state selected by the UE from among the candidates in a CLTM cell switch procedure.

Figure 1: Signalling procedure for intra-CU conditional LTM (inter-DU)

As can be seen from figure 1, other differences between intra-CU LTM and intra-CU CLTM are that in case of CLTM, UE does not send the measurement reports to the network unless configured and there is also no cell switch command sent from the network to the UE.

- In case of CLTM, the UE does not send measurement reports to the network, unless configured.

- In case of CLTM, the network does not send a cell switch command to the UE.

As a result, there are other steps in LTM which are absent in CLTM. Since the source DU does not take the decision to trigger the CLTM cell switch execution, it cannot notify the CU about it, i.e., the DU-CU CELL SWITCH NOTIFICATION F1AP message from the source DU to the CU is not used. As the CU is unaware of the CLTM execution, therefore it does not notify the target DU about it, i.e., the CU-DU CELL SWITCH NOTIFICATION F1AP message from the CU to the target DU is not used.

- The DU-CU CELL SWITCH NOTIFICATION F1AP message from the source DU to the CU is not used in CLTM

- The CU-DU CELL SWITCH NOTIFICATION F1AP message from the CU to the target DU is not used in CLTM.

## 2.2 Use-cases to be supported in Rel-20

As a starting point, the use cases that have already been agreed for rel-19 MRO for intra-CU LTM should also be discussed for MRO for intra-CU CLTM. The use cases include the CLTM cell switch failure cases due to the wrong beam selection, invalid (obsolete TA), CLTM ping-pong, and the case of BFD followed by BFR shortly after successful CLTM, which may be considered as a near-failure case. In view of the differences between the CLTM and LTM procedures discussed before, certain adaptations to these MRO features may also be necessary

- RAN3 to initiate discussions on MRO for intra-CU conditional LTM taking the agreements from rel-19 MRO for intra-CU LTM as the baseline.

### 2.2.1 BFR shortly after successful CLTM cell switch

For rel-19 MRO for intra-CU LTM, it was agreed that the target DU shares the SSB index of the recovery beam with the source DU via the CU in an F1AP message, along with the UE F1AP ID. We believe a similar approach can be followed for CLTM as well as a starting point.

- Upon BFR shortly after successful CLTM, the target DU shares the SSB index of the recovery beam and the UE F1AP ID with the source DU, via the CU.

However, as discussed earlier, the source DU is also unaware of the target cell selected by the UE from among the candidates. Whether an explicit indication of the target cell ID is required or if this is implicitly known needs to be discussed.

- RAN3 to discuss whether indication of the target cell ID is required upon BFR shortly after successful CLTM.

### 2.2.2 Failure due to wrong beam selection

Similar to the arguments presented in section 2.2.1, a similar approach as the rel-19 agreement can be followed in this case, with the additional consideration that the source DU is unaware of the beam that the UE selected from the candidate beams for CLTM cell switch, resulting in failure. Hence, the SSB index of the beam used in CLTM re-establishment or CLTM recovery can be signalled by the target DU to the source DU, via the CU.

- Upon CLTM cell switch failure due to the wrong beam selection by the UE, the target DU sends the SSB index of the beam used in CLTM re-establishment or CLTM recovery and the UE F1AP ID to the source DU, via the CU.

Similar to the discussion in section 2.2.1, the source DU is unaware of the beam used by the UE for the CLTM cell switch, hence, additional beam details such as the target cell ID can be useful for optimization.

- RAN3 to discuss whether an indication of the target cell ID is required upon CLTM cell switch failure due to wrong beam selection.

### 2.2.3 Failure due to invalid TA

Due to the similarity of this use case with the one in rel-19 intra-CU LTM, the agreed RAN3 solution can be reused here. For rel-19 intra-CU LTM, it was agreed to signal the TA value received in RAR (or msgB) upon LTM re-establishment, along with the UE F1AP ID.

- For CLTM cell switch failures due to invalid TA, the target DU shares the TA value received in RAR (or msgB) at CLTM re-establishment, along with the UE F1AP ID with the source DU, via the CU.

### 2.2.4 CLTM cell-switch ping-pong

As with LTM, CLTM cell switches can also experience ping-pong, hence possible solutions should be considered. As a starting point, a solution similar to the one agreed for intra-CU LTM can be considered.

At RAN 3 #129, it was decided that the CU shares the Last Visited LTM Cells list with the DU via the CU-DU CELL SWITCH NOTIFICATION F1AP message for resolving intra-CU LTM ping-pong. The following was agreed:

- Down-select options 1a and 2b. - CU sends relevant information to the DU whenever CU decides this is necessary - CU only send last consecutive LTM cell switch entries to the DU and an optional Mobility Issue indicator

In case of intra-CU CLTM ping-pong, the cells involved in the ping-pong are under the responsibility of the same CU. However, as discussed earlier, the CU-DU CELL SWITCH NOTIFICATION F1AP message is not used in CLTM. However, as seen in figure 1, the CU sends the UE CONTEXT MODIFICATION REQUEST message to both the source and target DUs during CLTM. Hence, we propose that in case of ping-pong, the Last Visited LTM Cells list can be shared via the UE CONTEXT MODIFICATION REQUEST message from the CU to the DU.

- When CU detects CLTM ping-pong, it sends the Last Visited LTM Cells list to the DU via the UE CONTEXT MODIFICATION REQUEST message.

# 3	Conclusion

According to the discussion, we have made the following observations:

Observation 1	In case of CLTM cell switch, the responsibility for triggering its execution is with the   UE rather than the network.

Observation 2	The network is sometimes unaware of the target cell, beam and TCI state selected by the UE from among the candidates in a CLTM cell switch procedure.

Observation 3	In case of CLTM, the UE does not send measurement reports to the network, unless configured.

Observation 4	In case of CLTM, the network does not send a cell switch command to the UE.

Observation 5	The DU-CU CELL SWITCH NOTIFICATION F1AP message from the source DU to the CU is not used in CLTM.

Observation 6	The CU-DU CELL SWITCH NOTIFICATION F1AP message from the CU to the target DU is not used in CLTM.

Based on these observations, we would like to make the following proposals:

Proposal 1	RAN3 to initiate discussions on MRO for intra-CU conditional LTM taking the agreements from rel-19 MRO for intra-CU LTM as the baseline.

Proposal 2	Upon BFR shortly after successful CLTM, the target DU shares the SSB index of the recovery beam and the UE F1AP ID with the source DU, via the CU.

Proposal 3	RAN3 to discuss whether indication of the target cell ID is required upon BFR shortly after successful CLTM.

Proposal 4	Upon CLTM cell switch failure due to the wrong beam selection by the UE, the target DU sends the SSB index of the beam used in CLTM re-establishment or CLTM recovery and the UE F1AP ID to the source DU, via the CU.

Proposal 5	RAN3 to discuss whether an indication of the target cell ID is required upon CLTM cell switch failure due to wrong beam selection.

Proposal 6	For CLTM cell switch failures due to invalid TA, the target DU shares the TA value received in RAR (or msgB) at CLTM re-establishment, along with the UE F1AP ID with the source DU, via the CU.

Proposal 7	When CU detects CLTM ping-pong, it sends the Last Visited LTM Cells list to the DU via the UE CONTEXT MODIFICATION REQUEST message.