3GPP TSG-RAN WG3 Meeting #129bis	R3-257118

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	13.2

Source:	InterDigital, Inc.

Title:	NR ISAC Network Architecture

Document for:	Discussion and Decision

- Introduction

The SID on Integrated Sensing and Communication (ISAC) for NR was approved in RAN#108 [1] and revised at RAN#109 (FS_Sensing_NR_bis) [2]. The objective of the study is to evaluate the performance of intra-gNB monostatic sensing (i.e., single TRP with co-located sensing transmitter and receiver) – RAN1 leads on evaluation and RAN3 addresses network architecture and RAN-CN procedures. This contribution presents technical views for AI 13.2 on architectural assumptions, gNB-internal functional model, and exposure principles aligned with the above scope.

- Architecture baseline

Per the SID [2], the RAN3#129b agenda [3] for AI 13.2 covers network architecture for gNB-based mono-static sensing targeting UAVs, with FR1 prioritised, no UE impacts, and no inter-gNB coordination is in the scope of the study. Applicability to gNB bi-static sensing may be considered only where it does not introduce additional architecture impacts. Coordination with SA2 is expected as needed. The companion agenda item AI 13.3 addresses RAN-CN procedures and signalling [2], [3].

For NR ISAC, the underlying principle is that the sensing is executed in the gNB following the existing CU/DU split. The architecture work is therefore constrained to the NG boundary, with no changes to F1/E1, and no new RAN network functions (NFs) introduced.

Observation-1: Hosting sensing inside the gNB and following the CU/DU split aligns with NG-RAN deployments.

Proposal-1: Network-based sensing in NR should be realized inside the gNB following the existing CU/DU split, without introducing new NFs in the RAN and without F1/E1 interface changes.

- Core sensing functions

For Rel-20 NR ISAC, core network-hosted Sensing Functions (SFs) can be assumed, noting that control and processing analytics may be logically split. Three NG control-plane realizations can be considered in the study. The first is an NGAP-native approach, carried over N2 between the gNB and the AMF, where sensing control and reporting use dedicated NGAP information elements. The second is an indirect realization via the AMF that reuses NGAP aligned mechanisms in an LPP-style flow between the RAN and the CN to the sensing function, also over N2. The third is a new direct NG-RAN↔SF control-plane reference point on the NG interface that provides a control path not going over the AMF.

In all cases, the exposure from RAN to Core Network concerns processed sensing information, for example, measurement-level or object/track-level outputs with associated quality indicators. Carriage of sensing control and results uses a single NG plane: either N2 (NG-c) or N3 (NG-u). Interface details and security/privacy are to be handled under AI 13.3 with SA2 alignment as necessary.

Observation-2: There are three NG control-plane approaches for NR ISAC: NGAP-native over N2, indirect via AMF over N2, and a direct NG-RAN↔SF control-plane reference point.

Proposal-2: Study the following NG control-plane approaches for NR ISAC: NGAP-native over N2, indirect via AMF over N2, and a direct NG-RAN↔SF reference point.

- Monostatic gNB-based sensing functional model

To keep Rel-20 focused and deployment-friendly, architectural work should be constrained to an aggregated (monolithic) gNB realization and to NG-only interaction at the RAN–CN boundary. Under this baseline, control of sensing resources and processing remain gNB-internal without introducing new Network Functions (NFs). The design should follow the existing CU/DU split and no F1/E1 changes.

In the DU, physical-layer processing can extract reflections from NR reference signals during sensing occasions and derive sensing measurements associated with those occasions. Measurement examples include delay, angles of arrival/departure, Doppler shift, received power/SINR, RCS, micro-Doppler, etc. The exact measurement set and estimation methods are implementation-dependent.

Within the CU (anchored at the CU-CP), a sensing resource controller can configure and schedule sensing occasions using NR reference signals. It can coordinate beams, time and frequency with the communications scheduler. Scheduling policies are down to implementation.

Also in the CU, a fusion and estimation function can associate detections, filter and track them over time, and maintain gNB-local sensing sessions that output object/track-level states with confidence indicators. State representation and filtering choices are also down to implementation.

Observation-3: Sensing measurements can be performed in the DU. Configuration and scheduling of sensing occasions, as well as fusion and session management can be hosted in CU.

Proposal-3: Place sensing measurement derivation in the DU. The CU should handle configuration and scheduling of sensing occasions, fusion, and session management.

- Sensing exposure

When exposure beyond the gNB is required, a CU-resident function can provide sensing outputs toward core network functions and application functions (AFs) via the core network. The exposed content can be processed information at measurement level and/or object/track level, which can be accompanied by quality indicators such as timestamps, uncertainty, and validity.

Carriage of sensing control and results uses a single NG plane: either N2 (NG-c) with NGAP-native information elements or NGAP-container, or N3 (NG-u) with GTP-U. Payload packaging follows normal NG conventions, and mapping is independent of DU/CU implementation.

Exposure carries the usual policy and security hooks: authorization context, scope and retention hints, and optional redaction/anonymization. Admission control and rate limits apply across concurrent sensing tasks. If reports are thinned or delayed, the included quality indicators reflect any resulting loss of confidence so downstream functions can react appropriately.

Observation-4: Exposure of processed results/measurements with associated quality indicators is carried on a single NG plane—either N2 (NG-c) or N3 (NG-u).

Proposal-4: Study carriage on a single NG plane: NG-c/N2 (NGAP-native or NGAP-container) or NG-u/N3 (GTP-U). Details of information elements/containers and security/privacy are handled in the procedures work.

- Conclusions

This paper made the following proposals regarding RAN architecture for enabling NR ISAC:

Proposal-1: Network-based sensing in NR should be realized inside the gNB following the existing CU/DU split, without introducing new NFs within the RAN and without F1/E1 interface changes.

Proposal-2: Study the following NG control-plane approaches for NR ISAC: NGAP-native over N2, indirect via AMF over N2, and a direct NG-RAN↔SF reference point.

Proposal-3: Place sensing measurement derivation in the DU. The CU should handle configuration and scheduling of sensing occasions, fusion, and session management.

Proposal-4: Study carriage on a single NG plane: NG-c/N2 (NGAP-native or NGAP-container) or NG-u/N3 (GTP-U payload encapsulation). Details of information elements/containers and security/privacy are handled in the procedures work.

- References

[1] RP-251861, “New SID: Study on Integrated Sensing And Communication (ISAC) for NR,” RAN#108, Prague, Czech Republic, June 2025.
[2] RP-252819, “Study on Integrated Sensing And Communication (ISAC) for NR (FS_Sensing_NR_bis),” RAN#109, Beijing, China, September 2025.
[3] R3-20250921, “RAN3#129b Agenda”, Prague, Czech Republic, October 2025.