3GPP TSG-RAN WG3 Meeting #129bis	R3-257122

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	10.4

Source: 	LG Electronics Inc.

Title: 	Considerations on 6G RAN internal functional split

Document for:	Discussion and Decision

# Introduction

6G Study (Rel-20) for RAN WGs was agreed at RAN#108 and further revised in RAN#109 [1]. RAN3 needs to lead the study on the objective (6) Radio Access Network architecture, interface protocols and procedures:

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items. From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] //////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In line with the objective (6), for the RAN architecture principles, we have proposed in [2] as follows:

Proposal 1: The design for 6G RAN architecture shall follow the principle of functional abstraction. Proposal 2: The 6G RAN architecture shall be specified as a collection of logical RAN functions with their standardized behaviours and interactions. Proposal 3: The 6G RAN architecture shall support a unified framework for RAN functions: - The 6G RAN architecture shall define RAN functions in a virtualized and modular way, allowing RAN functions to be deployed, scaled, or removed according to network needs. - The 6G RAN architecture shall ensure consistent resource management and lifecycle management across all RAN functions. - The 6G RAN architecture shall be designed to streamline feature development and facilitate easier standardization based on modularized RAN functions.

In line with these proposals for 6G RAN architecture principles, and considering the RAN#109 progress on 6G requirements for architecture and migration [3][4], in this contribution, we share some key considerations and proposals to guide the study of 6G RAN internal functional split.

# Discussion

In 5G, TS 38.300 [5] defined a list of gNB functions that it hosts and shall support:

## 4.2 Functional Split The gNB and ng-eNB host the following functions: - Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling); - IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data; - Selection of an AMF at UE attachment when no routing to an AMF can be determined from the information provided by the UE; - Routing of User Plane data towards UPF(s); - Routing of Control Plane information towards AMF; - Connection setup and release; - Scheduling and transmission of paging messages; - Scheduling and transmission of system broadcast information (originated from the AMF or OAM); - Measurement and measurement reporting configuration for mobility and scheduling; - Transport level packet marking in the uplink; - Session Management; - Support of Network Slicing; - QoS Flow management and mapping to data radio bearers; - Support of UEs in RRC_INACTIVE state; - Distribution function for NAS messages; - Radio access network sharing; - Dual Connectivity; - Tight interworking between NR and E-UTRA; - Maintain security and radio configuration for User Plane CIoT 5GS Optimisation, as defined in TS 23.501 [3] (ng-eNB only). NOTE 1: BL UE or UE in enhanced coverage is only supported by ng-eNB, see TS 36.300 [2]. NOTE 2: NB-IoT UE is only supported by ng-eNB, see TS 36.300 [2]. The AMF hosts the following main functions (see TS 23.501 [3]): //////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// This is summarized on the figure below where yellow boxes depict the logical nodes and white boxes depict the main functions. Figure 4.2-1: Functional Split between NG-RAN and 5GC

With respect to control-plane functionalities (e.g. radio bearer control, mobility control, radio admission control, etc.), once a gNB was disaggregated into multiple logical nodes (CU-CP, CU-UP, and DU), the mapping of those functions to logical nodes became fragmented because the split was driven by the protocol-oriented decisions [7]. The gNB-DU hosts the lower layers of RLC, MAC and PHY while the gNB-CU hosts the upper layers of PDCP and above, further segregated into gNB-CU-CP and gNB-CU-UP stacks.

Figure 1: 5G gNB – Logical functions and Split architecture

As a result, in a disaggregated gNB architecture, a control-plane operation often had to interact across those components, each responsible for its own resources and protocol stacks for UEs. Almost all internal control-plane procedures required tightly intertwined point-to-point signalling over F1 and E1 between logical nodes. It is easy to observe from [6] that many control decisions start in CU-CP, then sequentially involve CU-UP and DU, then CU-UP again to complete a single operation that otherwise could have been done by a simple function call within a gNB. Note that in a monolithic gNB, these functions are all implemented within a single logical node, and there was no fragmented mapping of functions or complex signalling overhead involved.

We are not saying that the signalling overhead due to internal interactions is the primary problem that we should avoid in 6G. To some extent, as RAN evolves toward functional abstraction, interactions between functional building blocks within RAN are inevitable and this is kind of a price to pay for deployment flexibility due to hardware-software decoupling and virtualization. However, what we observe is that the early study we did in 5G [7] focused on identifying which parts of the protocol flow with a UE could be realistically pooled in NW domain (i.e. centralized), rather than defining the building blocks to be modular so that they can be deployed, added, or removed according to network needs. We believe that such design choice at 5G is the fundamental cause of the operational and architectural complexity observed in 5G RAN, such as (1) lack of flexibility because adding a new function often required updating multiple logical nodes and interfaces at the same time; (2) difficulty to optimize or debug as a feature is spread across logical nodes; (3) non-intuitive ownership of functions and unclear boundaries between them, etc.

Having this in mind, it is also evident that the Uu protocol stacks will remain an integral component of 6G RAN for communications with UEs. While it is unclear yet how the 6G Uu stacks would evolve and under study, it is clear that 6G RAN shall serve the UEs and terminate control/user plane Uu protocols. However, we are already well aware that using the Uu protocol stack as an anchor for functional decomposition of RAN led to fragmented mappings of RAN functions in 5G. From this, in 6G, we believe that the Uu protocol stacks should no longer serve as the reference point for defining internal RAN functional splits or interactions.

Instead, the internal RAN functional split shall be guided by the principle of functional abstraction, defining RAN functions as self-contained and modularized with clear roles, behaviours, and interactions. The Uu protocol stack should be treated as a baseline implementation required for UE communication within 6G RAN, separate from the definitions of RAN functions. We believe that such decoupling is critical in enabling 6G RAN to evolve toward a service-plane abstraction where each function can be requested, executed, and managed as a standalone service rather than fragmented pieces distributed across multiple logical nodes.

Proposal 1: The 6G RAN internal functional split shall follow the principle of functional abstraction, so that each RAN function can be requested, executed, and managed as a self-contained function rather than fragmented pieces distributed across multiple logical nodes.

Proposal 2: RAN3 to study the feasibility of defining the 6G RAN internal functional split independent of the Uu protocol stack, to avoid the fragmented mapping occurred in 5G and to enable clearer functional roles and interactions.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: The 6G RAN internal functional split shall follow the principle of functional abstraction, so that each RAN function can be requested, executed, and managed as a self-contained function rather than fragmented pieces distributed across multiple logical nodes.

Proposal 2: RAN3 to study the feasibility of defining the 6G RAN internal functional split independent of the Uu protocol stack, to avoid the fragmented mapping occurred in 5G and to enable clearer functional roles and interactions.

# Reference

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R3-257121, “Design principles for 6G RAN architecture”, LG Electronics Inc.

[3] RP-252880, “Summary of Discussion on 6G Requirements for architecture and migration”,  NTT DOCOMO

[4] RP-252870, “pCR for 38.914 on requirements for architecture and migration for 6G”, NTT DOCOMO, INC. (Moderator)

[5] TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

[6] TS 38.401, NG-RAN; Architecture description

[7] TR 38.801, Study on new radio access technology: Radio access architecture and interfaces (Rel-14)