3GPP TSG-RAN WG3 Meeting #129bis	R3-257123

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	13.2

Source: 	LG Electronics Inc.

Title: 	Discussions for ISAC network architecture in RAN3

Document for:	Discussion and Decision

# Introduction

Rel-20 ISAC study (5G Advanced) for RAN WGs was agreed at RAN#108 and further revised in RAN#109 [1], with the following objectives:

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In Rel-19, various ISAC-related use cases were studied by SA1 and captured into TR 22.837 [2], and their corresponding service requirements were specified in TS 22.137 [3]. In parallel, RAN has been conducting studies on channel modeling for ISAC (FS_Sensing_NR, RP-242348 [4]), and SA2 recently initiated a Rel-20 study (FS_Sensing_ARC, SP-250833) to enhance the system architecture and 5GC for sensing, targeting for completion in December 2025.

So far, SA2 has identified the following 6 key issues and captured 39 solutions in TR 23.700-14 [5]:

## Key Issue #1: System Architecture to Support Sensing This key issue will address the general system architecture and function enhancements required to provide end-to-end support for Sensing. The general architecture aspects that need to be studied include any new and/or existing functions in the Core Network and their interactions that should be involved in providing support for Sensing. Potential privacy requirements if identified will be considered for this key issue. NOTE: Solutions for other key issues may also impact general architecture aspects covered by this key issue. ## Key Issue #2: Authorization and Revocation to Support Sensing Service The key issue is to address the authorization and revocation for a particular Sensing Service, specifically: - authorization of Sensing Service requests and revocation of an on-going Sensing Service; and - authorization and revocation for the Sensing Entities if applicable. For the authorization of the Sensing Service request, and the revocation of an on-going Sensing Service, the study needs to address the following aspects: - whether and how to perform Sensing Service request authorization if required, and the criteria for the Sensing Service request authorization, based on the requirements from TS 22.137 [2]; - whether and how to perform Sensing Service revocation if required, and the criteria for the revocation, based on the requirements from TS 22.137 [2]; - whether, how and which entity(/entities) performs the Sensing Service request authorization and Sensing Service revocation if required; - whether and where to store the Sensing Service request authorization information and Sensing Service revocation information, and when and how to update the authorization and revocation information if required. For the authorization and revocation for the Sensing Entities if applicable, the study needs to address the following aspects: - whether, when and which entity performs the authorization and revocation of the Sensing Entity(/Entities); - if needed, the granularity of the Sensing Entity authorization, e.g. based on TS 22.137 [2]; - if needed, storage and management of the authorization information for the Sensing Entities. NOTE: The authorization and revocation for the Sensing Entities can be related to the key issue #3 on Sensing Entities discovery and selection. ## Key Issue #3: Sensing Entity and Sensing Function Discovery and (Re-)Selection This key issue aims to study how to perform the Sensing Entity(ies) and Sensing Function(s) discovery and (re-)selection, in particular: - When required, for the discovery and (re-)selection of the Sensing Entity(ies), the study needs to address the following aspects: - Which entity(ies) performs the discovery and (re-)selection of the Sensing Entity(ies); - What the information and/or criteria are taken into account for the discovery and (re-)selection of the Sensing Entity(ies); and - The procedure(s) for the discovery and (re-)selection of the Sensing Entity(ies). - When required, for the discovery and (re-)selection of the Sensing Function(s), the study needs to address the following aspects: - Which entity(ies) performs the discovery and (re-)selection of Sensing Function(s); - What the information and/or criteria are taken into account for the discovery and (re-)selection of Sensing Function(s); and - The procedure(s) for the discovery and (re-)selection of the Sensing Function(s). ## Key Issue #4: Sensing Data and the Associated Information Collection and Transport Integration of sensing and communication requires 3GPP network to collect and transfer sensing data and the associated information to a Sensing Function. This Key Issue aims to study: - How to collect sensing data (and the associated information, if any) for Sensing result generation? - Which Sensing Function performs Sensing result generation? - Whether and what the above associated information is required for Sensing result generation NOTE: Implications to RAN or RAN dependent aspects will be coordinated with RAN WGs. ## Key Issue #5: Sensing Result Exposure This key issue addresses the exposure of Sensing results to support Sensing Services. The aspects to be studied in this key issue include: - Study what types of Sensing results with or without the Sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the Sensing Service Consumer. - How the Core Network exposes Sensing results to the Sensing Service Consumer (e.g. one time, periodic or event trigger). ## Key Issue #6: Configuration of Parameters for Sensing Entities This Key Issue will study how parameter configuration for Sensing Entities is performed. In particular, this Key Issue will study: - How and what parameters need to be exchanged between the Sensing Entities and the Core Network. NOTE: This KI needs to be aligned with RAN WGs.

In line with the SA2 progress, RAN3 needs to focus on the network architecture design for gNB-based mono-static sensing for UAV sensing use cases, which is related to SA2 key issue #1. Considering progresses conducted in various working groups for sensing, in this first meeting, we provide several proposals that we believe important to guide our study on the design of ISAC network architecture in RAN3.

# Discussion

## Unified framework

The first and foremost question for discussion is whether our architecture design for overall signalling flow and interactions between gNB and 5GC should be different depending on the sensing type (e.g. mono-static vs bi-static) or the specific use case.

Currently, Rel-20 focuses on gNB-based mono-static sensing and targets the UAV use case. However, this scope was constrained mainly due to practical considerations and limited study timeframe. The gNB-based bi-static sensing, where sensing transmission and reception are executed by different gNBs, was excluded from RAN1 evaluation and may be discussed in RAN3 at a later phase only from architecture perspective. The inter-gNB coordination was also excluded due to its architectural and procedural complexity, especially given that the mono-static sensing alone presents significant evaluation workload, and also considering that the role of this study is to lay the feasibility groundwork for ISAC in future 6G standardization.

From similar reasons, among the various sensing use cases defined in SA1’s TR 22.837 [2], UAVs were selected as the representative (and “only”) use case for study under Rel-20 ISAC in RAN, considering its practical tractability, technical feasibility, and standardization efficiency:

- UAVs provide well-defined targets for sensing system design. Compared to humans or vehicles, which involves complex, multi-path characteristics, UAVs are relatively simple physical objects, enabling easier modelling, performance metric definition and calibration, thus making them well-suited for mono-static sensing.

- UAVs were already considered in Rel-19 study (FS_Sensing_NR [4]) for channel model calibration, making them readily usable in Rel-20 RAN1 simulations. Moreover, UAVs are closely tied to growing industrial applications (aerial surveillance, disaster response, and autonomous delivery, etc.), which ensures clear market demands.

Moreover, needless to say, “Sensing” is an integral component of 6G RAN design, and UAV represents one of the most promising use cases to be studied in 6G [6]. For 6G, UAV use cases are also being addressed in TR 22.870 [7] and referred as “Uncrewed Aerial Vehicles”, as part of SA1’s New Study on 6G Use Cases and Service Requirements (FS_6G-REQ, SP-241391).

Furthermore, from a sensing function point of view, regardless of use cases, the purpose and operational requirements can be categorized into four fundamental objectives: (1) Detection (2) Localization (3) Tracking (4) Classification, to which most sensing use case can be mapped onto.

Given these considerations, we believe that our design for overall signalling flow and interactions between gNB and 5GC for sensing is better to be forward compatible and applicable the same regardless of which sensing type is used or which use case is considered.

Proposal 1: RAN3 designs the ISAC network architecture and signalling interactions between gNB and 5GC in a unified way that is agnostic to sensing type (e.g. mono-static vs bi-static) or a specific use case.

## Terminologies of SE (gNB) and SF (residing at 5GC)

One important progress observed from SA2’s study is about the “terminology” to be used to define the overall network architecture [5]:

## 3.1 Terms For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1]. Sensing Service Consumer: The entity that may consume the Sensing Result. The Sensing Service Consumer may also request the Sensing Result. Sensing Entity: The Sensing Entity referring to a Sensing Transmitter and/or to a Sensing Receiver. NOTE 1: In this release, the Sensing Entity is only referring to gNB. Sensing Function: Indicating the logical function which is involved to support Sensing Service. NOTE 2: The Sensing Function cannot be a Sensing Entity. Sensing Service: Capability to collect and provide information about object and/or characteristics of the environment using radio signals. gNB-based Sensing: it indicates gNB acting as Sensing Entity. This doesn't imply any assumptions for the location of the sensing result generation. The terms defined in TS 22.137 [2] apply to this study: - Sensing assistance information. - Sensing contextual information. - Sensing result. - Target sensing service area. - Sensing receiver. - Sensing transmitter. Editor's note: The terminology will be further revised during the study work, if needed.

Of course, they are subject to be revised during their study. But it is clear that SA2 already introduced two key concepts by sensing entity (SE) and sensing function (SF), where the former is to define the entity that executes sensing measurement operations (i.e. sensing signal transmission or reception), while the latter is to define a logical function to support and orchestrate sensing services in 5G.

In Rel-20 for 5G Advanced, SA2 currently assumes that a SE can only be a gNB, and SF cannot be a SE. But it is unclear whether a SF (normally assumed to reside in 5GC) can also be deployed at gNB. However, RAN has already agreed not to study inter-gNB coordination in Rel-20. If the network architecture for gNB-based sensing allows a SF to be located in gNB (not acting as a SE), it may require inter-gNB coordination between the gNB hosting SF and another gNB acting as an SE. As a result, it will be reasonable to assume that, in Rel-20, a SF can only be located in 5GC, which also aligns with many solutions captured for key issue #1 in [5].

Proposal 2: RAN3 confirms to study that a Sensing Entity (SE) is performed only by a gNB.

Proposal 3: RAN3 agrees that, in Rel-20, Sensing Function (SF) shall not be located at gNB, as it may require inter-gNB coordination with another gNB acting as a Sensing Entity (SE).

## Sensing CN Node

One other aspect to discuss is how we capture the entity in 5GC that SE (gNB) interacts with. Depending on solutions captured in SA2 [5], SE (gNB) and SF (in 5GC) could be directly interfaced, or direct interface may only be setup for sensing measurement configuration or reporting purpose, or AMF may act in the middle for bridging between them while performing some discovery or selection of SE instead of SF.

Regardless of which solution is adopted, from our architecture standpoint, it is evident that SE (gNB) interacts with 5GC over NG interface for sensing, either directly with a SF or indirectly via AMF. It may affect our stage-3 signalling design at a later stage, but from the architecture perspective, RAN3 does not need to waste time on how we should capture the entity that SE (gNB) interacts with for ISAC. For this, we can follow what Rel-19 A-IoT defined for CN node by “A-IoT CN node” (as captured in TS 38.300 by R2-2506490):

3.2 Definitions For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2]. 2Rx XR UE: two antenna port XR UE as specified in TS 38.101-1 [18]. A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, i.e. BRID or DAA. Access occasion: a time-frequency resource for A-IoT device(s) to transmit A-IoT MSG1 (i.e., the Random ID message) during an A-IoT CBRA procedure. Aerial UE communication: functionality enabling Aerial UE function, as defined in 16.18. A-IoT CN node: The core network node which the gNB connects to 5GC for A-IoT operation. More specifically, it is the AIOTF in case the gNB directly connects with the AIOTF, and it includes both the AMF and AIOTF in case the gNB connects with the AIOTF via AMF. A-IoT device: a device that supports A-IoT radio interface towards A-IoT reader, as defined in 16.x. A-IoT MSG1: first D2R message transmission in the A-IoT CBRA procedure, as defined in 16.x. A-IoT MSG2: R2D message in response to A-IoT MSG1 in the A-IoT CBRA procedure, as defined in 16.x. A-IoT reader: reader providing A-IoT protocol terminations towards the A-IoT device, as defined in 16.x. Air to Ground network: An NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000m and take-off/landing altitudes down to 3000m.

Proposal 4: RAN3 to capture the entity in 5GC that SE (gNB) interacts with, by defining “Sensing CN node” with the following definition (similar to “A-IoT CN node” in Rel-19):

- Sensing CN node: The core network node which the gNB connects to 5GC for sensing operation. More specifically, it is the SF (Sensing Function) in case the gNB directly connects with the SF, and it includes both AMF and SF in case the gNB connects with the SF via AMF.

## Basic signalling flow for RAN-CN sensing service cycle

Based on the above reasonings with SE (gNB), SF (residing at 5GC) and the new definition of “Sensing CN node”, the basic signalling flow focusing on the sensing service cycle (request, execution, report) can be described as follows:

- Sensing service consumer requests a sensing service to SF (no need to show in RAN3).

- “Sensing CN node” selects an appropriate gNB (SE).

- “Sensing CN node” requests the sensing execution to the selected gNB (SE).

- The gNB (SE) executes the sensing operations.

- The gNB (SE) reports the result back to “Sensing CN node”.

- “Sensing CN node” delivers the sensing result to the consumer. (no need to show in RAN3)

Proposal 5: RAN3 to capture the following figure as the baseline design for the sensing service cycle and interactions between gNB and Sensing CN node for Rel-20 ISAC network architecture study.

Figure 1: Signalling flow focusing on RAN-CN sensing service cycle (request, execution, report)

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: RAN3 designs the ISAC network architecture and signalling interactions between gNB and 5GC in a unified way that is agnostic to sensing type (e.g. mono-static vs bi-static) or a specific use case.

Proposal 2: RAN3 confirms to study that a Sensing Entity (SE) is performed only by a gNB.

Proposal 3: RAN3 agrees that, in Rel-20, Sensing Function (SF) shall not be located at gNB, as it may require inter-gNB coordination with another gNB acting as a Sensing Entity (SE).

Proposal 4: RAN3 to capture the entity in 5GC that SE (gNB) interacts with, by defining “Sensing CN node” with the following definition (similar to “A-IoT CN node” in Rel-19):

- Sensing CN node: The core network node which the gNB connects to 5GC for sensing operation. More specifically, it is the SF (Sensing Function) in case the gNB directly connects with the SF, and it includes both AMF and SF in case the gNB connects with the SF via AMF.

Proposal 5: RAN3 to capture the following figure as the baseline design for the sensing service cycle and interactions between gNB and Sensing CN node for Rel-20 ISAC network architecture study.

Figure 1: Signalling flow focusing on RAN-CN sensing service cycle (request, execution, report)

# Reference

[1] RP-252819, New SI: Study on Integrated Sensing And Communication (ISAC) for NR (RAN1-led)

[2] TR 22.837, Study on Integrated Sensing and Communication (Rel-19), SA1

[3] TS 22.137, Integrated Sensing and Communication (Rel-19), SA1

[4] RP-242348, Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR

[5] TR 23.700-14, Study on Stage 2 for Integrated Sensing and Communication (Rel-20), SA2

[6] RP-252912, SID: Study on 6G Radio

[7] TR 22.870, Study on 6G Use Cases and Service Requirements (Rel-20), SA1