3GPP TSG-RAN WG3 Meeting #129-bis	R3-257146

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	Essential corrections to XnAP for inter-CU LTM

Agenda Item:	9.2.5

Source:	Huawei, Jio Platforms, CMCC, Lenovo

Document for:	Discussion

1	Introduction

The Rel-19 Mobility Enhancement WI was completed at last meeting.

This contribution provides a couple of essential corrections to the XnAP part for inter-CU LTM.

2	Discussion

2.1	Refinement to Handover Request procedure

A mandatory LTM Indicator IE in LTM Information Setup IE was introduced in the F1AP UE Context Request and UE Context Modification in Rel-18 intra-CU LTM to explicitly indicate the gNB-DU about the LTM preparation. In principle, the same IE should be introduced in the Handover Request procedure for inter-CU LTM to have a harmonized design with F1AP.

Similarly, in F1AP for intra-CU LTM, a mandatory Request for RACH Configuration IE in Early Sync Information Request IE was also introduced in the UE Context Request and UE Context Modification for early RACH resource reservation. Therefore, in XnAP, for inter-CU LTM, a mandatory Early Sync Configuration Request IE is introduced in the Early Sync Information Request IE in the HANDOVER REQUEST message.

However, the LTM Indicator IE is still missing in the HANDOVER REQUEST message.

Proposal 1: To add a LTM Indicator IE in the HANDOVER REQUEST message for inter-CU LTM.

2.2	Refinement to LTM Configuration Update procedure

Removal of Reference Configuration

An optional Reference Configuration IE is added in the LTM CONFIGURATION UPDATE message. In response, the LTM Candidate Configuration IE and an optional Complete Candidate Configuration IE are added in the LTM CONFIGURATION UPDATE ACKNOLEDGE message.

The intention is to allow the candidate gNB to provide the LTM related RRC configurations based on the indicated reference configuration in the LTM CONFIGURATION UPDATE message. However, this function is not needed and needs to remove, because RAN3 agreed at last meeting that the source gNB is not allowed to fetch a reference configuration from any candidate gNBs by the Handover Request procedure. Hence, there is no any scenario to allow the source gNB to ask the candidate gNB to provide the LTM related RRC configurations in the second round preparation, i.e., in the LTM  Configuration Update.

In consequence, the Reference Configuration IE in the LTM CONFIGURATION UPDATE message and the Complete Candidate Configuration IE in the LTM CONFIGURATION UPDATE ACKNOLEDGE message should be removed. Note that the LTM Candidate Configuration IE in the LTM CONFIGURATION UPDATE ACKNOLEDGE should be kept as it such, as it may be used to carry the generated CSI report configuration by the candidate gNB based on the common CSI resource configuration.

Proposal 2: To remove the Reference Configuration IE in the LTM CONFIGURATION UPDATE message and the Complete Candidate Configuration IE in the LTM CONFIGURATION UPDATE ACKNOLEDGE message.

Handling last NG-RAN node UE XnAP IDs

In inter-CU LTM, there was agreement that “The old source gNB can deliver the old target UE XnAP ID(s) to the new serving gNB via Cell Switch Notification and LTM Configuration Update message.” In consequence, the last Target NG-RAN node UE XnAP ID IE for all candidate cells is included in the LTM CONFIGURATION UPDATE message as shown below.

>>Last Target NG-RAN node UE XnAP ID; O; NG-RAN node UE XnAP ID 9.2.3.16; –.

Transferring the last Target NG-RAN node UE XnAP IDs in the LTM CONFIGURATION UPDATE message was agreed for the LTM failure case. In which it assumes that a HANDOVER SUCCESS will be sent to the source gNB from the candidate gNB serving the new target cell where the UE performs fast LTM recovery.

However, such assumption is not correct. As based on RAN2 design, fast LTM recovery can be performed only in the candidate cells with the same LTM No Security Change ID as the source cell ( case 1: CELL SWITCH NOTIFICATION is not sent to the target gNB) or with the same LTM No Security Change ID as the target cell (case 2: CELL SWITCH NOTIFICATION is sent to the target gNB).

For case 1, there is no need to transfer the last Target NG-RAN node UE XnAP IDs of all candidate cells to the new serving gNBs, as the fast LTM recovery cell is in the source gNB. If the UE reconnects to another candidate gNB, the correct procedure should be RRCReestablishment. In this case, XnAP Retrieval UE Context procedure is applied. And the LTM preparation in the candidate gNBs has to be cancelled or released.

For case 2, there is no need to transfer the last Target NG-RAN node UE XnAP IDs of all candidate cells to the new serving gNBs once again either, as the CELL SWITCH NOTIFICATION has been already sent to the new serving gNBs in which contains the last Target NG-RAN node UE XnAP IDs.

Therefore, it is proposed:

Proposal 3: To remove the last Target NG-RAN node UE XnAP IDs of all candidate cells in the LTM CONFIGURATION UPDATE message.

Retrieve UE Context in the candidate gNB by LTM Configuration Update

One of the purpose of the LTM Configuration Update procedure is after every LTM cell switch to allow the new serving gNB to inform the other candidate gNBs about the LTM cell switch event for a concerned UE with LTM prepared in them.

So, functionally, the candidate gNB needs to retrieve the context of the LTM prepared UE. However, the old Target UE XnAP ID which has been agreed in the WI phase is still missing in the LTM CONFIGURATION UPDATE message. And furthermore, the following clarification was made to the old Target UE XnAP ID.

“To clarify that the “old target UE XnAP ID” is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch e.g., when receiving the LTM Configuration Update Request message from the new source gNB.”

Proposal 4: To introduce a node level new IE, named as Last Target NG-RAN node UE XnAP ID IE in the LTM CONFIGURATION UPDATE message to allow the candidate gNB to retrieve the UE context.

2.3	Refinement to CSI-RS coordination procedure

There are some redundant IEs in the CSI-RS Coordination procedure. The CSI-RS Resources for L1 measurements and for CSI acquisition are indicated separately. This is not needed from function perspective as the CSI resource configuration indicated by the CSI resource configuration ID can already distinguish the purpose or the CSI-RS resources.

In principle, the IE design in this procedure should be aligned between XnAP and F1AP.

Proposal 5: To remove redundant IEs in the CSI-RS Coordination procedure in XnAP (i.e., align with F1AP).

2.4	Miscellaneous

Moreover, there are some miscellaneous changes in the corresponding XnAP CR including rewording to the procedural text, correction of IE names, etc.

The first critical one is that the TCI States Configurations List IE is missing in the Early Sync Information IE in XnAP, which was introduced in Rel-18 intra-CU LTM mandatorily in F1AP.

Also the following procedural text is added in the interaction part in the Handover Success section to cover the interaction between the Handover Success procedure and the Cell Switch Notification procedure in case of fast LTM recovery.

Interactions with other procedures If a CONDITIONAL HANDOVER CANCEL message was received for this UE prior the reception of the HANDOVER SUCCESS message, the source NG-RAN node shall consider that the UE successfully executed the handover. The source NG-RAN node may initiate Handover Cancel procedure towards the other signalling connections or other candidate target NG-RAN nodes for this UE, if any. If a HANDOVER SUCCESS is received from the target NG-RAN node to which a CELL SWITCH NOTIFICATION is sent, and the Requested Target Cell ID IE contained in the HANDOVER SUCCESS message is not the target cell indicated in the corresponding CELL SWITCH NOTIFICATION message sending to that target NG-RAN node, the source NG-RAN node shall consider that the UE has performed fast LTM recovery successfully in the cell indicated by the Requested Target Cell ID IE.

Proposal 6: RAN3 to agree on the miscellaneous changes in the corresponding XnAP CR.

3.	Conclusion

This contribution discusses some essential corrections to the XnAP part for inter-CU LTM and proposes:

Proposal 1: To add a LTM Indicator IE in the HANDOVER REQUEST message for inter-CU LTM.

Proposal 2: To remove the Reference Configuration IE in the LTM CONFIGURATION UPDATE message and the Complete Candidate Configuration IE in the LTM CONFIGURATION UPDATE ACKNOLEDGE message.

Proposal 3: To remove the last Target NG-RAN node UE XnAP IDs of all candidate cells in the LTM CONFIGURATION UPDATE message.

Proposal 4: To introduce a node level new IE, named as Last Target NG-RAN node UE XnAP ID IE in the LTM CONFIGURATION UPDATE message to allow the candidate gNB to retrieve the UE context.

Proposal 5: To remove redundant IEs in the CSI-RS Coordination procedure in XnAP (i.e., align with F1AP).

Proposal 6: RAN3 to agree on the miscellaneous changes in the corresponding XnAP CR.

The XnAP CR is provided in [1].

4 	Reference

[1] R3-257147	Essential corrections to XnAP for inter-CU LTM