3GPP TSG-RAN WG3 Meeting #129-bis	R3-257148

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	Exchange of early RACH Resources Requester ID for inter-CU LTM

Agenda Item:	9.2.5

Source:	Huawei, Jio Platforms, CATT, CMCC, China Telecom, Lenovo

Document for:	Discussion

1	Introduction

The Rel-19 Mobility Enhancement WI was completed at last meeting.

This contribution focuses on the issue of early RACH Resources Requester ID exchange for inter-CU LTM and provides a solution for RAN3 consideration.

2	Discussion

Normally, the candidate gNB needs to reserve PRACH resource for UE early UL synchronization towards the candidate cells per source gNB-DU in inter-CU LTM. In order not to expose the NG-RAN internal architecture, a new identifier named as early RACH Resources Requester ID is introduced in the HANDOVER REQUEST message to avoid transferring the gNB-DU ID over XnAP. And the mapping between the early RACH Resources Requester ID and the gNB-DU ID in a gNB is up to implementation.

IE/Group Name; Presence; Range; IE type and reference; Semantics description. Early Sync Configuration Request; M; ENUMERATED (true, …). Early RACH Resources Providers List; 0..1. >Early RACH Resources Item; 1..< maxnoofEarlyRACHResourcesID>. >>Global gNB ID; M; 9.2.2.1. >>Early RACH Resources Requester ID; M; 9.2.3.220; Identifies the entity requesting Early RACH resources..

The problem is that the source gNB has no any knowledge about the early RACH Resources Requester IDs of other candidate gNBs. Therefore, it cannot fill in the early RACH Resources Requester ID IE correctly in the HANDOVER REQUEST message.

In order to allow the source gNB to provide correct Early RACH Resources Requester IDs of other candidate gNBs in the HANDOVER REQUEST message, the straight forward method is to exchange them between neighboring gNBs during Xn Setup and NG-RAN node configuration update.

One may think that we may reuse the HANDOVER REQUEST message to fetch the Early RACH Resources Requester IDs of other candidate gNBs and then ask the candidate gNB to provide early RACH resources in the second round LTM preparation, i.e., in LTM CONFIGURATION UPDATE message. This approach is not workable because it may need a third sync up step to circulate the obtained early RACH resources among all other candidate gNBs.

The early RACH Resources Requester ID needs to be included in the Serving Cell Information NR IE to allow the source gNB to identify the correct early RACH Resources Requester ID of each candidate cell in other gNBs.

Proposal: To exchange the early RACH Resources Requester IDs in the Served Cell Informationn IE between neighbouring gNBs during Xn Setup and NG-RAN node configuration update for support of inter-CU LTM.

3.	Conclusion

This contribution discusses the issue of early RACH Resources Requester ID exchange for inter-CU LTM and proposes:

Proposal: To exchange the early RACH Resources Requester IDs in the Served Cell Informationn IE between neighbouring gNBs during Xn Setup and NG-RAN node configuration update for support of inter-CU LTM.

The CR is provided in [1].

4 	Reference

[1] R3-257149	Exchange of early RACH Resources Requester ID for inter-CU LTM