3GPP TSG-RAN WG3 Meeting # 129bis	R3-257169

# Prague, Czech Republic, 13th – 17th October 2025

Agenda item:	10.4

Source:	Google

Title:	Discussion on supporting a F1-like interface in 6G RAN

Document for:	Discussion

- Introduction

This discussion paper proposes to continue the efforts within 3GPP for studying and potentially developing an open F1-like interface for the 6G base station architecture. This interface would serve to connect the Central Unit (CU) and the Distributed Unit (DU) of the next-generation Node B (6G NB), building upon the functional split concept introduced in 5G New Radio (NR).

- Discussion

The standardization of the 5G New Radio Access Network (NG-RAN) introduced the concept of functional decomposition, notably the split between the gNB Central Unit (gNB-CU) and the gNB Distributed Unit (gNB-DU), interconnected by the F1 interface.

Although discussions regarding 6G architecture aim for system simplification and minimizing multiple options, the continued necessity for deployment flexibility, support for cloud-native implementations, and advanced coordination mechanisms demands a standardized separation of centralized and distributed base station functions.

### 2.1 Background: The F1 Interface in 5G

The F1 interface interconnects the gNB-CU and the gNB-DU within the NG-RAN. Architecturally, the F1 interface consists of a control plane (F1-C) and a user plane (F1-U). The F1 Application Protocol (F1AP) specifies the signaling protocol for the F1 interface, supporting functions such as resource configuration, UE mobility, data transfer, and specialized network features like positioning, IAB and MBS.

This functional split was motivated by:

Flexibility in Deployment: Enabling Centralized Radio Access Network (C-RAN) or Virtualized RAN (vRAN) topologies where the baseband processing units (BBU, analogous to CU/DU) can be centralized or distributed.

Performance/Operational Benefits: The ability to deploy CUs and DUs at different locations has provided service and operational advantages, including enhanced capacity, improved latency, and benefits related to Dual Connectivity (e.g., ENDC and NR-DC).

However, the implementation of the 5G F1 interface also faced challenges:

Limited Multi-Vendor Deployment: The F1 interface has not been widely used in a multi-vendor setting, and deployment separating the CU and DU has been limited.

Performance Complications: Splitting the radio protocol stack complicates reconfiguration and can affect overall performance.

### 2.2. Experience Learned from 5G Design and Deployments

The experience with F1 provides critical architectural lessons for 6G planning:

- Fundamental Design Principle of Openness: A core design objective for the 5G F1 interface was that it be open and facilitate the inter-connection of a gNB-CU and a gNB-DU supplied by different manufacturers.

- Challenge of Multi-Vendor Adoption: Despite standardization efforts and some single-vendor solutions, the F1 interface was not widely used in a multi-vendor setting. For 6G, 3GPP defined interfaces must achieve true multi-vendor interoperability [3].

- Support for Cloud and Disaggregation: The CU-DU disaggregated architecture enabled by F1 is foundational for modern network implementation, allowing flexible HW implementations and offering scalable cost effective solutions [6]. This split is essential for realizing Network Function Virtualization within a cloud environment, and enables the separation of the Control Plane and User Plane.

- Operational Complexity: It has been observed that splitting the radio protocol stack complicates reconfiguration, potentially affecting performance. The 6G design must strive for system simplification and minimize functional ambiguity.

### 2.3 Justification for an Open F1-like Interface Study in 6G

The continuation of efforts to define an open F1-like interface is essential to meet the key requirements and architectural trends identified for 6G:

#### Alignment with Open Standardization and Cloud RAN Objectives

A core principle for 6G is that the RAN architecture shall support standardized, open interfaces for multi-vendor interoperability. Furthermore, 6G is expected to allow cloud implementation, enabling the independent selection of hardware and software for digital elements. We still see quite some support, especially from the mobile operators [1] [2], to continue studying an open F1-like interface in 6G.

- Cloud Implementation: The 6G RAN architecture shall allow cloud implementation. In Cloud RAN implementations, the F1 interface is expected to connect the virtualized DU, typically located toward the network edge, with the virtualized CU, which is more centralized. A well-defined, open F1-like interface is critical to realizing this flexible, multi-vendor Cloud RAN vision.

- Avoiding Fragmentation: Operators emphasize the need for open standardized interfaces for 6G RAT & RAN. Standardizing the CU-DU split interface within 3GPP mitigates the risk of fragmentation inherent in relying solely on proprietary or non-3GPP specifications for this critical split point.

#### Enabling Advanced 6G Features and Centralized Coordination

Many projected 6G features rely on coordination and centralization capabilities that are optimally facilitated by separating the CU and DU functions:

- Distributed MIMO/Multiple TRPs: The 6G RAN must support connectivity through multiple TRPs, either collocated or non-collocated. Enabling effective inter-site scheduling coordination and fast and dynamic resource arbitration between different TRPs serving the same UE requires RAN interfaces defined in 6G [3] and an F1-like interface shall be studied.

- AI/ML Integration: 6G is expected to be "AI native," integrating AI/ML across all layers and operations. Centralization of RAN functions has strong potential for benefits such as more efficient inter-cell coordination for interference management. To fulfil the high computing, low-latency and high security requirements of 6G RAN for AI new service, a centralized AI node, which serves multiple BSs, can be introduced [3]. A similar logic could be extended to the CU-DU split and an F1-like interface can enable such function. Additionally, it may be beneficial to separate the 6G base station to accommodate different AI functions in each entity so that the CU and DU design can be optimized parallely.

- Deployment Flexibility for Low Latency: The architecture must allow for deployment flexibility, enabling the hosting of relevant functions closer to the network edge, when needed, to enable low latency services [3]. This deployment model relies directly on the successful functional split defined by an F1-like interface.

#### Supporting Migration and Architecture Evolution

While the 6G architecture aims to be simpler than 5G (avoiding multiple architecture options), migration from 5G to 6G is crucial.

- Smooth Transition: A smooth migration path from NR connected to 5GCN is required for 6G RAT. Maintaining the concept of a functional split similar to F1 allows operators to leverage existing operational knowledge and infrastructure designed around the CU/DU separation model as they transition to 6G.

### 2.4 Proposed Discussion Points for 3GPP Study

The 6G RAN architecture study items specifically task RAN WGs with studying the need for open F1-like and E1-like interfaces. To continue the F1-like effort, the following study objectives for RAN3 (in coordination with RAN2 and RAN1) are proposed:

- Define the Scope and Functions of the F1-like Split: Investigate the optimal protocol layer split point (functional split) between the CU and DU for 6G (Option 2 was the basis for 5G F1). This includes clarifying which procedures and protocols for 6GR require separation, potentially leveraging lessons learned from 5G implementations. The study objectives already include investigating the RAN internal functional split, interfaces, protocol stacks, and procedures.

- Protocol Optimization for Cloud-Native Implementation: Study protocols optimized for cloud-native implementations for the F1-like interface. This involves considering the use of IETF-based evolution, such as QUIC, on the F1 interface, as expected in 6G CRAN architectures.

- Support for Advanced Coordination: Define the F1-like control plane signaling necessary to support new 6G features, such as coordination required for:

  - Distributed MIMO and multi-TRP connectivity.

  - Enhanced mobility procedures (e.g., lower-layer triggered mobility, multi-hop UE trajectory).

  - Data transfer design to support various types of data, including AI/ML and Sensing data.

- Addressing Performance Bottlenecks: Analyze how the 6G F1-like interface design can avoid the complexity and performance degradation issues noted in the 5G F1 interface, particularly concerning reconfiguration and coordination overheads.

- Interim Milestone: Support the agreed RAN3 objective to provide interim study results by TSG#112 (June/2026) to allow TSGs to make a decision on RAN internal interfaces, specifically the CU-DU split and CP-UP split.

Proposal: RAN3 should pursue the study and definition of requirements for an open F1-like interface between the DU and the CU within the 6G RAN architecture, ensuring its design addresses known performance limitations and supports novel 6G features.

# Conclusion

Defining an open and standardized F1-like interface is crucial for delivering a flexible, cost-efficient, and feature-rich 6G RAN capable of supporting multi-vendor cloud deployments and advanced network coordination mechanisms required by integrated services like AI/ML and Distributed MIMO. 3GPP must prioritize studying this interface, incorporating lessons from 5G to ensure a truly optimized and high-performance solution for the 6G era.

Proposal: RAN3 should pursue the study and definition of requirements for an open F1-like interface between the DU and the CU within the 6G RAN architecture, ensuring its design addresses known performance limitations and supports novel 6G features.

# Reference

[1] RP-251682, Key operator requirements for 6G, Deutsche Telekom, Vodafone, Orange, Telecom Italia, Telefonica, BT, Bouygues Telecom, Spark NZ, Odido, Turkcell, Telenor, Telus, CKH IOD UK, Jio Platforms, Reliance Jio, KPN, Boost Mobile Network

[2] RP-252474, TP on 6G Requirements for architecture and migration, NTT DOCOMO, INC., ZTE, Samsung, Qualcomm

[3] RP-252909, Summary of Discussion on 6G Requirements for architecture and migration, NTT DOCOMO, INC. (Rapporteur)

[4] T. Chen et al, “O-RAN’s Role in Shaping 6G: Industry Perspectives on Open and Smart RAN”, IEEE Wireless Communications • February 2025

[5] B. Agarwal et al, “Open RAN for 6G Networks: Architecture, Use Cases and Open Issues”, IEEE Communications Surveys & Tutorials, 2025

[6] 3GPP TS 38.801v14.0.0, Study on new radio access technology: Radio access architecture and interfaces

[7] 3GPP TS 38.470v19.0.0, F1 general aspects and principles