3GPP TSG-RAN WG3 Meeting #129-bis	R3-257172

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	9.2.5

Source:	Ericsson, Jio Platforms, Verizon Wireless, LG Electronics

Title:	Remaining issues for CSI-RS coordination

Document for:	Discussion, Agreement

# Introduction

The Rel-19 Mobility WI, covering inter-CU LTM, has been completed from functional point of view. RAN3 has agreed to support transferring CSI-RS resource configuration, for both L1-RSRP measurements and early CSI acquisition. The mechanism allows the network to coordinate CSI-RS resource allocation across NG-RAN nodes. However, the aspects related to early CSI acquisition require further analysis, particularly regarding necessity and signaling impacts.

This paper discusses these aspects and outlines the potential specification impacts of the proposed solutions.

# Discussion

## CSI-RS Resource Configuration for Early CSI Aquisition

At the last RAN3 meeting, the group discussed how the source node and candidate node should coordinate the transfer of CSI-RS resource configuration. The agreed solution is that the source node sends different requests depending on the type of CSI-RS transmission.

It was further noted that the handling of L1 measurements and early CSI acquisition is not identical for the source gNB. In particular, activation of semi-persistent CSI-RS transmission at the candidate network node may lead to one of the following cases:

- For L1 measurements: the UE measures physical quantities (e.g., L1-RSRP, L1-RSRQ, L1-SINR) for the activated resources and reports the results to the source network node. Based on these reports, the source node decides whether to trigger an LTM cell switch execution toward the cell where semi-persistent CSI-RS transmission was activated.

- Early CSI acquisition: the UE measures CSI-related quantities (e.g., CRI, RI, PMI, CQI) for the activated resources. The source node then triggers LTM cell switch execution toward the target node. After completion of the cell switch, the UE reports the measurements to the target node, which may use the information, for example, to optimize link adaptation when the UE arrives at the target cell.

Therefore, the role of the source node differs between the two cases. For early CSI acquisition, the source node acts as transparently, i.e., forwarding the configuration to the UE without making any decision. Consequently, the source node has no knowledge of when to trigger the request for CSI-RS resource configuration for early CSI acquisition, as this decision is determined by the candidate node.

- The source DU/gNB has no clue to decide when/whether to ask for CSI-RS resource configuration for early CSI acquisition.

With such an implementation, the request from the source node for CSI-RS resource configuration would effectively become a blind request, which provides no benefit.

Based on the above considerations, the candidate node should be able to provide the CSI-RS resource configuration to the UE whenever it decides the possibility of having early CSI acquisition, and this should be done independently of any explicit request from the source node. This ensures that the candidate node can optimize the timing and conditions for early CSI acquisition according to its own status, without relying on coordination or triggering from the source node.

To support this behavior in the specifications, it is necessary to define a solution whereby the candidate node can send the CSI-RS resource configuration. Currently, the specification is implemented as follows.

If the Request for CSI-RS Resource Configuration for L1 Measurements IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for L1 Measurements IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for Early CSI Acquisition IE in the HANDOVER REQUEST ACKNOWLEDGE message.

And in the XnAP HANDOVER REQUEST message it shows that,

| LTM Handover Information Request | O |  |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >Proposed LTM No Security Change ID List | O |  | 9.2.3.231 |  | – |  |
| >Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | – |  |
| >Reference Configuration | O |  | OCTET STRING | Includes the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >Request for CSI-RS Resource Configuration for L1 Measurements | O |  | ENUMERATED (true, …) |  | – |  |
| >Request for CSI-RS Resource Configuration for Early CSI Acquisition | O |  | ENUMERATED (true, …) |  | – |  |
| Early Sync Information Request | O |  | 9.2.3.217 |  | YES | ignore |

To allow the candidate DU/gNB to send the CSI-RS resource configuration without explicit request from the source DU/gNB, several possible options are described below.

Option 1:

The cleanest way would be removing the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE from the request message. In that case, the candidate node includes the optional CSI-RS Resource Configuration for early CSI acquisition whenever it wants to. An example of new procedural text is given below. This should cover both Handover Request Acknowledge message and LTM Configuration Update Acknowledge message.

The target NG-RAN node may include the CSI-RS Resource Configuration for Early CSI Acquisition IE in the LTM Handover Information Request Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

Option 2:

Instead of removing the existing IE, another possible workaround is to clarify more in the procedural text. For example,

If the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shallmay, if supported, include the CSI-RS Resource Configuration for Early CSI Acquisition IE in the HANDOVER REQUEST ACKNOWLEDGE message.

And add the description for the target NG-RAN node’s behavior, e.g.,

The target NG-RAN node may provide the CSI-RS Resource Configuration for Early CSI Acquisition IE in the HANDOVER REQUEST ACKNOWLEDGE message to the source NG-RAN node.

Option 3:

If no consensus can be reached in RAN3 regarding the distinction between CSI-RS resource configuration for L1 measurements and early CSI acquisition, RAN3 needs to consider sending an LS to RAN1 and RAN2 to confirm whether the source network node is expected to have the capability to trigger such a request for resources in the case of early CSI acquisition.

Among the three options discussed above, Option 1 is preferred as it provides the most straightforward approach and avoids potential confusion for this specific request.

- RAN3 agrees that the candidate DU/gNB is the one who decides whether and when to provide the CSI-RS resource configuration for early CSI acquisition.

- Agree on Option 1, i.e., remove the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE in TS 38.423.

- Similarly, remove the Request for CSI-RS Resource Configuration for CSI Acquisition IE in TS 38.473.

- Introduce the CSI-RS Resource Configuration for Early CSI Acquisition IE to the XnAP LTM CONFIGURATION UPDATE ACKNOWLEDGE message.

## CSI-IM Resource Configuration

Regarding CSI-IM resource configuration, RAN1 agreed at the RAN1#122 meeting that semi-persistent CSI-IM measurement resources are supported for CSI acquisition from candidate cells.

Accordingly, there is no need for an explicit request for CSI-IM resource configuration over the Xn or F1 interfaces. However, it remains necessary to distinguish between periodic and semi-persistent CSI-IM resource configurations when candidate DUs/gNBs provide them as part of the CSI-RS Resource Configuration.

- No explicit request from the source network node is needed for CSI-IM Resource Configuration.

- Separate the existing CSI-IM Resource Configuration IE in XnAP to two IEs, i.e., Periodic CSI-IM Resource Configuration, and Semi-persistent CSI-IM Resource Configuration IEs, within CSI-RS Resource Configuration.

- Introduce the new two IEs to CSI-RS Resource Configuration in F1AP, i.e., Periodic CSI-IM Resource Configuration, and Semi-persistent CSI-IM Resource Configuration IEs.

- Agree the CR to XnAP and F1AP in R3-257173 and R3-257174.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Observation 1	The source DU/gNB has no clue to decide when/whether to ask for CSI-RS resource configuration for early CSI acquisition.

Observation 2	No explicit request from the source network node is needed for CSI-IM Resource Configuration.

Proposal 1	RAN3 agrees that the candidate DU/gNB is the one who decides whether and when to provide the CSI-RS resource configuration for early CSI acquisition.

Proposal 2	Agree on Option 1, i.e., remove the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE in TS 38.423.

Proposal 3	Similarly, remove the Request for CSI-RS Resource Configuration for CSI Acquisition IE in TS 38.473.

Proposal 4	Introduce the CSI-RS Resource Configuration for Early CSI Acquisition IE to the XnAP LTM CONFIGURATION UPDATE ACKNOWLEDGE message.

Proposal 5	Separate the existing CSI-IM Resource Configuration IE in XnAP to two IEs, i.e., Periodic CSI-IM Resource Configuration, and Semi-persistent CSI-IM Resource Configuration IEs, within CSI-RS Resource Configuration.

Proposal 6	Introduce the new two IEs to CSI-RS Resource Configuration in F1AP, i.e., Periodic CSI-IM Resource Configuration, and Semi-persistent CSI-IM Resource Configuration IEs.

Proposal 7	Agree the CR to XnAP and F1AP in R3-257173 and R3-257174.